Chroma 62000L

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Chroma 62000L.

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instrument drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

For VXI instrument drivers:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

MXI

MXI2

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Each function in this instrument driver returns a status code that

either indicates success or describes an error or warning condition.

Your program should examine the status code from each call to an

instrument driver function to determine if an error occurred.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

The description of each instrument driver function lists possible error

codes and their meanings

________________________________________________________________________________

How To Use This Document:

Use this document as a programming reference manual.

It describes each function in the

Chroma 62000L

instrument.  The functions appear in alphabetical order, with a description

of the function and its C syntax, a description of each parameter, and a list

of possible error codes.

________________________________________________________________________________

Function Tree Layout:

Class/Panel Name:                                    Function Name:

Initialize                                        Chr62000L_init

Initialize With Options                           Chr62000L_InitWithOptions

Configuration Functions

Output

Configure CV CC Limit                       Chr62000L_ConfigureCVCCLimit

Configure Output Enable                     Chr62000L_ConfigureOutputEnable

Configure CC Limit                          Chr62000L_ConfigureCCLimit

Configure OCP                               Chr62000L_ConfigureOCP

Configure OCP Enabled                       Chr62000L_ConfigureOCPEnabled

Configure OCP Open Clear                    Chr62000L_ConfigureOCPOpenClear

Configure CV Limit                          Chr62000L_ConfigureCVLimit

Configured OVP                              Chr62000L_ConfiguredOVP

Configure OVP Enabled                       Chr62000L_ConfigureOVPEnabled

Configure OVP Open Clear                    Chr62000L_ConfigureOVPOpenClear

Query

Query CV CC Limit                        Chr62000L_QueryCVCCLimit

Query Output States                      Chr62000L_QueryOutputStates

Query CC Limit                           Chr62000L_QueryCCLimit

Query OCP                                Chr62000L_QueryOCP

Query OCP State                          Chr62000L_QueryOCPState

Query OCP Open                           Chr62000L_QueryOCPOpen

Query CV Limit                           Chr62000L_QueryCVLimit

Query OVP                                Chr62000L_QueryOVP

Query OVP State                          Chr62000L_QueryOVPState

Query OVP Open                           Chr62000L_QueryOVPOpen

Master & Slave

Configure Master Slave On Off Delay Time    Chr62000L_ConfigureMasterSlaveOnOffDelayTime

Configure Operation Mode                    Chr62000L_ConfigureOperationMode

Configure Master Slave Enabled              Chr62000L_ConfigureMasterSlaveEnabled

Query

Query Master Slave On Off Delay Time     Chr62000L_QueryMasterSlaveOnOffDelayTime

Query Operation Mode                     Chr62000L_QueryOperationMode

Query Master Slave Enabled               Chr62000L_QueryMasterSlaveEnabled

Triggering

Configure Triggered Current                 Chr62000L_ConfigureTriggeredCurrent

Configure Triggered Voltage                 Chr62000L_ConfigureTriggeredVoltage

Configure Trigger Sourced                   Chr62000L_ConfigureTriggerSourced

Configure Triggered Delay                   Chr62000L_ConfigureTriggeredDelay

Configure *TRG                              Chr62000L_ConfigureTRG

Query

Query Triggered Current                  Chr62000L_QueryTriggeredCurrent

Query Triggered Voltage                  Chr62000L_QueryTriggeredVoltage

Query Trigger Source                     Chr62000L_QueryTriggerSource

Query Triggered Delay                    Chr62000L_QueryTriggeredDelay

Continuous Output

Configure Continuous Output Enabled         Chr62000L_ConfigureContinuousOutputEnabled

Configure Continuous Output Mode            Chr62000L_ConfigureContinuousOutputMode

Configure Continuous Output Cycles          Chr62000L_ConfigureContinuousOutputCycles

Configure Continuous Output Step            Chr62000L_ConfigureContinuousOutputStep

Configure Step Voltage                      Chr62000L_ConfigureStepVoltage

Configure Step Current                      Chr62000L_ConfigureStepCurrent

Configure Rise Fall Time                    Chr62000L_ConfigureRiseFallTime

Configure DWELL Time                        Chr62000L_ConfigureDWELLTime

Query

Query Continuous Output State            Chr62000L_QueryContinuousOutputState

Query Continuous Output Finish           Chr62000L_QueryContinuousOutputFinish

Query Continuous Output Mode             Chr62000L_QueryContinuousOutputMode

Query Continuous Output Cycles           Chr62000L_QueryContinuousOutputCycles

Query Continuous Output Step             Chr62000L_QueryContinuousOutputStep

Query Step Voltage                       Chr62000L_QueryStepVoltage

Query Step Current                       Chr62000L_QueryStepCurrent

Query Rise Fall Time                     Chr62000L_QueryRiseFallTime

Query DWELL Time                         Chr62000L_QueryDWELLTime

Query Step Setting                       Chr62000L_QueryStepSetting

System Function

Configure Display Enabled                   Chr62000L_ConfigureDisplayEnabled

Configure Display Text                      Chr62000L_ConfigureDisplayText

Configure Display Text Clear                Chr62000L_ConfigureDisplayTextClear

Configure Beeper                            Chr62000L_ConfigureBeeper

Configure OVP Beeper Enabled                Chr62000L_ConfigureOVPBeeperEnabled

Configure OCP Beeper Enabled                Chr62000L_ConfigureOCPBeeperEnabled

Configure Normal Beeper Enabled             Chr62000L_ConfigureNormalBeeperEnabled

Query

Query Display State                      Chr62000L_QueryDisplayState

Query Display Text                       Chr62000L_QueryDisplayText

Query SCPI Version                       Chr62000L_QuerySCPIVersion

Query OVP Beeper State                   Chr62000L_QueryOVPBeeperState

Query OCP Beeper State                   Chr62000L_QueryOCPBeeperState

Query Normal Beeper State                Chr62000L_QueryNormalBeeperState

Calibration

Configure CC Calibration                    Chr62000L_ConfigureCCCalibration

Configure CC Calibration Level              Chr62000L_ConfigureCCCalibrationLevel

Configure Calibration Password              Chr62000L_ConfigureCalibrationPassword

Configure Calibration Protection Enabled    Chr62000L_ConfigureCalibrationProtectionEnabled

Configure Calibration Message               Chr62000L_ConfigureCalibrationMessage

Configure CV Calibration                    Chr62000L_ConfigureCVCalibration

Configure CV Calibration Level              Chr62000L_ConfigureCVCalibrationLevel

Query

Query Calibration Number                 Chr62000L_QueryCalibrationNumber

Query Calibration Protection Enabled     Chr62000L_QueryCalibrationProtectionEnabled

Query Calibration Message                Chr62000L_QueryCalibrationMessage

Set/Get/Check Attribute

Set Attribute

Set Attribute ViInt32                    Chr62000L_SetAttributeViInt32

Set Attribute ViReal64                   Chr62000L_SetAttributeViReal64

Set Attribute ViString                   Chr62000L_SetAttributeViString

Set Attribute ViBoolean                  Chr62000L_SetAttributeViBoolean

Set Attribute ViSession                  Chr62000L_SetAttributeViSession

Get Attribute

Get Attribute ViInt32                    Chr62000L_GetAttributeViInt32

Get Attribute ViReal64                   Chr62000L_GetAttributeViReal64

Get Attribute ViString                   Chr62000L_GetAttributeViString

Get Attribute ViBoolean                  Chr62000L_GetAttributeViBoolean

Get Attribute ViSession                  Chr62000L_GetAttributeViSession

Check Attribute

Check Attribute ViInt32                  Chr62000L_CheckAttributeViInt32

Check Attribute ViReal64                 Chr62000L_CheckAttributeViReal64

Check Attribute ViString                 Chr62000L_CheckAttributeViString

Check Attribute ViBoolean                Chr62000L_CheckAttributeViBoolean

Check Attribute ViSession                Chr62000L_CheckAttributeViSession

Measure Output

Measured                                       Chr62000L_Measured

Action/Status Functions

Initiate                                       Chr62000L_Initiate

Save Operation State                           Chr62000L_SaveOperationState

Read Operation State                           Chr62000L_ReadOperationState

Set Questionable Enabled Register              Chr62000L_SetQuestionableEnabledRegister

Set CLS                                        Chr62000L_SetCLS

Set ESE                                        Chr62000L_SetESE

Set OPC                                        Chr62000L_SetOPC

Set PSC                                        Chr62000L_SetPSC

Set SRE                                        Chr62000L_SetSRE

Set WAI                                        Chr62000L_SetWAI

Query

Query Operation Mode Status                 Chr62000L_QueryOperationModeStatus

Query Questionable Event Register Status    Chr62000L_QueryQuestionableEventRegisterStatus

Query Questionable Enabled Register         Chr62000L_QueryQuestionableEnabledRegister

Query ESE                                   Chr62000L_QueryESE

Query ESR                                   Chr62000L_QueryESR

Query OPC                                   Chr62000L_QueryOPC

Query PSC                                   Chr62000L_QueryPSC

Query SRE                                   Chr62000L_QuerySRE

Query STB                                   Chr62000L_QuerySTB

System

Write                                          Chr62000L_viWrite

Read                                           Chr62000L_viRead

Utility Functions

Reset                                          Chr62000L_reset

Reset With Defaults                            Chr62000L_ResetWithDefaults

Self-Test                                      Chr62000L_self_test

Revision Query                                 Chr62000L_revision_query

Error-Query                                    Chr62000L_error_query

Error Message                                  Chr62000L_error_message

Invalidate All Attributes                      Chr62000L_InvalidateAllAttributes

Error Info

Get Error                                   Chr62000L_GetError

Clear Error                                 Chr62000L_ClearError

Coercion Info

Get Next Coercion Record                    Chr62000L_GetNextCoercionRecord

Interchangeability Info

Get Next Interchange Warning                Chr62000L_GetNextInterchangeWarning

Clear Interchange Warnings                  Chr62000L_ClearInterchangeWarnings

Reset Interchange Check                     Chr62000L_ResetInterchangeCheck

Locking

Lock Session                                Chr62000L_LockSession

Unlock Session                              Chr62000L_UnlockSession

Close                                             Chr62000L_close

Obsolete

Instrument I/O

Write Instrument Data                       Chr62000L_WriteInstrData

Read Instrument Data                        Chr62000L_ReadInstrData

________________________________________________________________________________

Chroma 62000L

This instrument driver provides programming support for the Chr62000L.

The driver contains all the functions that IVI and VXIplug&play require.

In addition, the driver contains high-level functions that configure the

power supply and generate output in a single operation.  The driver also

contains lower level functions that configure the power supply and

initiate the output changes in separate operations.

Note:  This driver requires the VISA and IVI libraries.

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

Chr62000L_CheckAttributeViBoolean

ViStatus Chr62000L_CheckAttributeViBoolean (ViSession instrumentHandle,

ViConstString channelName,

ViAttr attributeID,

ViBoolean attributeValue);

Purpose

This function checks the validity of a value you specify for a ViBoolean

attribute.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViConstString

If the attribute is channel-based, this parameter specifies the name

of the channel on which to check the attribute value. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViBoolean

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViBoolean are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViBoolean

Pass the value which you want to verify as a valid value for the

attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_CheckAttributeViInt32

ViStatus Chr62000L_CheckAttributeViInt32 (ViSession instrumentHandle,

ViConstString channelName,

ViAttr attributeID,

ViInt32 attributeValue);

Purpose

This function checks the validity of a value you specify for a ViInt32

attribute.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViConstString

If the attribute is channel-based, this parameter specifies the name

of the channel on which to check the attribute value. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViInt32 type.

If you choose to see all IVI attributes, the data types appear

to the right of the attribute names in the list box.

Attributes with data types other than ViInt32 are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViInt32

Pass the value which you want to verify as a valid value for the

attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_CheckAttributeViReal64

ViStatus Chr62000L_CheckAttributeViReal64 (ViSession instrumentHandle,

ViConstString channelName,

ViAttr attributeID,

ViReal64 attributeValue);

Purpose

This function checks the validity of a value you specify for a ViReal64

attribute.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViConstString

If the attribute is channel-based, this parameter specifies the name

of the channel on which to check the attribute value. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViReal64

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViReal64 are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViReal64

Pass the value which you want to verify as a valid value for the

attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_CheckAttributeViSession

ViStatus Chr62000L_CheckAttributeViSession (ViSession instrumentHandle,

ViConstString channelName,

ViAttr attributeID,

ViSession attributeValue);

Purpose

This function checks the validity of a value you specify for a ViSession

attribute.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViConstString

If the attribute is channel-based, this parameter specifies the name

of the channel on which to check the attribute value. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViSession

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViSession are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViSession

Pass the value which you want to verify as a valid value for the

attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_CheckAttributeViString

ViStatus Chr62000L_CheckAttributeViString (ViSession instrumentHandle,

ViConstString channelName,

ViAttr attributeID,

ViChar _VI_FAR attributeValue[]);

Purpose

This function checks the validity of a value you specify for a ViString

attribute.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViConstString

If the attribute is channel-based, this parameter specifies the name

of the channel on which to check the attribute value. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViString

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViString are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViChar[]

Pass the value which you want to verify as a valid value for the

attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ClearError

ViStatus Chr62000L_ClearError (ViSession instrumentHandle);

Purpose

This function clears the error code and error description for the IVI

session. If the user specifies a valid IVI session for the

InstrumentHandle parameter, this function clears the error information

for the session. If the user passes VI_NULL for the Vi parameter, this

function clears the error information for the current execution thread.

If the Vi parameter is an invalid session, the function does nothing and

returns an error.

The function clears the error code by setting it to VI_SUCCESS.  If the

error description string is non-NULL, the function de-allocates the error

description string and sets the address to VI_NULL.

Maintaining the error information separately for each thread is useful if

the user does not have a session handle to pass to the Chr62000L_GetError

function, which occurs when a call to Chr62000L_init or

Chr62000L_InitWithOptions fails.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ClearInterchangeWarnings

ViStatus Chr62000L_ClearInterchangeWarnings

(ViSession instrumentHandle);

Purpose

This function clears the list of current interchange warnings.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_close

ViStatus Chr62000L_close (ViSession instrumentHandle);

Purpose

This function performs the following operations:

- Closes the instrument I/O session.

- Destroys the instrument driver session and all of its attributes.

- Deallocates any memory resources the driver uses.

Notes:

(1) You must unlock the session before calling Chr62000L_close.

(2) After calling Chr62000L_close, you cannot use the instrument driver

again until you call Chr62000L_init or Chr62000L_InitWithOptions.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureBeeper

ViStatus Chr62000L_ConfigureBeeper (ViSession instrumentHandle);

Purpose

Start a beep.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureCalibrationMessage

ViStatus Chr62000L_ConfigureCalibrationMessage

(ViSession instrumentHandle, ViChar _VI_FAR message[]);

Purpose

Write about correction messages. For example, use this command to store

the correction date. The correction string can contain up to 35 letters

and numbers, and must contain quotation marks.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

message

Variable Type       ViChar[]

Write about correction messages. For example, use this command to

store the correction date. The correction string can contain up to 35

letters and numbers, and must contain quotation marks.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureCalibrationPassword

ViStatus Chr62000L_ConfigureCalibrationPassword

(ViSession instrumentHandle, ViChar _VI_FAR password[]);

Purpose

Change the calibration protection password. To change the password, you

must first unprotect the calibration, and then use the new password to

restart the calibration after resetting the new password. The password

consists of 6 letters and numbers without quotation marks, regardless of

case.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

password

Variable Type       ViChar[]

Change the calibration protection password. To change the password,

you must first unprotect the calibration, and then use the new

password to restart the calibration after resetting the new password.

The password consists of 6 letters and numbers without quotation

marks, regardless of case.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureCalibrationProtectionEnabled

ViStatus Chr62000L_ConfigureCalibrationProtectionEnabled

(ViSession instrumentHandle, ViChar _VI_FAR password[],

ViBoolean enabled);

Purpose

Turns the calibration protection on / off. The password consists of 6

letters and numbers without quotation marks, regardless of case.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

password

Variable Type       ViChar[]

The password consists of 6 letters and numbers without quotation

marks, regardless of case.

enabled

Variable Type       ViBoolean

Turns the calibration protection on / off.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureCCCalibration

ViStatus Chr62000L_ConfigureCCCalibration (ViSession instrumentHandle,

ViReal64 CCCalibration);

Purpose

Enter the CC calibration information. Before using this command, you need

to set the calibration standard. Otherwise, the calibration interference

error will occur.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

CCCalibration

Variable Type       ViReal64

Enter the CC calibration information. Before using this command, you

need to set the calibration standard. Otherwise, the calibration

interference error will occur.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureCCCalibrationLevel

ViStatus Chr62000L_ConfigureCCCalibrationLevel

(ViSession instrumentHandle, ViInt32 CCCalibrationLevel);

Purpose

Set the power supply of the CC calibration standard, there are two

calibration points, you need to correct the MIN or otherwise corrective

interference error.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

CCCalibrationLevel

Variable Type       ViInt32

Set the power supply of the CC calibration standard, there are two

calibration points, you need to correct the MIN or otherwise

corrective interference error.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureCCLimit

ViStatus Chr62000L_ConfigureCCLimit (ViSession instrumentHandle,

ViInt32 current,

ViReal64 userDefineCurrentValue,

ViInt32 currentStep,

ViReal64 userDefineCurrentStepValue);

Purpose

This is used to set the power of the CC limit.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

current

Variable Type       ViInt32

This is used to set the power of the CC limit.

userDefineCurrentValue

Variable Type       ViReal64

When Current set UserDef, then use this value.

currentStep

Variable Type       ViInt32

This is used to set the CC limit value increases or decreases the

amplitude.

userDefineCurrentStepValue

Variable Type       ViReal64

When Current Step set UserDef, then use this value.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureContinuousOutputCycles

ViStatus Chr62000L_ConfigureContinuousOutputCycles

(ViSession instrumentHandle, ViInt32 cycles);

Purpose

Set the number of cycles for continuous output. The setting range is 0 ~

65535, and 0 means infinite loop.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

cycles

Variable Type       ViInt32

Set the number of cycles for continuous output. The setting range is

0 ~ 65535, and 0 means infinite loop.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureContinuousOutputEnabled

ViStatus Chr62000L_ConfigureContinuousOutputEnabled

(ViSession instrumentHandle, ViBoolean enable);

Purpose

This is used to enable / disable continuous output function.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enable

Variable Type       ViBoolean

This is used to enable / disable continuous output function.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureContinuousOutputMode

ViStatus Chr62000L_ConfigureContinuousOutputMode

(ViSession instrumentHandle, ViInt32 mode);

Purpose

This is used to set the continuous output mode. 0 means only the voltage,

1 means only the current, 2 means that the voltage and current are

subject to continuous output control.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

mode

Variable Type       ViInt32

This is used to set the continuous output mode. 0 means only the

voltage, 1 means only the current, 2 means that the voltage and

current are subject to continuous output control.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureContinuousOutputStep

ViStatus Chr62000L_ConfigureContinuousOutputStep

(ViSession instrumentHandle, ViInt32 startStep,

ViInt32 stopStep);

Purpose

Set the start and stop step of the output. The set values are S0 to S7.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

startStep

Variable Type       ViInt32

Set the start step of the output. The set values are S0 to S7.

stopStep

Variable Type       ViInt32

Set the stop step of the output. The set values are S0 to S7.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureCVCalibration

ViStatus Chr62000L_ConfigureCVCalibration (ViSession instrumentHandle,

ViReal64 CVCalibration);

Purpose

Enter the CV calibration information. Before using this command, you need

to set the calibration standard. Otherwise, the calibration interference

error will occur.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

CVCalibration

Variable Type       ViReal64

Enter the CV calibration information. Before using this command, you

need to set the calibration standard. Otherwise, the calibration

interference error will occur.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureCVCalibrationLevel

ViStatus Chr62000L_ConfigureCVCalibrationLevel

(ViSession instrumentHandle, ViInt32 CVCalibrationLevel);

Purpose

Set the power supply of the CV calibration standard, there are two

calibration points, you need to correct the MIN or otherwise corrective

interference error.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

CVCalibrationLevel

Variable Type       ViInt32

Set the power supply of the CV calibration standard, there are two

calibration points, you need to correct the MIN or otherwise

corrective interference error.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureCVCCLimit

ViStatus Chr62000L_ConfigureCVCCLimit (ViSession instrumentHandle,

ViInt32 voltage,

ViReal64 userDefineVoltageValue,

ViInt32 current,

ViReal64 userDefineCurrentValue);

Purpose

Set CV/CC Limit value.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

voltage

Variable Type       ViInt32

This is used to set the power of the CV limit.

userDefineVoltageValue

Variable Type       ViReal64

When Voltage set UserDef, then use this value.

current

Variable Type       ViInt32

This is used to set the power of the CC limit.

userDefineCurrentValue

Variable Type       ViReal64

When Current set UserDef, then use this value.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureCVLimit

ViStatus Chr62000L_ConfigureCVLimit (ViSession instrumentHandle,

ViInt32 voltage,

ViReal64 userDefineVoltageValue,

ViInt32 voltageStep,

ViReal64 userDefineVoltageStepValue);

Purpose

This is used to set the power of the CV limit.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

voltage

Variable Type       ViInt32

This is used to set the power of the CV limit.

userDefineVoltageValue

Variable Type       ViReal64

When Voltage set UserDef, then use this value.

voltageStep

Variable Type       ViInt32

This is used to set the CV limit value increases or decreases the

amplitude.

userDefineVoltageStepValue

Variable Type       ViReal64

When Voltage Step set UserDef, then use this value.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureDisplayEnabled

ViStatus Chr62000L_ConfigureDisplayEnabled (ViSession instrumentHandle,

ViBoolean enabled);

Purpose

Turns the front panel display on or off. When turned off, the display

will not display anything except the ERR indicator. Press the LOCAL

button on the front panel to return to the manual operation, the display

will automatically turn on.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enabled

Variable Type       ViBoolean

Turns the front panel display on or off.

Default Value: VI_TRUE

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureDisplayText

ViStatus Chr62000L_ConfigureDisplayText (ViSession instrumentHandle,

ViChar _VI_FAR text[]);

Purpose

Sets the screen display string. When this command is executed, the front

panel screen is closed and the set string is displayed. The string must

be at most 49 characters and must be enclosed in quotation marks, as it

is limited to the number of digits displayed on the screen, and a string

of more than 6 characters will be rendered in a scrolling manner. All

characters in the string are rendered in uppercase, and characters that

can not be rendered are replaced with whitespace. Once you turn on your

screen, the string will no longer appear.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

text

Variable Type       ViChar[]

Set the screen display string.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureDisplayTextClear

ViStatus Chr62000L_ConfigureDisplayTextClear

(ViSession instrumentHandle);

Purpose

Clear the front panel display string.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfiguredOVP

ViStatus Chr62000L_ConfiguredOVP (ViSession instrumentHandle,

ViReal64 limitValue, ViInt32 limit);

Purpose

This command is used to set the voltage-current protection mechanism

standard voltage startup, if the output voltage is higher than the

standard OVP, internal SCR device causes the output short circuit while

setting question the status register "OVP" bit, the result lifted OVP

Once the cause protection mechanisms to start, you can use the VOLT:

PROT: CLE this command to clear the state.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

limitValue

Variable Type       ViReal64

When Limit set UserDef, then use this value.

limit

Variable Type       ViInt32

This is used to set the OVP.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureDWELLTime

ViStatus Chr62000L_ConfigureDWELLTime (ViSession instrumentHandle,

ViInt32 time, ViInt32 timeValue,

ViInt32 step);

Purpose

Set the DWELL time of the step, step S0 ~ S7, the maximum can be set

86399999ms.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

time

Variable Type       ViInt32

Set the DWELL time of the step, the maximum can be set 86399999ms.

timeValue

Variable Type       ViInt32

When Time set UserDef, then use this value.

step

Variable Type       ViInt32

Set the DWELL time of the step, step S0 ~ S7.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureMasterSlaveEnabled

ViStatus Chr62000L_ConfigureMasterSlaveEnabled

(ViSession instrumentHandle, ViBoolean enable);

Purpose

This is used to enable / disable master-slave control.

if your instrument SN Number less than 0200000122,the instrument not

support ,Please contact Chroma Technical Services.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enable

Variable Type       ViBoolean

This is used to enable / disable master-slave control.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureMasterSlaveOnOffDelayTime

ViStatus Chr62000L_ConfigureMasterSlaveOnOffDelayTime

(ViSession instrumentHandle, ViInt32 onDelay,

ViInt32 onDelayTime, ViInt32 offDelay,

ViInt32 offDelayTime);

Purpose

This is used to set the master-slave control on-delay time and off-delay

time, which can be set from 0 to 60 seconds.

if your instrument SN Number less than 0200000122,the instrument not

support ,Please contact Chroma Technical Services.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

onDelay

Variable Type       ViInt32

This is used to set the master-slave on delay time.

onDelayTime

Variable Type       ViInt32

When On Delay set UserDef, then use this value.(mS)

offDelay

Variable Type       ViInt32

This is used to set the master-slave off delay time.

offDelayTime

Variable Type       ViInt32

When Off Delay set UserDef, then use this value.(mS)

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureNormalBeeperEnabled

ViStatus Chr62000L_ConfigureNormalBeeperEnabled

(ViSession instrumentHandle, ViBoolean enabled);

Purpose

This is used to configure the normal beeper on or off.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enabled

Variable Type       ViBoolean

This is used to configure the normal beeper on or off.

Default Value: VI_TRUE

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureOCP

ViStatus Chr62000L_ConfigureOCP (ViSession instrumentHandle,

ViInt32 limit, ViReal64 limitValue);

Purpose

This is used to set the current standard current overcurrent protection

mechanism startup, if the output current is higher than the OCP standard,

the output current is controlled to zero, while setting question the

status register "OCP" bits, resulting in OCP protection in disarmament

Once the cause of the startup mechanism, you can use the CURR: PROT: CLE

this command to clear the state.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

limit

Variable Type       ViInt32

This is used to set the OCP.

limitValue

Variable Type       ViReal64

When Limit set UserDef, then use this value.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureOCPBeeperEnabled

ViStatus Chr62000L_ConfigureOCPBeeperEnabled (ViSession instrumentHandle,

ViBoolean enabled);

Purpose

This is used to configure the OCP alarm beeper on or off.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enabled

Variable Type       ViBoolean

This is used to configure the OCP alarm beeper on or off.

Default Value: VI_TRUE

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureOCPEnabled

ViStatus Chr62000L_ConfigureOCPEnabled (ViSession instrumentHandle,

ViBoolean enabled);

Purpose

Configures the OCP is on or off.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enabled

Variable Type       ViBoolean

Configures the OCP is on or off.

Default Value: VI_TRUE

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureOCPOpenClear

ViStatus Chr62000L_ConfigureOCPOpenClear (ViSession instrumentHandle);

Purpose

This is used to clear the overcurrent trip event. After the next

instruction, the output current will be re-stored in the state before the

current interruption protection, OCP trip remains the standard values

currently set. Before clearing breaking event, to make the output current

is lower than the standard OCP trip, or adjust the output current is

greater than the standard OCP trip.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureOperationMode

ViStatus Chr62000L_ConfigureOperationMode (ViSession instrumentHandle,

ViInt32 mode);

Purpose

This is used to set the master-slave control mode of operation.

if your instrument SN Number less than 0200000122,the instrument not

support ,Please contact Chroma Technical Services.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

mode

Variable Type       ViInt32

This is used to set the master-slave control mode of operation.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureOutputEnable

ViStatus Chr62000L_ConfigureOutputEnable (ViSession instrumentHandle,

ViBoolean enabled);

Purpose

Configures the power supply is on or off.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enabled

Variable Type       ViBoolean

Configures the power supply is on or off.

Default Value: VI_TRUE

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureOVPBeeperEnabled

ViStatus Chr62000L_ConfigureOVPBeeperEnabled (ViSession instrumentHandle,

ViBoolean enabled);

Purpose

This is used to configure the OVP alarm beeper on or off.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enabled

Variable Type       ViBoolean

This is used to configure the OVP alarm beeper on or off.

Default Value: VI_TRUE

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureOVPEnabled

ViStatus Chr62000L_ConfigureOVPEnabled (ViSession instrumentHandle,

ViBoolean enabled);

Purpose

Configures the OVP is on or off.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enabled

Variable Type       ViBoolean

Configures the OVP is on or off.

Default Value: VI_TRUE

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureOVPOpenClear

ViStatus Chr62000L_ConfigureOVPOpenClear (ViSession instrumentHandle);

Purpose

This is used to clear the overvoltage trip event. After the next

instruction, the output voltage will be re-stored in the state before the

interruption voltage protection, OVP trip remains the standard values

currently set. Before clearing breaking event, to make the output voltage

is below standard OVP trip, or adjust the OVP trip is greater than the

standard output voltage.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureRiseFallTime

ViStatus Chr62000L_ConfigureRiseFallTime (ViSession instrumentHandle,

ViInt32 time,

ViInt32 timeValue,

ViInt32 step);

Purpose

Set the rising and falling time, step set value is S0 ~ S7. Rise and fall

time setting range is 0 ~ 3599999ms.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

time

Variable Type       ViInt32

Set the rising and falling time. Rise and fall time setting range is

0 ~ 3599999ms.

timeValue

Variable Type       ViInt32

When Time set UserDef, then use this value.

step

Variable Type       ViInt32

Set the rising and falling time, step set value is S0 ~ S7.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureStepCurrent

ViStatus Chr62000L_ConfigureStepCurrent (ViSession instrumentHandle,

ViInt32 current,

ViReal64 currentValue,

ViInt32 step);

Purpose

Set the current value of the step, step is S0 ~ S7.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

current

Variable Type       ViInt32

Set the current value of the step.

currentValue

Variable Type       ViReal64

When Current set UserDef, then use this value.

step

Variable Type       ViInt32

Step is S0 ~ S7.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureStepVoltage

ViStatus Chr62000L_ConfigureStepVoltage (ViSession instrumentHandle,

ViInt32 voltage,

ViReal64 voltageValue,

ViInt32 step);

Purpose

Set the voltage value of the step, step is S0 ~ S7.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

voltage

Variable Type       ViInt32

Set the voltage value of the step.

voltageValue

Variable Type       ViReal64

When Voltage set UserDef, then use this value.

step

Variable Type       ViInt32

Step is S0 ~ S7.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureTRG

ViStatus Chr62000L_ConfigureTRG (ViSession instrumentHandle);

Purpose

This command is used to generate a trigger to the trigger subsystem when

the serial bus trigger is selected as the trigger source.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureTriggeredCurrent

ViStatus Chr62000L_ConfigureTriggeredCurrent (ViSession instrumentHandle,

ViInt32 current,

ViReal64 triggeredCurrent);

Purpose

This function is used to control waiting for a trigger current standards,

when the trigger value is transferred to the storage output. After

CURRent instruction will not affect any standard trigger waiting.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

current

Variable Type       ViInt32

This is used to set the trigger current.

triggeredCurrent

Variable Type       ViReal64

When Current set UserDef, this is used to control waiting for a

trigger current standards,

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureTriggeredDelay

ViStatus Chr62000L_ConfigureTriggeredDelay (ViSession instrumentHandle,

ViInt32 triggeredDelayValue,

ViInt32 triggeredDelay);

Purpose

This command sets the delay between the detection of the selected trigger

source and the start of any trigger action on the output. The setting

ranges from 0 to 3600 seconds, with a minimum of 0 seconds and a maximum

of 3600 seconds. The value * RST can be set to 0 seconds.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

triggeredDelayValue

Variable Type       ViInt32

When Triggered Delay set UserDef, then use this value.

triggeredDelay

Variable Type       ViInt32

This is used to set the trigger delay.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureTriggeredVoltage

ViStatus Chr62000L_ConfigureTriggeredVoltage (ViSession instrumentHandle,

ViReal64 triggeredVoltage,

ViInt32 voltage);

Purpose

This function is used to set the waiting trigger voltage standard, the

stored value is transferred to the output terminal when when triggered.

After VOLTage instruction will not affect any standard trigger waiting.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

triggeredVoltage

Variable Type       ViReal64

When Voltage set UserDef, this is used to control waiting for a

trigger voltage standards,

voltage

Variable Type       ViInt32

This is used to set the trigger voltage.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ConfigureTriggerSourced

ViStatus Chr62000L_ConfigureTriggerSourced (ViSession instrumentHandle,

ViInt32 source);

Purpose

Configures the trigger source the power supply waits for before changing

the output signal.  When the power supply receives a trigger signal on

the source you specify, it changes its current limit and voltage level to

the values of the CHR62000L_ATTR_CURRENT_LIMIT and

CHR62000L_ATTR_VOLTAGE_LEVEL attributes, respectively.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

source

Variable Type       ViInt32

Pass the trigger source to you want the power supply to wait for

before changing the output signal.  The driver sets the

CHR62000L_ATTR_TRIGGER_SOURCE attribute to this value.

Valid Values:

CHR62000L_VAL_TRIG_IMMEDIATE - The power supply does not wait for a

trigger to change the output signal.

CHR62000L_VAL_TRIG_BUS       - The power supply waits for a trigger

on the bus input.

Default Value: CHR62000L_VAL_TRIG_IMMEDIATE

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_error_message

ViStatus Chr62000L_error_message (ViSession instrumentHandle,

ViStatus errorCode,

ViChar _VI_FAR errorMessage[]);

Purpose

This function converts a status code returned by an instrument driver

function into a user-readable string.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

You can pass VI_NULL for this parameter.  This is useful when one of

the initialize functions fail.

Default Value:  VI_NULL

errorCode

Variable Type       ViStatus

Pass the Status parameter that is returned from any of the instrument

driver functions.

Default Value:  0  (VI_SUCCESS)

Chr62000L Status Codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

IviDCPwr Status Codes:

Status    Description

-------------------------------------------------

ERRORS:

BFFA1001  The trigger source is not software trigger.

IVI Engine Status Codes:

Status    Description

-------------------------------------------------

ERRORS:

BFFA0001  Instrument error. Call Chr62000L_error_query.

BFFA0002  Cannot open file.

BFFA0003  Error reading from file.

BFFA0004  Error writing to file.

BFFA0005  Driver module file not found.

BFFA0006  Cannot open driver module file for reading.

BFFA0007  Driver module has invalid file format or invalid data.

BFFA0008  Driver module contains undefined references.

BFFA0009  Cannot find function in driver module.

BFFA000A  Failure loading driver module.

BFFA000B  Invalid path name.

BFFA000C  Invalid attribute.

BFFA000D  IVI attribute is not writable.

BFFA000E  IVI attribute is not readable.

BFFA000F  Invalid parameter.

BFFA0010  Invalid value.

BFFA0011  Function not supported.

BFFA0012  Attribute not supported.

BFFA0013  Value not supported.

BFFA0014  Invalid type.

BFFA0015  Types do not match.

BFFA0016  Attribute already has a value waiting to be updated.

BFFA0017  Specified item already exists.

BFFA0018  Not a valid configuration.

BFFA0019  Requested item does not exist or value not available.

BFFA001A  Requested attribute value not known.

BFFA001B  No range table.

BFFA001C  Range table is invalid.

BFFA001D  Object or item is not initialized.

BFFA001E  Non-interchangeable behavior.

BFFA001F  No channel table has been built for the session.

BFFA0020  Channel name specified is not valid.

BFFA0021  Unable to allocate system resource.

BFFA0022  Permission to access file was denied.

BFFA0023  Too many files are already open.

BFFA0024  Unable to create temporary file in target directory.

BFFA0025  All temporary filenames already used.

BFFA0026  Disk is full.

BFFA0027  Cannot find configuration file on disk.

BFFA0028  Cannot open configuration file.

BFFA0029  Error reading configuration file.

BFFA002A  Invalid ViInt32 value in configuration file.

BFFA002B  Invalid ViReal64 value in configuration file.

BFFA002C  Invalid ViBoolean value in configuration file.

BFFA002D  Entry missing from configuration file.

BFFA002E  Initialization failed in driver DLL.

BFFA002F  Driver module has unresolved external reference.

BFFA0030  Cannot find CVI Run-Time Engine.

BFFA0031  Cannot open CVI Run-Time Engine.

BFFA0032  CVI Run-Time Engine has invalid format.

BFFA0033  CVI Run-Time Engine is missing required function(s).

BFFA0034  CVI Run-Time Engine initialization failed.

BFFA0035  CVI Run-Time Engine has unresolved external reference.

BFFA0036  Failure loading CVI Run-Time Engine.

BFFA0037  Cannot open DLL for read exports.

BFFA0038  DLL file is corrupt.

BFFA0039  No DLL export table in DLL.

BFFA003A  Unknown attribute name in default configuration file.

BFFA003B  Unknown attribute value in default configuration file.

BFFA003C  Memory pointer specified is not known.

BFFA003D  Unable to find any channel strings.

BFFA003E  Duplicate channel string.

BFFA003F  Duplicate virtual channel name.

BFFA0040  Missing virtual channel name.

BFFA0041  Bad virtual channel name.

BFFA0042  Unassigned virtual channel name.

BFFA0043  Bad virtual channel assignment.

BFFA0044  Channel name required.

BFFA0045  Channel name not allowed.

BFFA0046  Attribute not valid for channel.

BFFA0047  Attribute must be channel based.

BFFA0048  Channel already excluded.

BFFA0049  Missing option name (nothing before the '=').

BFFA004A  Missing option value (nothing after the '=').

BFFA004B  Bad option name.

BFFA004C  Bad option value.

BFFA004D  Operation only valid on a class driver session.

BFFA004E  "ivi.ini" filename is reserved.

BFFA004F  Duplicate run-time configuration entry.

BFFA0050  Index parameter is one-based.

BFFA0051  Index parameter is too high.

BFFA0052  Attribute is not cacheable.

BFFA0053  You cannot export a ViAddr attribute to the end-user.

BFFA0054  Bad channel string in channel string list.

BFFA0055  Bad prefix name in default configuration file.

VISA Status Codes:

Status    Description

-------------------------------------------------

WARNINGS:

3FFF0002 Event enabled for one or more specified mechanisms.

3FFF0003 Event disabled for one or more specified mechanisms.

3FFF0004 Successful, but queue already empty.

3FFF0005 Specified termination character was read.

3FFF0006 Number of bytes transferred equals input count.

3FFF0077 Configuration non-existant or could not be loaded.

3FFF007D Open successful, but the device not responding.

3FFF0080 Wait successful, but more event objects available.

3FFF0082 Specified object reference is uninitialized.

3FFF0084 Attribute value not supported.

3FFF0085 Status code could not be interpreted.

3FFF0088 Specified I/O buffer type not supported.

3FFF0098 Successful,  but invoke no handlers for this event.

3FFF0099 Successful but session has nested shared locks.

3FFF009A Successful but session has nested exclusive locks.

3FFF009B Successful but operation not asynchronous.

ERRORS:

BFFF0000 Unknown system error (miscellaneous error).

BFFF000E Session or object reference is invalid.

BFFF000F Resource is locked.

BFFF0010 Invalid expression specified for search.

BFFF0011 Resource is not present in the system.

BFFF0012 Invalid resource reference specified.  Parsing error.

BFFF0013 Invalid access mode.

BFFF0015 Timeout expired before operation completed.

BFFF0016 Unable to deallocate session data structures.

BFFF001B Specified degree is invalid.

BFFF001C Specified job identifier is invalid.

BFFF001D Attribute is not supported by the referenced object.

BFFF001E Attribute state not supported by the referenced object.

BFFF001F Specified attribute is read-only.

BFFF0020 Lock type lock not supported by this resource.

BFFF0021 Invalid access key.

BFFF0026 Specified event type not supported by the resource.

BFFF0027 Invalid mechanism specified.

BFFF0028 A handler was not installed.

BFFF0029 Handler reference either invalid or was not installed.

BFFF002A Specified event context invalid.

BFFF002D Event queue for specified type has overflowed.

BFFF002F Event type must be enabled in order to receive.

BFFF0030 User abort during transfer.

BFFF0034 Violation of raw write protocol during transfer.

BFFF0035 Violation of raw read protocol during transfer.

BFFF0036 Device reported output protocol error during transfer.

BFFF0037 Device reported input protocol error during transfer.

BFFF0038 Bus error during transfer.

BFFF0039 Unable to queue asynchronous operation.

BFFF003A Unable to start operation because setup is invalid.

BFFF003B Unable to queue the asynchronous operation.

BFFF003C Insufficient resources to perform memory allocation.

BFFF003D Invalid buffer mask specified.

BFFF003E I/O error.

BFFF003F Format specifier invalid.

BFFF0041 Format specifier not supported.

BFFF0042 Trigger line is currently in use.

BFFF004A Service request not received for the session.

BFFF004E Invalid address space specified.

BFFF0051 Invalid offset specified.

BFFF0052 Invalid access width specified.

BFFF0054 Offset not accessible from this hardware.

BFFF0055 Source and destination widths are different.

BFFF0057 Session not currently mapped.

BFFF0059 Previous response still pending.

BFFF005F No listeners condition detected.

BFFF0060 Interface not currently the controller in charge.

BFFF0061 Interface not the system controller.

BFFF0067 Session does not support this operation.

BFFF006A A parity error occurred during transfer.

BFFF006B A framing error occurred during transfer.

BFFF006C An overrun error occurred during transfer.

BFFF0070 Offset not properly aligned for operation access width.

BFFF0071 Specified user buffer not valid.

BFFF0072 Resource valid, but VISA cannot access it.

BFFF0076 Width not supported by this hardware.

BFFF0078 Invalid parameter value, parameter unknown.

BFFF0079 Invalid protocol.

BFFF007B Invalid window size.

BFFF0080 Session currently contains a mapped window.

BFFF0081 Operation not implemented.

BFFF0083 Invalid length.

BFFF0091 Invalid mode.

BFFF009C Session did not have a lock on the resource.

BFFF009D The device does not export any memory.

BFFF009E VISA-required code library not located or not loaded.

VXIPnP Driver Status Codes:

Status    Description

-------------------------------------------------

WARNINGS:

3FFC0101  Instrument does not have ID Query capability.

3FFC0102  Instrument does not have Reset capability.

3FFC0103  Instrument does not have Self-Test capability.

3FFC0104  Instrument does not have Error Query capability.

3FFC0105  Instrument does not have Revision Query capability.

ERRORS:

BFFC0001  Parameter 1 out of range, or error trying to set it.

BFFC0002  Parameter 2 out of range, or error trying to set it.

BFFC0003  Parameter 3 out of range, or error trying to set it.

BFFC0004  Parameter 4 out of range, or error trying to set it.

BFFC0005  Parameter 5 out of range, or error trying to set it.

BFFC0006  Parameter 6 out of range, or error trying to set it.

BFFC0007  Parameter 7 out of range, or error trying to set it.

BFFC0008  Parameter 8 out of range, or error trying to set it.

BFFC0011  Instrument failed the ID Query.

BFFC0012  Invalid response from instrument.

errorMessage

Variable Type       ViChar[]

Returns the user-readable message string that corresponds to the

status code you specify.

You must pass a ViChar array with at least 256 bytes.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_error_query

ViStatus Chr62000L_error_query (ViSession instrumentHandle,

ViPInt32 errorCode,

ViChar _VI_FAR errorMessage[]);

Purpose

This function reads an error code and a message from the instrument's

error queue.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

errorCode

Variable Type       ViInt32 (passed by reference)

Returns the error code read from the instrument's error queue.

errorMessage

Variable Type       ViChar[]

Returns the error message string read from the instrument's error

message queue.

You must pass a ViChar array with at least 256 bytes.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_GetAttributeViBoolean

ViStatus Chr62000L_GetAttributeViBoolean (ViSession instrumentHandle,

ViConstString channelName,

ViAttr attributeID,

ViPBoolean attributeValue);

Purpose

This function queries the value of a ViBoolean attribute.

You can use this function to get the values of instrument- specific

attributes and inherent IVI attributes.  If the attribute represents an

instrument state, this function performs instrument I/O in the following

cases:

- State caching is disabled for the entire session or for the particular

attribute.

- State caching is enabled and the currently cached value is invalid.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViConstString

If the attribute is channel-based, this parameter specifies the name

of the channel on which to obtain the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Help text is

shown for each attribute.  Select an attribute by

double-clicking on it or by selecting it and then pressing

<ENTER>.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViBoolean

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViBoolean are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViBoolean (passed by reference)

Returns the current value of the attribute.  Pass the address of a

ViBoolean variable.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has named constants as valid values, you can view a

list of the constants by pressing <ENTER> on this control.

Select a value by double-clicking on it or by selecting it and

then pressing <ENTER>.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_GetAttributeViInt32

ViStatus Chr62000L_GetAttributeViInt32 (ViSession instrumentHandle,

ViConstString channelName,

ViAttr attributeID,

ViPInt32 attributeValue);

Purpose

This function queries the value of a ViInt32 attribute.

You can use this function to get the values of instrument- specific

attributes and inherent IVI attributes.  If the attribute represents an

instrument state, this function performs instrument I/O in the following

cases:

- State caching is disabled for the entire session or for the particular

attribute.

- State caching is enabled and the currently cached value is invalid.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViConstString

If the attribute is channel-based, this parameter specifies the name

of the channel on which to obtain the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Help text is

shown for each attribute.  Select an attribute by

double-clicking on it or by selecting it and then pressing

<ENTER>.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViInt32 type.

If you choose to see all IVI attributes, the data types appear

to the right of the attribute names in the list box.

Attributes with data types other than ViInt32 are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViInt32 (passed by reference)

Returns the current value of the attribute.  Pass the address of a

ViInt32 variable.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has named constants as valid values, you can view a

list of the constants by pressing <ENTER> on this control.

Select a value by double-clicking on it or by selecting it and

then pressing <ENTER>.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_GetAttributeViReal64

ViStatus Chr62000L_GetAttributeViReal64 (ViSession instrumentHandle,

ViConstString channelName,

ViAttr attributeID,

ViPReal64 attributeValue);

Purpose

This function queries the value of a ViReal64 attribute.

You can use this function to get the values of instrument- specific

attributes and inherent IVI attributes.  If the attribute represents an

instrument state, this function performs instrument I/O in the following

cases:

- State caching is disabled for the entire session or for the particular

attribute.

- State caching is enabled and the currently cached value is invalid.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViConstString

If the attribute is channel-based, this parameter specifies the name

of the channel on which to obtain the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Help text is

shown for each attribute.  Select an attribute by

double-clicking on it or by selecting it and then pressing

<ENTER>.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViReal64

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViReal64 are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViReal64 (passed by reference)

Returns the current value of the attribute.  Pass the address of a

ViReal64 variable.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has named constants as valid values, you can view a

list of the constants by pressing <ENTER> on this control.

Select a value by double-clicking on it or by selecting it and

then pressing <ENTER>.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_GetAttributeViSession

ViStatus Chr62000L_GetAttributeViSession (ViSession instrumentHandle,

ViConstString channelName,

ViAttr attributeID,

ViPSession attributeValue);

Purpose

This function queries the value of a ViSession attribute.

You can use this function to get the values of instrument- specific

attributes and inherent IVI attributes.  If the attribute represents an

instrument state, this function performs instrument I/O in the following

cases:

- State caching is disabled for the entire session or for the particular

attribute.

- State caching is enabled and the currently cached value is invalid.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViConstString

If the attribute is channel-based, this parameter specifies the name

of the channel on which to obtain the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Help text is

shown for each attribute.  Select an attribute by

double-clicking on it or by selecting it and then pressing

<ENTER>.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViSession

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViSession are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViSession (passed by reference)

Returns the current value of the attribute.  Pass the address of a

ViSession variable.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has named constants as valid values, you can view a

list of the constants by pressing <ENTER> on this control.

Select a value by double-clicking on it or by selecting it and

then pressing <ENTER>.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_GetAttributeViString

ViStatus Chr62000L_GetAttributeViString (ViSession instrumentHandle,

ViConstString channelName,

ViAttr attributeID,

ViInt32 bufferSize,

ViChar _VI_FAR attributeValue[]);

Purpose

This function queries the value of a ViString attribute.

You can use this function to get the values of instrument- specific

attributes and inherent IVI attributes.  If the attribute represents an

instrument state, this function performs instrument I/O in the following

cases:

- State caching is disabled for the entire session or for the particular

attribute.

- State caching is enabled and the currently cached value is invalid.

You must provide a ViChar array to serve as a buffer for the value.  You

pass the number of bytes in the buffer as the Buffer Size parameter.  If

the current value of the attribute, including the terminating NUL byte,

is larger than the size you indicate in the Buffer Size parameter, the

function copies Buffer Size - 1 bytes into the buffer, places an ASCII

NUL byte at the end of the buffer, and returns the buffer size you must

pass to get the entire value.  For example, if the value is "123456" and

the Buffer Size is 4, the function places "123" into the buffer and

returns 7.

If you want to call this function just to get the required buffer size,

you can pass 0 for the Buffer Size and VI_NULL for the Attribute Value

buffer.

If you want the function to fill in the buffer regardless of the   number

of bytes in the value, pass a negative number for the Buffer Size

parameter.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViConstString

If the attribute is channel-based, this parameter specifies the name

of the channel on which to obtain the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Help text is

shown for each attribute.  Select an attribute by

double-clicking on it or by selecting it and then pressing

<ENTER>.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViString

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViString are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

bufferSize

Variable Type       ViInt32

Pass the number of bytes in the ViChar array you specify for the

Attribute Value parameter.

If the current value of the attribute, including the terminating NUL

byte, contains more bytes that you indicate in this parameter, the

function copies Buffer Size - 1 bytes into the buffer, places an

ASCII NUL byte at the end of the buffer, and returns the buffer size

you must pass to get the entire value.  For example, if the value is

"123456" and the Buffer Size is 4, the function places "123" into the

buffer and returns 7.

If you pass a negative number, the function copies the value to the

buffer regardless of the number of bytes in the value.

If you pass 0, you can pass VI_NULL for the Attribute Value buffer

parameter.

Default Value: 512

attributeValue

Variable Type       ViChar[]

The buffer in which the function returns the current value of the

attribute.  The buffer must be of type ViChar and have at least as

many bytes as indicated in the Buffer Size parameter.

If the current value of the attribute, including the terminating NUL

byte, contains more bytes that you indicate in this parameter, the

function copies Buffer Size - 1 bytes into the buffer, places an

ASCII NUL byte at the end of the buffer, and returns the buffer size

you must pass to get the entire value.  For example, if the value is

"123456" and the Buffer Size is 4, the function places "123" into the

buffer and returns 7.

If you specify 0 for the Buffer Size parameter, you can pass VI_NULL

for this parameter.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has named constants as valid values, you can view a

list of the constants by pressing <ENTER> on this control.

Select a value by double-clicking on it or by selecting it and

then pressing <ENTER>.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

If the current value of the return buffer, including the terminating

NUL byte, is larger than the size you indicate in the Buffer Size

parameter, the function copies Buffer Size - 1 bytes into the buffer,

places an ASCII NUL byte at the end of the buffer, and returns the

buffer size you must pass to get the entire value.  For example, if

the value is "123456" and the Buffer Size is 4, the function places

"123" into the buffer and returns 7.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_GetError

ViStatus Chr62000L_GetError (ViSession instrumentHandle, ViPStatus code,

ViInt32 bufferSize,

ViChar _VI_FAR description[]);

Purpose

This function retrieves and then clears the IVI error information for the

session or the current execution thread. One exception exists: If the

BufferSize parameter is 0, the function does not clear the error

information. By passing 0 for the buffer size, the caller can ascertain

the buffer size required to get the entire error description string and

then call the function again with a sufficiently large buffer.

If the user specifies a valid IVI session for the InstrumentHandle

parameter, Get Error retrieves and then clears the error information for

the session.  If the user passes VI_NULL for the InstrumentHandle

parameter, this function retrieves and then clears the error information

for the current execution thread.  If the InstrumentHandle parameter is

an invalid session, the function does nothing and returns an error.

Normally, the error information describes the first error that occurred

since the user last called Chr62000L_GetError or Chr62000L_ClearError.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

code

Variable Type       ViStatus (passed by reference)

Returns the error code for the session or execution thread.

If you pass 0 for the Buffer Size, you can pass VI_NULL for this

parameter.

bufferSize

Variable Type       ViInt32

Pass the number of bytes in the ViChar array you specify for the

Description parameter.

If the error description, including the terminating NUL byte,

contains more bytes than you indicate in this parameter, the function

copies BufferSize - 1 bytes into the buffer, places an ASCII NUL byte

at the end of the buffer, and returns the buffer size you must pass

to get the entire value.  For example, if the value is "123456" and

the Buffer Size is 4, the function places "123" into the buffer and

returns 7.

If you pass a negative number, the function copies the value to the

buffer regardless of the number of bytes in the value.

If you pass 0, you can pass VI_NULL for the Description buffer

parameter.

Default Value:  None

description

Variable Type       ViChar[]

Returns the error description for the IVI session or execution

thread.  If there is no description, the function returns an empty

string.

The buffer must contain at least as many elements as the value you

specify with the Buffer Size parameter.  If the error description,

including the terminating NUL byte, contains more bytes than you

indicate with the Buffer Size parameter, the function copies Buffer

Size - 1 bytes into the buffer, places an ASCII NUL byte at the end

of the buffer, and returns the buffer size you must pass to get the

entire value.  For example, if the value is "123456" and the Buffer

Size is 4, the function places "123" into the buffer and returns 7.

If you pass 0 for the Buffer Size, you can pass VI_NULL for this

parameter.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

If the current value of the return buffer, including the terminating

NUL byte, is larger than the size you indicate in the Buffer Size

parameter, the function copies Buffer Size - 1 bytes into the buffer,

places an ASCII NUL byte at the end of the buffer, and returns the

buffer size you must pass to get the entire value.  For example, if

the value is "123456" and the Buffer Size is 4, the function places

"123" into the buffer and returns 7.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_GetNextCoercionRecord

ViStatus Chr62000L_GetNextCoercionRecord (ViSession instrumentHandle,

ViInt32 bufferSize,

ViChar _VI_FAR coercionRecord[]);

Purpose

This function returns the coercion information associated with the IVI

session.  This function retrieves and clears the oldest instance in which

the instrument driver coerced a value you specified to another value.

If you set the CHR62000L_ATTR_RECORD_COERCIONS attribute to VI_TRUE, the

instrument driver keeps a list of all coercions it makes on ViInt32 or

ViReal64 values you pass to instrument driver functions.  You use this

function to retrieve information from that list.

If the next coercion record string, including the terminating NUL byte,

contains more bytes than you indicate in this parameter, the function

copies Buffer Size - 1 bytes into the buffer, places an ASCII NUL byte at

the end of the buffer, and returns the buffer size you must pass to get

the entire value.  For example, if the value is "123456" and the Buffer

Size is 4, the function places "123" into the buffer and returns 7.

If you pass a negative number, the function copies the value to the

buffer regardless of the number of bytes in the value.

If you pass 0, you can pass VI_NULL for the Coercion Record buffer

parameter.

The function returns an empty string in the Coercion Record parameter if

no coercion records remain for the session.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init

function.  The handle identifies a particular instrument session.

Default Value:  None

bufferSize

Variable Type       ViInt32

Pass the number of bytes in the ViChar array you specify for the

Coercion Record parameter.

If the next coercion record string, including the terminating NUL

byte, contains more bytes than you indicate in this parameter, the

function copies Buffer Size - 1 bytes into the buffer, places an

ASCII NUL byte at the end of the buffer, and returns the buffer size

you must pass to get the entire value.  For example, if the value is

"123456" and the Buffer Size is 4, the function places "123" into the

buffer and returns 7.

If you pass a negative number, the function copies the value to the

buffer regardless of the number of bytes in the value.

If you pass 0, you can pass VI_NULL for the Coercion Record buffer

parameter.

Default Value:  None

coercionRecord

Variable Type       ViChar[]

Returns the next coercion record for the IVI session.  If there are

no coercion records, the function returns an empty string.

The buffer must contain at least as many elements as the value you

specify with the Buffer Size parameter.  If the next coercion record

string, including the terminating NUL byte, contains more bytes than

you indicate with the Buffer Size parameter, the function copies

Buffer Size - 1 bytes into the buffer, places an ASCII NUL byte at

the end of the buffer, and returns the buffer size you must pass to

get the entire value.  For example, if the value is "123456" and the

Buffer Size is 4, the function places "123" into the buffer and

returns 7.

This parameter returns an empty string if no coercion records remain

for the session.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

If the current value of the return buffer, including the terminating

NUL byte, is larger than the size you indicate in the Buffer Size

parameter, the function copies Buffer Size - 1 bytes into the buffer,

places an ASCII NUL byte at the end of the buffer, and returns the

buffer size you must pass to get the entire value.  For example, if

the value is "123456" and the Buffer Size is 4, the function places

"123" into the buffer and returns 7.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_GetNextInterchangeWarning

ViStatus Chr62000L_GetNextInterchangeWarning (ViSession instrumentHandle,

ViInt32 bufferSize,

ViChar _VI_FAR interchangeWarning[]);

Purpose

This function returns the interchangeability warnings associated with the

IVI session. It retrieves and clears the oldest instance in which the

class driver recorded an interchangeability warning.  Interchangeability

warnings indicate that using your application with a different instrument

might cause different behavior. You use this function to retrieve

interchangeability warnings.

The driver performs interchangeability checking when the

CHR62000L_ATTR_INTERCHANGE_CHECK attribute is set to VI_TRUE.

The function returns an empty string in the Interchange Warning parameter

if no interchangeability warnings remain for the session.

In general, the instrument driver generates interchangeability warnings

when an attribute that affects the behavior of the instrument is in a

state that you did not specify.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

bufferSize

Variable Type       ViInt32

Pass the number of bytes in the ViChar array you specify for the

Interchange Warning parameter.

If the next interchangeability warning string, including the

terminating NUL byte, contains more bytes than you indicate in this

parameter, the function copies Buffer Size - 1 bytes into the buffer,

places an ASCII NUL byte at the end of the buffer, and returns the

buffer size you must pass to get the entire value. For example, if

the value is "123456" and the Buffer Size is 4, the function places

"123" into the buffer and returns 7.

If you pass a negative number, the function copies the value to the

buffer regardless of the number of bytes in the value.

If you pass 0, you can pass VI_NULL for the Interchange Warning

buffer parameter.

Default Value:  None

interchangeWarning

Variable Type       ViChar[]

Returns the next interchange warning for the IVI session. If there

are no interchange warnings, the function returns an empty  string.

The buffer must contain at least as many elements as the value you

specify with the Buffer Size parameter. If the next

interchangeability warning string, including the terminating NUL

byte, contains more bytes than you indicate with the Buffer Size

parameter, the function copies Buffer Size - 1 bytes into the buffer,

places an ASCII NUL byte at the end of the buffer, and returns the

buffer size you must pass to get the entire value.  For example, if

the value is "123456" and the Buffer Size is 4, the function places

"123" into the buffer and returns 7.

This parameter returns an empty string if no interchangeability

warnings remain for the session.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

If the current value of the return buffer, including the terminating

NUL byte, is larger than the size you indicate in the Buffer Size

parameter, the function copies Buffer Size - 1 bytes into the buffer,

places an ASCII NUL byte at the end of the buffer, and returns the

buffer size you must pass to get the entire value.  For example, if

the value is "123456" and the Buffer Size is 4, the function places

"123" into the buffer and returns 7.

To obtain a text description of the status code, or if the status

code is not listed below, call the Chr62000L_error_message function.

To obtain additional information about the error condition, use the

Chr62000L_GetError and Chr62000L_ClearError functions.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_init

ViStatus Chr62000L_init (ViRsrc resourceName, ViBoolean IDQuery,

ViBoolean resetDevice,

ViPSession instrumentHandle);

Purpose

This function performs the following initialization actions:

- Creates a new IVI instrument driver session.

- Opens a session to the specified device using the interface and address

you specify for the Resource Name parameter.

- If the ID Query parameter is set to VI_TRUE, this function queries the

instrument ID and checks that it is valid for this instrument driver.

- If the Reset parameter is set to VI_TRUE, this function resets the

instrument to a known state.

- Sends initialization commands to set the instrument to the state

necessary for the operation of the instrument driver.

- Returns a ViSession handle that you use to identify the instrument in

all subsequent instrument driver function calls.

Note:  This function creates a new session each time you invoke it.

Although you can open more than one IVI session for the same resource, it

is best not to do so.  You can use the same session in multiple program

threads.  You can use the Chr62000L_LockSession and

Chr62000L_UnlockSession functions to protect sections of code that

require exclusive access to the resource.

Content from the IVI specifications reproduced with permission from the

IVI Foundation.

The IVI Foundation and its member companies make no warranty of any kind

with regard to this material, including, but not limited to, the implied

warranties of merchantability and fitness for a particular purpose. The

IVI Foundation and its member companies shall not be liable for errors

contained herein or for incidental or consequential damages in connection

with the furnishing, performance, or use of this material.

Parameter List

resourceName

Variable Type       ViRsrc

Pass the resource name of the device to initialize.

You can also pass the name of a virtual instrument or logical name

that you configure with the IVI Configuration utility.  The virtual

instrument identifies a specific device and specifies the initial

settings for the session.  A logical Name identifies a particular

virtual instrument.

Refer to the following table below for the exact grammar to use for

this parameter.  Optional fields are shown in square brackets ([]).

Syntax

------------------------------------------------------

GPIB[board]::<primary address>[::secondary address]::INSTR

VXI[board]::<logical address>::INSTR

GPIB-VXI[board]::<logical address>::INSTR

ASRL<port>::INSTR

<LogicalName>

<DriverSession>

If you do not specify a value for an optional field, the following

values are used:

Optional Field - Value

------------------------------------------------------

board - 0

secondary address - none (31)

The following table contains example valid values for this parameter.

"Valid Value" - Description

------------------------------------------------------

"GPIB::22::INSTR" - GPIB board 0, primary address 22 no

secondary address

"GPIB::22::5::INSTR" - GPIB board 0, primary address 22

secondary address 5

"GPIB1::22::5::INSTR" - GPIB board 1, primary address 22

secondary address 5

"VXI::64::INSTR" - VXI board 0, logical address 64

"VXI1::64::INSTR" - VXI board 1, logical address 64

"GPIB-VXI::64::INSTR" - GPIB-VXI board 0, logical address 64

"GPIB-VXI1::64::INSTR" - GPIB-VXI board 1, logical address 64

"ASRL2::INSTR" - COM port 2

"SampleInstr" - Logical name "SampleInstr"

"xyz432" - Logical Name or Driver Session "xyz432"

/*=CHANGE:===================================================*

Modify the following default value so that it reflects the default

address for your instrument.  You must make the corresponding change

to the Default Value entry for the control.

*================================================END=CHANGE=*/

Default Value:  "USB0::0x0699::0x0408::Q001074::INSTR"

IDQuery

Variable Type       ViBoolean

Specify whether you want the instrument driver to perform an ID

Query.

Valid Range:

VI_TRUE  (1) - Perform ID Query (Default Value)

VI_FALSE (0) - Skip ID Query

When you set this parameter to VI_TRUE, the driver verifies that the

instrument you initialize is a type that this driver supports.

Circumstances can arise where it is undesirable to send an ID Query

command string to the instrument.  When you set this parameter to

VI_FALSE, the function initializes the instrument without performing

an ID Query.

resetDevice

Variable Type       ViBoolean

Specify whether you want the to reset the instrument during the

initialization procedure.

Valid Range:

VI_TRUE  (1) - Reset Device (Default Value)

VI_FALSE (0) - Don't Reset

instrumentHandle

Variable Type       ViSession (passed by reference)

Returns a ViSession handle that you use to identify the instrument in

all subsequent instrument driver function calls.

Notes:

(1) This function creates a new session each time you invoke it.

This is useful if you have multiple physical instances of the same

type of instrument.

(2) Avoid creating multiple concurrent sessions to the same physical

instrument.  Although you can create more than one IVI session for

the same resource, it is best not to do so.  A better approach is to

use the same IVI session in multiple execution threads.  You can use

functions Chr62000L_LockSession and Chr62000L_UnlockSession to

protect sections of code that require exclusive access to the

resource.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_Initiate

ViStatus Chr62000L_Initiate (ViSession instrumentHandle);

Purpose

This function initiates output changes that you previously specified.

If the power supply is not currently waiting for a trigger, the power

supply waits for the trigger you specify with the

Chr62000L_ConfigureTriggerSource function after you call this function.

After the power supply detects the trigger, it updates its voltage level

and current limit to the values you specify with the

Chr62000L_ConfigureTriggeredVoltageLevel and

Chr62000L_ConfigureTriggeredCurrentLimit functions.

If the power supply is already waiting for a trigger, this function does

nothing and returns VI_SUCCESS.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_InitWithOptions

ViStatus Chr62000L_InitWithOptions (ViRsrc resourceName,

ViBoolean IDQuery,

ViBoolean resetDevice,

ViChar _VI_FAR optionString[],

ViPSession instrumentHandle);

Purpose

This function performs the following initialization actions:

- Creates a new IVI instrument driver and optionally sets the initial

state of the following session attributes:

CHR62000L_ATTR_RANGE_CHECK

CHR62000L_ATTR_QUERY_INSTRUMENT_STATUS

CHR62000L_ATTR_CACHE

CHR62000L_ATTR_SIMULATE

CHR62000L_ATTR_RECORD_COERCIONS

- Opens a session to the specified device using the interface and address

you specify for the Resource Name parameter.

- If the ID Query parameter is set to VI_TRUE, this function queries the

instrument ID and checks that it is valid for this instrument driver.

- If the Reset parameter is set to VI_TRUE, this function resets the

instrument to a known state.

- Sends initialization commands to set the instrument to the state

necessary for the operation of the instrument driver.

- Returns a ViSession handle that you use to identify the instrument in

all subsequent instrument driver function calls.

Note:  This function creates a new session each time you invoke it.

Although you can open more than one IVI session for the same resource, it

is best not to do so.  You can use the same session in multiple program

threads.  You can use the Chr62000L_LockSession and

Chr62000L_UnlockSession functions to protect sections of code that

require exclusive access to the resource.

Content from the IVI specifications reproduced with permission from the

IVI Foundation.

The IVI Foundation and its member companies make no warranty of any kind

with regard to this material, including, but not limited to, the implied

warranties of merchantability and fitness for a particular purpose. The

IVI Foundation and its member companies shall not be liable for errors

contained herein or for incidental or consequential damages in connection

with the furnishing, performance, or use of this material.

Parameter List

resourceName

Variable Type       ViRsrc

Pass the resource name of the device to initialize.

You can also pass the name of a virtual instrument or logical name

that you configure with the IVI Configuration utility.  The virtual

instrument identifies a specific device and specifies the initial

settings for the session.  A logical Name identifies a particular

virtual instrument.

Refer to the following table below for the exact grammar to use for

this parameter.  Optional fields are shown in square brackets ([]).

Syntax

------------------------------------------------------

GPIB[board]::<primary address>[::secondary address]::INSTR

VXI[board]::<logical address>::INSTR

GPIB-VXI[board]::<logical address>::INSTR

ASRL<port>::INSTR

<LogicalName>

<DriverSession>

If you do not specify a value for an optional field, the following

values are used:

Optional Field - Value

------------------------------------------------------

board - 0

secondary address - none (31)

The following table contains example valid values for this parameter.

"Valid Value" - Description

------------------------------------------------------

"GPIB::22::INSTR" - GPIB board 0, primary address 22 no

secondary address

"GPIB::22::5::INSTR" - GPIB board 0, primary address 22

secondary address 5

"GPIB1::22::5::INSTR" - GPIB board 1, primary address 22

secondary address 5

"VXI::64::INSTR" - VXI board 0, logical address 64

"VXI1::64::INSTR" - VXI board 1, logical address 64

"GPIB-VXI::64::INSTR" - GPIB-VXI board 0, logical address 64

"GPIB-VXI1::64::INSTR" - GPIB-VXI board 1, logical address 64

"ASRL2::INSTR" - COM port 2

"SampleInstr" - Logical name "SampleInstr"

"xyz432" - Logical Name or Driver Session "xyz432"

/*=CHANGE:===================================================*

Modify the following default value so that it reflects the default

address for your instrument.  You must make the corresponding change

to the Default Value entry for the control.

*================================================END=CHANGE=*/

Default Value:  "USB0::0x0699::0x0408::Q001074::INSTR"

IDQuery

Variable Type       ViBoolean

Specify whether you want the instrument driver to perform an ID

Query.

Valid Range:

VI_TRUE  (1) - Perform ID Query (Default Value)

VI_FALSE (0) - Skip ID Query

When you set this parameter to VI_TRUE, the driver verifies that the

instrument you initialize is a type that this driver supports.

Circumstances can arise where it is undesirable to send an ID Query

command string to the instrument.  When you set this parameter to

VI_FALSE, the function initializes the instrument without performing

an ID Query.

resetDevice

Variable Type       ViBoolean

Specify whether you want the to reset the instrument during the

initialization procedure.

Valid Range:

VI_TRUE  (1) - Reset Device (Default Value)

VI_FALSE (0) - Don't Reset

optionString

Variable Type       ViChar[]

You can use this control to set the initial value of certain

attributes for the session.  The following table lists the attributes

and the name you use in this parameter to identify the attribute.

Name              Attribute Defined Constant

--------------------------------------------

RangeCheck        CHR62000L_ATTR_RANGE_CHECK

QueryInstrStatus  CHR62000L_ATTR_QUERY_INSTRUMENT_STATUS

Cache             CHR62000L_ATTR_CACHE

Simulate          CHR62000L_ATTR_SIMULATE

RecordCoercions   CHR62000L_ATTR_RECORD_COERCIONS

The format of this string is, "AttributeName=Value" where

AttributeName is the name of the attribute and Value is the value to

which the attribute will be set.  To set multiple attributes,

separate their assignments with a comma.

If you pass NULL or an empty string for this parameter and a VISA

resource descriptor for the Resource Name parameter, the session uses

the default values for the attributes. The default values for the

attributes are shown below:

Attribute Name     Default Value

----------------   -------------

RangeCheck         VI_TRUE

QueryInstrStatus   VI_FALSE

Cache              VI_TRUE

Simulate           VI_FALSE

RecordCoercions    VI_FALSE

If you pass NULL or an empty string for this parameter and a virtual

instrument or logical name for the Resource Name parameter, the

session uses the values that you configure for virtual instrument or

logical name with the IVI Configuration utility.

You can override the values of the attributes by assigning a value

explicitly in a string you pass for this parameter.  You do not have

to specify all of the attributes and may leave any of them out.  If

you do not specify one of the attributes, its default value or the

value that you configure with the IVI Configuration utility will be

used.

The following are the valid values for ViBoolean attributes:

True:     1, TRUE, or VI_TRUE

False:    0, False, or VI_FALSE

Default Value:

"Simulate=0,RangeCheck=1,QueryInstrStatus=0,Cache=1"

instrumentHandle

Variable Type       ViSession (passed by reference)

Returns a ViSession handle that you use to identify the instrument in

all subsequent instrument driver function calls.

Notes:

(1) This function creates a new session each time you invoke it.

This is useful if you have multiple physical instances of the same

type of instrument.

(2) Avoid creating multiple concurrent sessions to the same physical

instrument.  Although you can create more than one IVI session for

the same resource, it is best not to do so.  A better approach is to

use the same IVI session in multiple execution threads.  You can use

functions Chr62000L_LockSession and Chr62000L_UnlockSession to

protect sections of code that require exclusive access to the

resource.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_InvalidateAllAttributes

ViStatus Chr62000L_InvalidateAllAttributes (ViSession instrumentHandle);

Purpose

This function invalidates the cached values of all attributes for the

session.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_LockSession

ViStatus Chr62000L_LockSession (ViSession instrumentHandle,

ViPBoolean callerHasLock);

Purpose

This function obtains a multithread lock on the instrument session.

Before it does so, it waits until all other execution threads have

released their locks on the instrument session.

Other threads might have obtained a lock on this session in the following

ways:

- The user's application called Chr62000L_LockSession.

- A call to the instrument driver locked the session.

- A call to the IVI engine locked the session.

After your call to Chr62000L_LockSession returns successfully, no other

threads can access the instrument session until you call

Chr62000L_UnlockSession.

Use Chr62000L_LockSession and Chr62000L_UnlockSession around a sequence

of calls to instrument driver functions if you require that the

instrument retain its settings through the end of the sequence.

You can safely make nested calls to Chr62000L_LockSession within the same

thread.  To completely unlock the session, you must balance each call to

Chr62000L_LockSession with a call to Chr62000L_UnlockSession.  If,

however, you use the Caller Has Lock parameter in all calls to

Chr62000L_LockSession and Chr62000L_UnlockSession within a function, the

IVI Library locks the session only once within the function regardless of

the number of calls you make to Chr62000L_LockSession.  This allows you

to call Chr62000L_UnlockSession just once at the end of the function.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

callerHasLock

Variable Type       ViBoolean (passed by reference)

This parameter serves as a convenience.  If you do not want to use

this parameter, pass VI_NULL.

Use this parameter in complex functions to keep track of whether you

obtain a lock and therefore need to unlock the session.  Pass the

address of a local ViBoolean variable.  In the declaration of the

local variable, initialize it to VI_FALSE.  Pass the address of the

same local variable to any other calls you make to

Chr62000L_LockSession or Chr62000L_UnlockSession in the same

function.

The parameter is an input/output parameter.  Chr62000L_LockSession

and Chr62000L_UnlockSession each inspect the current value and take

the following actions:

- If the value is VI_TRUE, Chr62000L_LockSession does not lock the

session again.  If the value is VI_FALSE, Chr62000L_LockSession

obtains the lock and sets the value of the parameter to VI_TRUE.

- If the value is VI_FALSE, Chr62000L_UnlockSession does not attempt

to unlock the session.  If the value is VI_TRUE,

Chr62000L_UnlockSession releases the lock and sets the value of the

parameter to VI_FALSE.

Thus, you can, call Chr62000L_UnlockSession at the end of your

function without worrying about whether you actually have the lock.

Example:

ViStatus TestFunc (ViSession vi, ViInt32 flags)

{

ViStatus error = VI_SUCCESS;

ViBoolean haveLock = VI_FALSE;

if (flags & BIT_1)

{

viCheckErr( Chr62000L_LockSession(vi, &haveLock));

viCheckErr( TakeAction1(vi));

if (flags & BIT_2)

{

viCheckErr( Chr62000L_UnlockSession(vi, &haveLock));

viCheckErr( TakeAction2(vi));

viCheckErr( Chr62000L_LockSession(vi, &haveLock);

}

if (flags & BIT_3)

viCheckErr( TakeAction3(vi));

}

Error:

/*

At this point, you cannot really be sure that

you have the lock.  Fortunately, the haveLock

variable takes care of that for you.

*/

Chr62000L_UnlockSession(vi, &haveLock);

return error;

}

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_Measured

ViStatus Chr62000L_Measured (ViSession instrumentHandle,

ViInt32 measurementType,

ViPReal64 measurement);

Purpose

This function takes a measurement on the output signal and returns the

measured value.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

measurementType

Variable Type       ViInt32

Pass the measurement you want the power supply to take.

Valid Values:

CHR62000L_VAL_MEASURE_VOLTAGE - DC voltage measurement

CHR62000L_VAL_MEASURE_CURRENT - DC current measurement

Default Value: CHR62000L_VAL_MEASURE_VOLTAGE

measurement

Variable Type       ViReal64 (passed by reference)

Returns the measured value.

Units: volts (for voltage measurement)

amps  (for current measurement)

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryCalibrationMessage

ViStatus Chr62000L_QueryCalibrationMessage (ViSession instrumentHandle,

ViChar _VI_FAR message[]);

Purpose

Query the calibration string.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

message

Variable Type       ViChar[]

Query the calibration string.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryCalibrationNumber

ViStatus Chr62000L_QueryCalibrationNumber (ViSession instrumentHandle,

ViPInt32 number);

Purpose

Query the number of calibrations.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

number

Variable Type       ViInt32 (passed by reference)

Query the number of calibrations.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryCalibrationProtectionEnabled

ViStatus Chr62000L_QueryCalibrationProtectionEnabled

(ViSession instrumentHandle, ViPBoolean enabled);

Purpose

Ask to correct the protection status. The return value 1 indicates that

the protection status is ON and the return value 0 indicates that the

protection status is OFF.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enabled

Variable Type       ViBoolean (passed by reference)

Ask to correct the protection status. The return value 1 indicates

that the protection status is ON and the return value 0 indicates

that the protection status is OFF.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryCCLimit

ViStatus Chr62000L_QueryCCLimit (ViSession instrumentHandle,

ViInt32 current,

ViPReal64 currentValue,

ViInt32 currentStep,

ViPReal64 userDefineCurrentStepValue);

Purpose

This is used to query the power of the CC limit.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

current

Variable Type       ViInt32

Query the current value.

currentValue

Variable Type       ViReal64 (passed by reference)

Query the current value.

currentStep

Variable Type       ViInt32

Query the user define or default current amplitude.

userDefineCurrentStepValue

Variable Type       ViReal64 (passed by reference)

Query the current amplitude.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryContinuousOutputCycles

ViStatus Chr62000L_QueryContinuousOutputCycles

(ViSession instrumentHandle, ViPInt32 cycles);

Purpose

Query the number of cycles for continuous output.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

cycles

Variable Type       ViInt32 (passed by reference)

Query the number of cycles for continuous output.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryContinuousOutputFinish

ViStatus Chr62000L_QueryContinuousOutputFinish

(ViSession instrumentHandle, ViPBoolean enable);

Purpose

This is used to query the continuous output to be finish.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enable

Variable Type       ViBoolean (passed by reference)

This is used to query the continuous output to be finish.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryContinuousOutputMode

ViStatus Chr62000L_QueryContinuousOutputMode (ViSession instrumentHandle,

ViChar _VI_FAR mode[]);

Purpose

This is used to query the continuous output mode.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

mode

Variable Type       ViChar[]

This is used to query the continuous output mode.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryContinuousOutputState

ViStatus Chr62000L_QueryContinuousOutputState

(ViSession instrumentHandle, ViPBoolean enable);

Purpose

This is used to query the continuous output function to be on or off.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enable

Variable Type       ViBoolean (passed by reference)

This is used to query the continuous output function to be on or off.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryContinuousOutputStep

ViStatus Chr62000L_QueryContinuousOutputStep (ViSession instrumentHandle,

ViChar _VI_FAR startStep[],

ViChar _VI_FAR stopStep[]);

Purpose

Query the start and stop step.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

startStep

Variable Type       ViChar[]

Query the start step.

stopStep

Variable Type       ViChar[]

Query the stop step.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryCVCCLimit

ViStatus Chr62000L_QueryCVCCLimit (ViSession instrumentHandle,

ViPReal64 userDefineVoltageValue,

ViPReal64 userDefineCurrentValue);

Purpose

Query CV/CC Limit value.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

userDefineVoltageValue

Variable Type       ViReal64 (passed by reference)

Query CV Limit value.

userDefineCurrentValue

Variable Type       ViReal64 (passed by reference)

Query CC Limit value.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryCVLimit

ViStatus Chr62000L_QueryCVLimit (ViSession instrumentHandle,

ViInt32 voltage,

ViPReal64 voltageValue,

ViInt32 voltageStep,

ViPReal64 userDefineVoltageStepValue);

Purpose

This is used to query the power of the CV limit.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

voltage

Variable Type       ViInt32

Query the voltage value.

voltageValue

Variable Type       ViReal64 (passed by reference)

Query the voltage value.

voltageStep

Variable Type       ViInt32

Query the user define or default voltage amplitude.

userDefineVoltageStepValue

Variable Type       ViReal64 (passed by reference)

Query the voltage amplitude.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryDisplayState

ViStatus Chr62000L_QueryDisplayState (ViSession instrumentHandle,

ViPBoolean enable);

Purpose

Query the screen to show whether it is turned on or off. A value of 0

means the screen is OFF and a value of 1 means the screen is ON.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enable

Variable Type       ViBoolean (passed by reference)

Query the screen to show whether it is turned on or off. A value of 0

means the screen is OFF and a value of 1 means the screen is ON.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryDisplayText

ViStatus Chr62000L_QueryDisplayText (ViSession instrumentHandle,

ViChar _VI_FAR text[]);

Purpose

Query the set display string.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

text

Variable Type       ViChar[]

Query the set display string.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryDWELLTime

ViStatus Chr62000L_QueryDWELLTime (ViSession instrumentHandle,

ViPInt32 timeValue, ViInt32 step);

Purpose

Query the step of the DWELL time, step settings for the S0 ~ S7, the time

for the return of ms.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

timeValue

Variable Type       ViInt32 (passed by reference)

Query the step of the DWELL time, the time for the return of ms.

step

Variable Type       ViInt32

Query the step of the DWELL time, step settings for the S0 ~ S7.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryESE

ViStatus Chr62000L_QueryESE (ViSession instrumentHandle,

ViPInt32 enableValue);

Purpose

Query the standard event start register ,and the meter return value is

expressed in decimal.

*ESE?

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enableValue

Variable Type       ViInt32 (passed by reference)

Query the standard event start register ,and the meter return value

is expressed in decimal.

*ESE?

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryESR

ViStatus Chr62000L_QueryESR (ViSession instrumentHandle,

ViPInt32 enableValue);

Purpose

Query the standard event register ,and the meter return value is

expressed in decimal.

*ESR?

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enableValue

Variable Type       ViInt32 (passed by reference)

Query the standard event register ,and the meter return value is

expressed in decimal.

*ESR?

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryMasterSlaveEnabled

ViStatus Chr62000L_QueryMasterSlaveEnabled (ViSession instrumentHandle,

ViPBoolean enable);

Purpose

This is used to query the master-slave control to be on or off.

if your instrument SN Number less than 0200000122,the instrument not

support ,Please contact Chroma Technical Services.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enable

Variable Type       ViBoolean (passed by reference)

This is used to query the master-slave control to be on or off.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryMasterSlaveOnOffDelayTime

ViStatus Chr62000L_QueryMasterSlaveOnOffDelayTime

(ViSession instrumentHandle, ViPInt32 onDelayTime,

ViPInt32 offDelayTime);

Purpose

This is used to query the master-slave control on and off delay time.

if your instrument SN Number less than 0200000122,the instrument not

support ,Please contact Chroma Technical Services.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

onDelayTime

Variable Type       ViInt32 (passed by reference)

This is used to query the master-slave control on delay time.(mS)

offDelayTime

Variable Type       ViInt32 (passed by reference)

This is used to query the master-slave control off delay time.(mS)

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryNormalBeeperState

ViStatus Chr62000L_QueryNormalBeeperState (ViSession instrumentHandle,

ViPBoolean enable);

Purpose

This is used to query the normal beeper on or off.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enable

Variable Type       ViBoolean (passed by reference)

This is used to query the normal beeper on or off.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryOCP

ViStatus Chr62000L_QueryOCP (ViSession instrumentHandle, ViInt32 limit,

ViPReal64 limitValue);

Purpose

The is used to inquire about the current set overcurrent protection

standards.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

limit

Variable Type       ViInt32

This is used to query the OCP.

limitValue

Variable Type       ViReal64 (passed by reference)

This is used to query the OCP value.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryOCPBeeperState

ViStatus Chr62000L_QueryOCPBeeperState (ViSession instrumentHandle,

ViPBoolean enable);

Purpose

This is used to query the OCP alarm beeper on or off.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enable

Variable Type       ViBoolean (passed by reference)

This is used to query the OCP alarm beeper on or off.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryOCPOpen

ViStatus Chr62000L_QueryOCPOpen (ViSession instrumentHandle,

ViPBoolean OCPOpenState);

Purpose

Query the OCP open is happend or not.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

OCPOpenState

Variable Type       ViBoolean (passed by reference)

This parameter returns VI_TRUE if the OCP open is happend, and

VI_FALSE if it is not.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryOCPState

ViStatus Chr62000L_QueryOCPState (ViSession instrumentHandle,

ViPBoolean OCPState);

Purpose

Query the OCP is on or off.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

OCPState

Variable Type       ViBoolean (passed by reference)

This parameter returns VI_TRUE if the OCP is on, and VI_FALSE if it

is not.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryOPC

ViStatus Chr62000L_QueryOPC (ViSession instrumentHandle,

ViPInt32 value);

Purpose

After executing this instruction, the return value is 1.

*OPC?

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

value

Variable Type       ViInt32 (passed by reference)

After executing this instruction, the return value is 1.

*OPC?

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryOperationMode

ViStatus Chr62000L_QueryOperationMode (ViSession instrumentHandle,

ViChar _VI_FAR mode[]);

Purpose

This is used to query the master-slave control mode of operation.

if your instrument SN Number less than 0200000122,the instrument not

support ,Please contact Chroma Technical Services.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

mode

Variable Type       ViChar[]

This is used to query the master-slave control mode of operation.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryOperationModeStatus

ViStatus Chr62000L_QueryOperationModeStatus (ViSession instrumentHandle,

ViPInt32 mode);

Purpose

Query the power supply operation mode. Return value 0 means output is off

or irregular output. Return value 1 is in CC mode. Return value 2 is in

CV mode and return value 3 means CP mode.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

mode

Variable Type       ViInt32 (passed by reference)

Query the power supply operation mode. Return value 0 means output is

off or irregular output. Return value 1 is in CC mode. Return value 2

is in CV mode and return value 3 means CP mode.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryOutputStates

ViStatus Chr62000L_QueryOutputStates (ViSession instrumentHandle,

ViPBoolean outputState);

Purpose

This function returns whether the power supply is on or off.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

outputState

Variable Type       ViBoolean (passed by reference)

This parameter returns VI_TRUE if the power supply is output on, and

VI_FALSE if it is not.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryOVP

ViStatus Chr62000L_QueryOVP (ViSession instrumentHandle, ViInt32 limit,

ViPReal64 limitValue);

Purpose

The is used to inquire about the current set overvoltage protection

standards.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

limit

Variable Type       ViInt32

This is used to query the OVP.

limitValue

Variable Type       ViReal64 (passed by reference)

This is used to query the OVP value.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryOVPBeeperState

ViStatus Chr62000L_QueryOVPBeeperState (ViSession instrumentHandle,

ViPBoolean enable);

Purpose

This is used to query the OVP alarm beeper on or off.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enable

Variable Type       ViBoolean (passed by reference)

This is used to query the OVP alarm beeper on or off.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryOVPOpen

ViStatus Chr62000L_QueryOVPOpen (ViSession instrumentHandle,

ViPBoolean OVPOpenState);

Purpose

Query the OVP open is happend or not.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

OVPOpenState

Variable Type       ViBoolean (passed by reference)

This parameter returns VI_TRUE if the OVP open is happend, and

VI_FALSE if it is not.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryOVPState

ViStatus Chr62000L_QueryOVPState (ViSession instrumentHandle,

ViPBoolean OVPState);

Purpose

Query the OVP is on or off.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

OVPState

Variable Type       ViBoolean (passed by reference)

This parameter returns VI_TRUE if the OVP is on, and VI_FALSE if it

is not.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryPSC

ViStatus Chr62000L_QueryPSC (ViSession instrumentHandle,

ViPInt32 value);

Purpose

Ask the power-on status to clear the settings. The return value 1 is the

start of the PSC, and the return value 0 is the PSC off.

*PSC?

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

value

Variable Type       ViInt32 (passed by reference)

Ask the power-on status to clear the settings. The return value 1 is

the start of the PSC, and the return value 0 is the PSC off.

*PSC?

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryQuestionableEnabledRegister

ViStatus Chr62000L_QueryQuestionableEnabledRegister

(ViSession instrumentHandle, ViPInt32 enableValue);

Purpose

Query the questionable status enable register, The return value is a

decimal representation of the bits in the enable register.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enableValue

Variable Type       ViInt32 (passed by reference)

Query the questionable status enable register, The return value is a

decimal representation of the bits in the enable register.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryQuestionableEventRegisterStatus

ViStatus Chr62000L_QueryQuestionableEventRegisterStatus

(ViSession instrumentHandle, ViPInt32 mode);

Purpose

Query the status of the questionable event register, the return value in

decimal, said all the bits of the binary register bit value. These bits

are latched, and the event registers are read and cleared.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

mode

Variable Type       ViInt32 (passed by reference)

Query the status of the questionable event register, the return value

in decimal, said all the bits of the binary register bit value. These

bits are latched, and the event registers are read and cleared.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryRiseFallTime

ViStatus Chr62000L_QueryRiseFallTime (ViSession instrumentHandle,

ViPInt32 timeValue, ViInt32 step);

Purpose

Query the step of the rise and fall time, step settings for the S0 ~ S7,

the time for the return of ms.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

timeValue

Variable Type       ViInt32 (passed by reference)

Query the step of the rise and fall time, the time for the return of

ms.

step

Variable Type       ViInt32

Query the step of the rise and fall time, step settings for the S0 ~

S7.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QuerySCPIVersion

ViStatus Chr62000L_QuerySCPIVersion (ViSession instrumentHandle,

ViChar _VI_FAR version[]);

Purpose

Query the SCPI instruction version used by the power supply. The return

format is YYYY.V, Y is the year of the version, and V is the version of

the year (for example, 1996.0).

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

version

Variable Type       ViChar[]

Query the SCPI instruction version used by the power supply. The

return format is YYYY.V, Y is the year of the version, and V is the

version of the year (for example, 1996.0).

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QuerySRE

ViStatus Chr62000L_QuerySRE (ViSession instrumentHandle,

ViPInt32 enableValue);

Purpose

*SRE?

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enableValue

Variable Type       ViInt32 (passed by reference)

*SRE?

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QuerySTB

ViStatus Chr62000L_QuerySTB (ViSession instrumentHandle,

ViPInt32 value);

Purpose

*STB?

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

value

Variable Type       ViInt32 (passed by reference)

*STB?

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryStepCurrent

ViStatus Chr62000L_QueryStepCurrent (ViSession instrumentHandle,

ViPReal64 currentValue,

ViInt32 step);

Purpose

Query step set current, step for the S0 ~ S7, the return value of the

current value of amps.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

currentValue

Variable Type       ViReal64 (passed by reference)

Query step set current, step for the S0 ~ S7, the return value of the

current value of amps.

step

Variable Type       ViInt32

Query step set current, step for the S0 ~ S7.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryStepSetting

ViStatus Chr62000L_QueryStepSetting (ViSession instrumentHandle,

ViInt32 step,

ViPReal64 voltageValue,

ViPReal64 currentValue,

ViPInt32 DWELLTimeValue,

ViPInt32 riseFallTimeValue);

Purpose

Query the step setting.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

step

Variable Type       ViInt32

Query the step setting, step settings for the S0 ~ S7.

voltageValue

Variable Type       ViReal64 (passed by reference)

Query step set voltage, step for the S0 ~ S7, the return value of the

voltage value of Volts.

currentValue

Variable Type       ViReal64 (passed by reference)

Query step set current, step for the S0 ~ S7, the return value of the

current value of amps.

DWELLTimeValue

Variable Type       ViInt32 (passed by reference)

Query the step of the DWELL time, the time for the return of ms.

riseFallTimeValue

Variable Type       ViInt32 (passed by reference)

Query the step of the rise and fall time, the time for the return of

ms.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryStepVoltage

ViStatus Chr62000L_QueryStepVoltage (ViSession instrumentHandle,

ViPReal64 voltageValue,

ViInt32 step);

Purpose

Query step set voltage, step for the S0 ~ S7, the return value of the

voltage value of Volts.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

voltageValue

Variable Type       ViReal64 (passed by reference)

Query step set voltage, step for the S0 ~ S7, the return value of the

voltage value of Volts.

step

Variable Type       ViInt32

Query step set voltage, step for the S0 ~ S7.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryTriggeredCurrent

ViStatus Chr62000L_QueryTriggeredCurrent (ViSession instrumentHandle,

ViPReal64 triggeredCurrent,

ViInt32 current);

Purpose

The is used to query the trigger current.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

triggeredCurrent

Variable Type       ViReal64 (passed by reference)

The is used to query the trigger current.

current

Variable Type       ViInt32

This is used to query the trigger current.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryTriggeredDelay

ViStatus Chr62000L_QueryTriggeredDelay (ViSession instrumentHandle,

ViPInt32 triggeredDelayValue);

Purpose

This function is used to query the trigger delay time.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

triggeredDelayValue

Variable Type       ViInt32 (passed by reference)

This is used to query the trigger delay.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryTriggeredVoltage

ViStatus Chr62000L_QueryTriggeredVoltage (ViSession instrumentHandle,

ViPReal64 triggeredVoltage,

ViInt32 voltage);

Purpose

The is used to query the trigger voltage.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

triggeredVoltage

Variable Type       ViReal64 (passed by reference)

The is used to query the trigger current.

voltage

Variable Type       ViInt32

This is used to query the trigger current.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_QueryTriggerSource

ViStatus Chr62000L_QueryTriggerSource (ViSession instrumentHandle,

ViChar _VI_FAR source[]);

Purpose

Query the trigger source.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

source

Variable Type       ViChar[]

Query the trigger source.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ReadInstrData

ViStatus Chr62000L_ReadInstrData (ViSession instrumentHandle,

ViInt32 numberOfBytesToRead,

ViChar _VI_FAR readBuffer[],

ViPInt32 numBytesRead);

Purpose

This function reads data from the instrument.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

numberOfBytesToRead

Variable Type       ViInt32

Pass the maximum number of bytes to read from the instruments.

Valid Range:  0 to the number of elements in the Read Buffer.

Default:  0

readBuffer

Variable Type       ViChar[]

After this function executes, this parameter contains the data that

was read from the instrument.

numBytesRead

Variable Type       ViInt32 (passed by reference)

Returns the number of bytes actually read from the instrument and

stored in the Read Buffer.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ReadOperationState

ViStatus Chr62000L_ReadOperationState (ViSession instrumentHandle,

ViInt32 read);

Purpose

Reads the stored operation status.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

read

Variable Type       ViInt32

Reads the stored operation status.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_reset

ViStatus Chr62000L_reset (ViSession instrumentHandle);

Purpose

This function resets the instrument to a known state and sends

initialization commands to the instrument.  The initialization commands

set instrument settings such as Headers Off, Short Command form, and Data

Transfer Binary to the state necessary for the operation of the

instrument driver.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ResetInterchangeCheck

ViStatus Chr62000L_ResetInterchangeCheck (ViSession instrumentHandle);

Purpose

When developing a complex test system that consists of multiple test

modules, it is generally a good idea to design the test modules so that

they can run in any order.  To do so requires ensuring that each test

module completely configures the state of each instrument it uses.  If a

particular test module does not completely configure the state of an

instrument, the state of the instrument depends on the configuration from

a previously executed test module.  If you execute the test modules in a

different order, the behavior of the instrument and therefore the entire

test module is likely to change.  This change in behavior is generally

instrument specific and represents an interchangeability problem.

You can use this function to test for such cases.  After you call this

function, the interchangeability checking algorithms in the specific

driver ignore all previous configuration operations.  By calling this

function at the beginning of a test module, you can determine whether the

test module has dependencies on the operation of previously executed test

modules.

This function does not clear the interchangeability warnings from the

list of previously recorded interchangeability warnings.  If you want to

guarantee that the Chr62000L_GetNextInterchangeWarning function only

returns those interchangeability warnings that are generated after

calling this function, you must clear the list of interchangeability

warnings.  You can clear the interchangeability warnings list by

repeatedly calling the Chr62000L_GetNextInterchangeWarning function until

no more interchangeability warnings are returned.  If you are not

interested in the content of those warnings, you can call the

Chr62000L_ClearInterchangeWarnings function.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_ResetWithDefaults

ViStatus Chr62000L_ResetWithDefaults (ViSession instrumentHandle);

Purpose

This function resets the instrument and applies initial user specified

settings from the Logical Name which was used to initialize the session.

If the session was created without a Logical Name, this function is

equivalent to the Chr62000L_reset function.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_revision_query

ViStatus Chr62000L_revision_query (ViSession instrumentHandle,

ViChar _VI_FAR instrumentDriverRevision[],

ViChar _VI_FAR firmwareRevision[]);

Purpose

This function returns the revision numbers of the instrument driver and

instrument firmware.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

instrumentDriverRevision

Variable Type       ViChar[]

Returns the instrument driver software revision numbers in the form

of a string.

You must pass a ViChar array with at least 256 bytes.

firmwareRevision

Variable Type       ViChar[]

Returns the instrument firmware revision numbers in the form of a

string.

You must pass a ViChar array with at least 256 bytes.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_SaveOperationState

ViStatus Chr62000L_SaveOperationState (ViSession instrumentHandle,

ViInt32 save);

Purpose

Stores the current operating state into non-temporary memory. It can be

stored in 0 ~ 15 storage points, and the operating state stored in

storage point 0 is set to power-on state.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

save

Variable Type       ViInt32

Stores the current operating state into non-temporary memory. It can

be stored in 0 ~ 15 storage points, and the operating state stored in

storage point 0 is set to power-on state.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_self_test

ViStatus Chr62000L_self_test (ViSession instrumentHandle,

ViPInt16 selfTestResult,

ViChar _VI_FAR selfTestMessage[]);

Purpose

This function runs the instrument's self test routine and returns the

test result(s).

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

selfTestResult

Variable Type       ViInt16 (passed by reference)

This control contains the value returned from the instrument self

test.  Zero means success.  For any other code, see the device's

operator's manual.

Self-Test Code    Description

---------------------------------------

0              Passed self test

1              Self test failed

selfTestMessage

Variable Type       ViChar[]

Returns the self-test response string from the instrument. See the

device's operation manual for an explanation of the string's

contents.

You must pass a ViChar array with at least 256 bytes.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_SetAttributeViBoolean

ViStatus Chr62000L_SetAttributeViBoolean (ViSession instrumentHandle,

ViConstString channelName,

ViAttr attributeID,

ViBoolean attributeValue);

Purpose

This function sets the value of a ViBoolean attribute.

This is a low-level function that you can use to set the values of

instrument-specific attributes and inherent IVI attributes.  If the

attribute represents an instrument state, this function performs

instrument I/O in the following cases:

- State caching is disabled for the entire session or for the particular

attribute.

- State caching is enabled and the currently cached value is invalid or

is different than the value you specify.

This instrument driver contains high-level functions that set most of the

instrument attributes.  It is best to use the high-level driver functions

as much as possible.  They handle order dependencies and multithread

locking for you.  In addition, they perform status checking only after

setting all of the attributes.  In contrast, when you set multiple

attributes using the SetAttribute functions, the functions check the

instrument status after each call.

Also, when state caching is enabled, the high-level functions that

configure multiple attributes perform instrument I/O only for the

attributes whose value you change.  Thus, you can safely call the

high-level functions without the penalty of redundant instrument I/O.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViConstString

If the attribute is channel-based, this parameter specifies the name

of the channel on which to set the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViBoolean

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViBoolean are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViBoolean

Pass the value to which you want to set the attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_SetAttributeViInt32

ViStatus Chr62000L_SetAttributeViInt32 (ViSession instrumentHandle,

ViConstString channelName,

ViAttr attributeID,

ViInt32 attributeValue);

Purpose

This function sets the value of a ViInt32 attribute.

This is a low-level function that you can use to set the values of

instrument-specific attributes and inherent IVI attributes.  If the

attribute represents an instrument state, this function performs

instrument I/O in the following cases:

- State caching is disabled for the entire session or for the particular

attribute.

- State caching is enabled and the currently cached value is invalid or

is different than the value you specify.

This instrument driver contains high-level functions that set most of the

instrument attributes.  It is best to use the high-level driver functions

as much as possible.  They handle order dependencies and multithread

locking for you.  In addition, they perform status checking only after

setting all of the attributes.  In contrast, when you set multiple

attributes using the SetAttribute functions, the functions check the

instrument status after each call.

Also, when state caching is enabled, the high-level functions that

configure multiple attributes perform instrument I/O only for the

attributes whose value you change.  Thus, you can safely call the

high-level functions without the penalty of redundant instrument I/O.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViConstString

If the attribute is channel-based, this parameter specifies the name

of the channel on which to set the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViInt32 type.

If you choose to see all IVI attributes, the data types appear

to the right of the attribute names in the list box.

Attributes with data types other than ViInt32 are dim.  If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViInt32

Pass the value to which you want to set the attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_SetAttributeViReal64

ViStatus Chr62000L_SetAttributeViReal64 (ViSession instrumentHandle,

ViConstString channelName,

ViAttr attributeID,

ViReal64 attributeValue);

Purpose

This function sets the value of a ViReal64 attribute.

This is a low-level function that you can use to set the values of

instrument-specific attributes and inherent IVI attributes.  If the

attribute represents an instrument state, this function performs

instrument I/O in the following cases:

- State caching is disabled for the entire session or for the particular

attribute.

- State caching is enabled and the currently cached value is invalid or

is different than the value you specify.

This instrument driver contains high-level functions that set most of the

instrument attributes.  It is best to use the high-level driver functions

as much as possible.  They handle order dependencies and multithread

locking for you.  In addition, they perform status checking only after

setting all of the attributes.  In contrast, when you set multiple

attributes using the SetAttribute functions, the functions check the

instrument status after each call.

Also, when state caching is enabled, the high-level functions that

configure multiple attributes perform instrument I/O only for the

attributes whose value you change.  Thus, you can safely call the

high-level functions without the penalty of redundant instrument I/O.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViConstString

If the attribute is channel-based, this parameter specifies the name

of the channel on which to set the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViReal64

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViReal64 are dim.  If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViReal64

Pass the value to which you want to set the attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_SetAttributeViSession

ViStatus Chr62000L_SetAttributeViSession (ViSession instrumentHandle,

ViConstString channelName,

ViAttr attributeID,

ViSession attributeValue);

Purpose

This function sets the value of a ViSession attribute.

This is a low-level function that you can use to set the values of

instrument-specific attributes and inherent IVI attributes.  If the

attribute represents an instrument state, this function performs

instrument I/O in the following cases:

- State caching is disabled for the entire session or for the particular

attribute.

- State caching is enabled and the currently cached value is invalid or

is different than the value you specify.

This instrument driver contains high-level functions that set most of the

instrument attributes.  It is best to use the high-level driver functions

as much as possible.  They handle order dependencies and multithread

locking for you.  In addition, they perform status checking only after

setting all of the attributes.  In contrast, when you set multiple

attributes using the SetAttribute functions, the functions check the

instrument status after each call.

Also, when state caching is enabled, the high-level functions that

configure multiple attributes perform instrument I/O only for the

attributes whose value you change.  Thus, you can safely call the

high-level functions without the penalty of redundant instrument I/O.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViConstString

If the attribute is channel-based, this parameter specifies the name

of the channel on which to set the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViSession

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViSession are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViSession

Pass the value to which you want to set the attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_SetAttributeViString

ViStatus Chr62000L_SetAttributeViString (ViSession instrumentHandle,

ViConstString channelName,

ViAttr attributeID,

ViChar _VI_FAR attributeValue[]);

Purpose

This function sets the value of a ViString attribute.

This is a low-level function that you can use to set the values of

instrument-specific attributes and inherent IVI attributes.  If the

attribute represents an instrument state, this function performs

instrument I/O in the following cases:

- State caching is disabled for the entire session or for the particular

attribute.

- State caching is enabled and the currently cached value is invalid or

is different than the value you specify.

This instrument driver contains high-level functions that set most of the

instrument attributes.  It is best to use the high-level driver functions

as much as possible.  They handle order dependencies and multithread

locking for you.  In addition, they perform status checking only after

setting all of the attributes.  In contrast, when you set multiple

attributes using the SetAttribute functions, the functions check the

instrument status after each call.

Also, when state caching is enabled, the high-level functions that

configure multiple attributes perform instrument I/O only for the

attributes whose value you change.  Thus, you can safely call the

high-level functions without the penalty of redundant instrument I/O.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViConstString

If the attribute is channel-based, this parameter specifies the name

of the channel on which to set the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViString

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViString are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViChar[]

Pass the value to which you want to set the attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_SetCLS

ViStatus Chr62000L_SetCLS (ViSession instrumentHandle);

Purpose

Clears all event registers and status bit registers.

*CLS

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_SetESE

ViStatus Chr62000L_SetESE (ViSession instrumentHandle,

ViInt32 enableValue);

Purpose

The bits in the standard event start buffer are turned on and the opened

bit is sent to the status byte.

*ESE

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enableValue

Variable Type       ViInt32

The bits in the standard event start buffer are turned on and the

opened bit is sent to the status byte.

*ESE

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_SetOPC

ViStatus Chr62000L_SetOPC (ViSession instrumentHandle);

Purpose

When this instruction is executed, the "OPC" bit 0 of the standard event

register is set to 1.

*OPC

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_SetPSC

ViStatus Chr62000L_SetPSC (ViSession instrumentHandle,

ViBoolean enable);

Purpose

Turns the power-on status clear function on / off. When the default PSC

is on, the status byte and the standard event boot register are cleared

at power-on. The instruction * PSC 0 will not be cleared.

*PSC

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enable

Variable Type       ViBoolean

Turns the power-on status clear function on / off. When the default

PSC is on, the status byte and the standard event boot register are

cleared at power-on. The instruction * PSC 0 will not be cleared.

*PSC

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_SetQuestionableEnabledRegister

ViStatus Chr62000L_SetQuestionableEnabledRegister

(ViSession instrumentHandle, ViInt32 enableValue);

Purpose

Set the questionable status enable register, the corresponding bit will

be turned on and transferred to the status byte.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enableValue

Variable Type       ViInt32

Set the questionable status enable register, the corresponding bit

will be turned on and transferred to the status byte.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_SetSRE

ViStatus Chr62000L_SetSRE (ViSession instrumentHandle,

ViInt32 enableValue);

Purpose

*SRE

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enableValue

Variable Type       ViInt32

*SRE

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_SetWAI

ViStatus Chr62000L_SetWAI (ViSession instrumentHandle);

Purpose

Instructs the power supply to wait until all pending operations have been

completed before executing the next instruction, only in the trigger

mode.

*WAI

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_UnlockSession

ViStatus Chr62000L_UnlockSession (ViSession instrumentHandle,

ViPBoolean callerHasLock);

Purpose

This function releases a lock that you acquired on an instrument session

using Chr62000L_LockSession.  Refer to Chr62000L_LockSession for

additional information on session locks.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

callerHasLock

Variable Type       ViBoolean (passed by reference)

This parameter serves as a convenience.  If you do not want to use

this parameter, pass VI_NULL.

Use this parameter in complex functions to keep track of whether you

obtain a lock and therefore need to unlock the session.

Pass the address of a local ViBoolean variable.  In the declaration

of the local variable, initialize it to VI_FALSE.  Pass the address

of the same local variable to any other calls you make to

Chr62000L_LockSession or Chr62000L_UnlockSession in the same

function.

The parameter is an input/output parameter.  Chr62000L_LockSession

and Chr62000L_UnlockSession each inspect the current value and take

the following actions:

- If the value is VI_TRUE, Chr62000L_LockSession does not lock the

session again.  If the value is VI_FALSE, Chr62000L_LockSession

obtains the lock and sets the value of the parameter to VI_TRUE.

- If the value is VI_FALSE, Chr62000L_UnlockSession does not attempt

to unlock the session.  If the value is VI_TRUE,

Chr62000L_UnlockSession releases the lock and sets the value of the

parameter to VI_FALSE.

Thus, you can, call Chr62000L_UnlockSession at the end of your

function without worrying about whether you actually have the lock.

Example:

ViStatus TestFunc (ViSession vi, ViInt32 flags)

{

ViStatus error = VI_SUCCESS;

ViBoolean haveLock = VI_FALSE;

if (flags & BIT_1)

{

viCheckErr( Chr62000L_LockSession(vi, &haveLock));

viCheckErr( TakeAction1(vi));

if (flags & BIT_2)

{

viCheckErr( Chr62000L_UnlockSession(vi, &haveLock));

viCheckErr( TakeAction2(vi));

viCheckErr( Chr62000L_LockSession(vi, &haveLock);

}

if (flags & BIT_3)

viCheckErr( TakeAction3(vi));

}

Error:

/*

At this point, you cannot really be sure that

you have the lock.  Fortunately, the haveLock

variable takes care of that for you.

*/

Chr62000L_UnlockSession(vi, &haveLock);

return error;

}

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA2000 to 3FFA3FFF     IviDCPwr Warnings

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA2000 to BFFA3FFF     IviDCPwr Errors

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_viRead

ViStatus Chr62000L_viRead (ViSession instrumentHandle,

ViInt64 numberBytesToRead,

ViByte _VI_FAR readBuffer[],

ViPInt64 numberBytesRead);

Purpose

This function reads data directly from an instrument using VISA I/O. The

function bypasses the attribute state caching mechanism.

The function assumes that the IVI_ATTR_SYSTEM_IO_SESSION attribute for

the IVI session you specify holds a valid VISA session for the

instrument.

If data is not available at the instrument's output buffer when you call

this function, the instrument might hang up. In that case, the function

does not return until the VISA I/O call times out. If you have disabled

the timeout, the function hangs indefinitely.

Note: This function is supported with the IVI Compliance Package (ICP)

4.6.1 and later. If you are using an earlier version of ICP, you can

still use the WriteInstrData and ReadInstrData functions under the

Obsolete folder.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

numberBytesToRead

Variable Type       ViInt64

Specify the maximum number of bytes to read from the instrument. The

Read Buffer parameter must be a ViChar buffer that contains at least

the number of bytes you specify.

If the number of bytes you specify is less than the number of bytes

in the instrument's output buffer, you must call this function again

to empty the output buffer. If you do not empty the instrument's

output buffer, the instrument might return invalid data in response

to subsequent requests.

If data is not available at the instrument's output buffer when you

call this function, the instrument might hang up. In that case, the

function does not return until the VISA I/O call times out. If you

have disabled the timeout, the function hangs indefinitely.

readBuffer

Variable Type       ViByte[]

A buffer in which the function places the data it receives from the

instrument.

The buffer must be a ViChar array that has at least as many bytes as

you specify in the Number Bytes To Read parameter.

This function does not write an ASCII NUL byte to terminate the data,

nor does it clear the buffer beyond the bytes it actually receives

from the instrument.

numberBytesRead

Variable Type       ViInt64 (passed by reference)

This control returns the actual number of bytes the function received

from the instrument. This is the value that the VISA viRead function

returns.

If the actual number of bytes received is less than the number of

bytes you specify in the Number Bytes To Read parameter, the

instrument's output buffer has probably emptied.

If the number of bytes received is 0, the most probable cause is that

no data was available at the instrument's output buffer.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_viWrite

ViStatus Chr62000L_viWrite (ViSession instrumentHandle,

ViByte _VI_FAR writeBuffer[],

ViInt64 numberBytesToWrite,

ViPInt64 numberBytesWrite);

Purpose

This function writes a command string directly to an instrument using

VISA I/O. The function bypasses the attribute state caching mechanism and

therefore always invalidates all attribute cache values for the session.

The function assumes that the IVI_ATTR_SYSTEM_IO_SESSION attribute for

the IVI session you specify holds a valid VISA session for the

instrument.

The function calls Ivi_SetNeedToCheckStatus with VI_TRUE.

Note: This function is supported with the IVI Compliance Package (ICP)

4.6.1 and later. If you are using an earlier version of ICP, you can

still use the WriteInstrData and ReadInstrData functions under the

Obsolete folder.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

writeBuffer

Variable Type       ViByte[]

Specify the string you want to send to the instrument.

numberBytesToWrite

Variable Type       ViInt64

Specify the maximum number of bytes to read from the instrument. The

Read Buffer parameter must be a ViChar buffer that contains at least

the number of bytes you specify.

If the number of bytes you specify is less than the number of bytes

in the instrument's output buffer, you must call this function again

to empty the output buffer. If you do not empty the instrument's

output buffer, the instrument might return invalid data in response

to subsequent requests.

If data is not available at the instrument's output buffer when you

call this function, the instrument might hang up. In that case, the

function does not return until the VISA I/O call times out. If you

have disabled the timeout, the function hangs indefinitely.

numberBytesWrite

Variable Type       ViInt64 (passed by reference)

This control returns the actual number of bytes the function received

from the instrument. This is the value that the VISA viRead function

returns.

If the actual number of bytes received is less than the number of

bytes you specify in the Number Bytes To Read parameter, the

instrument's output buffer has probably emptied.

If the number of bytes received is 0, the most probable cause is that

no data was available at the instrument's output buffer.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr62000L_WriteInstrData

ViStatus Chr62000L_WriteInstrData (ViSession instrumentHandle,

ViChar _VI_FAR writeBuffer[]);

Purpose

This function writes a user-specified string to the instrument.

Note:  This function bypasses IVI attribute state caching.  Therefore,

when you call this function, the cached values for all attributes will be

invalidated.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr62000L_init or

Chr62000L_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

writeBuffer

Variable Type       ViChar[]

Pass the string to be written to the instrument.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr62000L_error_message function.  To obtain additional information

about the error condition, call the Chr62000L_GetError function.  To

clear the error information from the driver, call the

Chr62000L_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

Status    Description

-------------------------------------------------

WARNINGS:

/*=CHANGE:===================================================*

Insert Instrument-defined warning codes here.

*================================================END=CHANGE=*/

ERRORS:

/*=CHANGE:===================================================*

Insert Instrument-defined error codes here.

*================================================END=CHANGE=*/

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Attribute Information for the Following Functions:

Chr62000L_SetAttributeViInt32

Chr62000L_GetAttributeViInt32

Chr62000L_CheckAttributeViInt32

Chr62000L_SetAttributeViInt64

Chr62000L_GetAttributeViInt64

Chr62000L_CheckAttributeViInt64

Chr62000L_SetAttributeViReal64

Chr62000L_GetAttributeViReal64

Chr62000L_CheckAttributeViReal64

Chr62000L_SetAttributeViSession

Chr62000L_GetAttributeViSession

Chr62000L_CheckAttributeViSession

Chr62000L_SetAttributeViBoolean

Chr62000L_GetAttributeViBoolean

Chr62000L_CheckAttributeViBoolean

Chr62000L_SetAttributeViString

Chr62000L_GetAttributeViString

Chr62000L_CheckAttributeViString

Inherent IVI Attributes

User Options

Range Check                              CHR62000L_ATTR_RANGE_CHECK

Query Instrument Status                  CHR62000L_ATTR_QUERY_INSTRUMENT_STATUS

Cache                                    CHR62000L_ATTR_CACHE

Simulate                                 CHR62000L_ATTR_SIMULATE

Record Value Coercions                   CHR62000L_ATTR_RECORD_COERCIONS

Interchange Check                        CHR62000L_ATTR_INTERCHANGE_CHECK

Driver Identification

Description                              CHR62000L_ATTR_SPECIFIC_DRIVER_DESCRIPTION

Driver Prefix                            CHR62000L_ATTR_SPECIFIC_DRIVER_PREFIX

Driver Vendor                            CHR62000L_ATTR_SPECIFIC_DRIVER_VENDOR

Revision                                 CHR62000L_ATTR_SPECIFIC_DRIVER_REVISION

Class Specification Major Version        CHR62000L_ATTR_SPECIFIC_DRIVER_CLASS_SPEC_MAJOR_VERSION

Class Specification Minor Version        CHR62000L_ATTR_SPECIFIC_DRIVER_CLASS_SPEC_MINOR_VERSION

Driver Capabilities

Supported Instrument Models              CHR62000L_ATTR_SUPPORTED_INSTRUMENT_MODELS

Class Group Capabilities                 CHR62000L_ATTR_GROUP_CAPABILITIES

Instrument Identification

Manufacturer                             CHR62000L_ATTR_INSTRUMENT_MANUFACTURER

Model                                    CHR62000L_ATTR_INSTRUMENT_MODEL

Firmware Revision                        CHR62000L_ATTR_INSTRUMENT_FIRMWARE_REVISION

Advanced Session Information

Logical Name                             CHR62000L_ATTR_LOGICAL_NAME

I/O Resource Descriptor                  CHR62000L_ATTR_IO_RESOURCE_DESCRIPTOR

Driver Setup                             CHR62000L_ATTR_DRIVER_SETUP

Basic Operation

Chr62000L_ATTR_CURRENT_STRING               CHR62000L_ATTR_CURRENT_STRING

Chr62000L_ATTR_VOLTAGE_STRING               CHR62000L_ATTR_VOLTAGE_STRING

Chr62000L_ATTR_CV_CC_LIMIT                  CHR62000L_ATTR_CV_CC_LIMIT

Output Enabled                              CHR62000L_ATTR_OUTPUT_ENABLED

Chr62000L_ATTR_CURRENT_STEP_STRING          CHR62000L_ATTR_CURRENT_STEP_STRING

Chr62000L_ATTR_CURRENT_STEP                 CHR62000L_ATTR_CURRENT_STEP

Chr62000L_ATTR_CURRENT_LIMIT                CHR62000L_ATTR_CURRENT_LIMIT

Chr62000L_ATTR_OCP_LIMIT_STRING             CHR62000L_ATTR_OCP_LIMIT_STRING

Chr62000L_ATTR_OCP_LIMIT                    CHR62000L_ATTR_OCP_LIMIT

Chr62000L_ATTR_OCP_ENABLED                  CHR62000L_ATTR_OCP_ENABLED

Chr62000L_ATTR_OCP_OPEN                     CHR62000L_ATTR_OCP_OPEN

Chr62000L_ATTR_OCP_OPEN_CLEAR               CHR62000L_ATTR_OCP_OPEN_CLEAR

Chr62000L_ATTR_VOLTAGE_STEP_STRING          CHR62000L_ATTR_VOLTAGE_STEP_STRING

Chr62000L_ATTR_VOLTAGE_STEP                 CHR62000L_ATTR_VOLTAGE_STEP

Chr62000L_ATTR_VOLTAGE_LEVEL                CHR62000L_ATTR_VOLTAGE_LEVEL

Chr62000L_ATTR_OVP_LIMIT_STRING             CHR62000L_ATTR_OVP_LIMIT_STRING

OVP Limit                                   CHR62000L_ATTR_OVP_LIMIT

OVP Enabled                                 CHR62000L_ATTR_OVP_ENABLED

Chr62000L_ATTR_OVP_OPEN                     CHR62000L_ATTR_OVP_OPEN

Chr62000L_ATTR_OVP_OPEN_CLEAR               CHR62000L_ATTR_OVP_OPEN_CLEAR

Current Limit Behavior                      CHR62000L_ATTR_CURRENT_LIMIT_BEHAVIOR

Trigger Subsystem

Chr62000L_ATTR_TRIGGERED_CURRENT_STRING     CHR62000L_ATTR_TRIGGERED_CURRENT_STRING

Chr62000L_ATTR_TRIGGERED_CURRENT_LIMIT      CHR62000L_ATTR_TRIGGERED_CURRENT_LIMIT

Chr62000L_ATTR_TRIGGERED_VOLTAGE_STRING     CHR62000L_ATTR_TRIGGERED_VOLTAGE_STRING

Chr62000L_ATTR_TRIGGERED_VOLTAGE_LEVEL      CHR62000L_ATTR_TRIGGERED_VOLTAGE_LEVEL

Trigger Source                              CHR62000L_ATTR_TRIGGER_SOURCE

Chr62000L_ATTR_TRIGGER_STRING               CHR62000L_ATTR_TRIGGER_STRING

Chr62000L_ATTR_TRIGGER_DELAY                CHR62000L_ATTR_TRIGGER_DELAY

Master Slave

Chr62000L_ATTR_ON_DELAY_STRING              CHR62000L_ATTR_ON_DELAY_STRING

Chr62000L_ATTR_OFF_DELAY_STRING             CHR62000L_ATTR_OFF_DELAY_STRING

Chr62000L_ATTR_ON_DELAY                     CHR62000L_ATTR_ON_DELAY

Chr62000L_ATTR_OFF_DELAY                    CHR62000L_ATTR_OFF_DELAY

Chr62000L_ATTR_MODE                         CHR62000L_ATTR_MODE

Chr62000L_ATTR_MASTER_SLAVE_ENABLED         CHR62000L_ATTR_MASTER_SLAVE_ENABLED

Continuous Output

Chr62000L_ATTR_CONTINUOUS_OUTPUT_FINISH     CHR62000L_ATTR_CONTINUOUS_OUTPUT_FINISH

Chr62000L_ATTR_CONTINUOUS_OUTPUT_ENABLED    CHR62000L_ATTR_CONTINUOUS_OUTPUT_ENABLED

Chr62000L_ATTR_CONTINUOUS_OUTPUT_MODE       CHR62000L_ATTR_CONTINUOUS_OUTPUT_MODE

Chr62000L_ATTR_CONTINUOUS_OUTPUT_CYCLE      CHR62000L_ATTR_CONTINUOUS_OUTPUT_CYCLE

Chr62000L_ATTR_CONTINUOUS_OUTPUT_STEP_START  CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_START

Chr62000L_ATTR_CONTINUOUS_OUTPUT_STEP_STOP  CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_STOP

Chr62000L_ATTR_CONTINUOUS_OUTPUT_STEP_VOLTAGE_STEP  CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_VOLTAGE_STEP

Chr62000L_ATTR_CONTINUOUS_OUTPUT_STEP_VOLTAGE_STRING  CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_VOLTAGE_STRING

Chr62000L_ATTR_CONTINUOUS_OUTPUT_STEP_VOLTAGE  CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_VOLTAGE

Chr62000L_ATTR_CONTINUOUS_OUTPUT_STEP_CURRENT_STEP  CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_CURRENT_STEP

Chr62000L_ATTR_CONTINUOUS_OUTPUT_STEP_CURRENT_STRING  CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_CURRENT_STRING

Chr62000L_ATTR_CONTINUOUS_OUTPUT_STEP_CURRENT  CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_CURRENT

Chr62000L_ATTR_CONTINUOUS_OUTPUT_STEP_RISE_FALL_STEP  CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_RISE_FALL_STEP

Chr62000L_ATTR_CONTINUOUS_OUTPUT_STEP_RISE_FALL_STRING  CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_RISE_FALL_STRING

Chr62000L_ATTR_CONTINUOUS_OUTPUT_STEP_RISE_FALL  CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_RISE_FALL

Chr62000L_ATTR_CONTINUOUS_OUTPUT_STEP_DWELL_STRING  CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_DWELL_STRING

Chr62000L_ATTR_CONTINUOUS_OUTPUT_STEP_DWELL_STEP  CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_DWELL_STEP

Chr62000L_ATTR_CONTINUOUS_OUTPUT_STEP_DWELL  CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_DWELL

Chr62000L_ATTR_CONTINUOUS_OUTPUT_STEP_SETTING_STEP  CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_SETTING_STEP

Chr62000L_ATTR_CONTINUOUS_OUTPUT_STEP_SETTING  CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_SETTING

System Function

Chr62000L_ATTR_FRONT_PANEL_DISPLAY          CHR62000L_ATTR_FRONT_PANEL_DISPLAY

Chr62000L_ATTR_DISPLAY_TEXT                 CHR62000L_ATTR_DISPLAY_TEXT

Chr62000L_ATTR_SCPI_VERSION                 CHR62000L_ATTR_SCPI_VERSION

Chr62000L_ATTR_OVP_BEEPER                   CHR62000L_ATTR_OVP_BEEPER

Chr62000L_ATTR_OCP_BEEPER                   CHR62000L_ATTR_OCP_BEEPER

Chr62000L_ATTR_NORMAL_BEEPER                CHR62000L_ATTR_NORMAL_BEEPER

Calibration

Chr62000L_ATTR_CALIBRATION_NUMBER           CHR62000L_ATTR_CALIBRATION_NUMBER

Chr62000L_ATTR_CALIBRATION_CC               CHR62000L_ATTR_CALIBRATION_CC

Chr62000L_ATTR_CALIBRATION_CC_LEVEL         CHR62000L_ATTR_CALIBRATION_CC_LEVEL

Chr62000L_ATTR_CALIBRATION_PASSWORD         CHR62000L_ATTR_CALIBRATION_PASSWORD

Chr62000L_ATTR_CALIBRATION_PROT_ENABLE      CHR62000L_ATTR_CALIBRATION_PROT_ENABLE

Chr62000L_ATTR_CALIBRATION_PROT_PASSWORD    CHR62000L_ATTR_CALIBRATION_PROT_PASSWORD

Chr62000L_ATTR_CALIBRATION_MESSAGE          CHR62000L_ATTR_CALIBRATION_MESSAGE

Chr62000L_ATTR_CALIBRATION_CV               CHR62000L_ATTR_CALIBRATION_CV

Chr62000L_ATTR_CALIBRATION_CV_LEVEL         CHR62000L_ATTR_CALIBRATION_CV_LEVEL

Status

Chr62000L_ATTR_OPERATION_MODE_STATUS        CHR62000L_ATTR_OPERATION_MODE_STATUS

Chr62000L_ATTR_QUESTIONABLE_OPERATION_REGISTER  CHR62000L_ATTR_QUESTIONABLE_OPERATION_REGISTER

Chr62000L_ATTR_QUESTIONABLE_ENABLE_REGISTER  CHR62000L_ATTR_QUESTIONABLE_ENABLE_REGISTER

Miscellaneous Attributes

ID Query Response                           CHR62000L_ATTR_ID_QUERY_RESPONSE

CHR62000L_ATTR_CACHE

Data Type:     ViBoolean

Description:   Specifies whether to cache the value of attributes.  When

caching is enabled, the instrument driver keeps track of the current instrument

settings and avoids sending redundant commands to the instrument.  Thus, you

can significantly increase execution speed.

The instrument driver can choose always to cache or never to cache

particular attributes regardless of the setting of this attribute.

The default value is VI_TRUE.  Use the Chr62000L_InitWithOptions function

to override this value.

CHR62000L_ATTR_CALIBRATION_CC

Data Type:     ViReal64

Description:

CHR62000L_ATTR_CALIBRATION_CC_LEVEL

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_MIN                              3

CHR62000L_VAL_MAX                              4

CHR62000L_ATTR_CALIBRATION_CV

Data Type:     ViReal64

Description:

CHR62000L_ATTR_CALIBRATION_CV_LEVEL

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_MIN                              3

CHR62000L_VAL_MAX                              4

CHR62000L_ATTR_CALIBRATION_MESSAGE

Data Type:     ViString

Description:

CHR62000L_ATTR_CALIBRATION_NUMBER

Data Type:     ViInt32

Description:

CHR62000L_ATTR_CALIBRATION_PASSWORD

Data Type:     ViString

Description:

CHR62000L_ATTR_CALIBRATION_PROT_ENABLE

Data Type:     ViBoolean

Description:

CHR62000L_ATTR_CALIBRATION_PROT_PASSWORD

Data Type:     ViString

Description:

CHR62000L_ATTR_CONTINUOUS_OUTPUT_CYCLE

Data Type:     ViInt32

Description:

CHR62000L_ATTR_CONTINUOUS_OUTPUT_ENABLED

Data Type:     ViBoolean

Description:

CHR62000L_ATTR_CONTINUOUS_OUTPUT_FINISH

Data Type:     ViBoolean

Description:   This is used to query the continuous output to be finish.

CHR62000L_ATTR_CONTINUOUS_OUTPUT_MODE

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_VOLTAGE                          0

CHR62000L_VAL_CURRENT                          1

CHR62000L_VAL_BOTH                             2

CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_CURRENT

Data Type:     ViReal64

Description:

CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_CURRENT_STEP

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_S0                               0

CHR62000L_VAL_S1                               1

CHR62000L_VAL_S2                               2

CHR62000L_VAL_S3                               3

CHR62000L_VAL_S4                               4

CHR62000L_VAL_S5                               5

CHR62000L_VAL_S6                               6

CHR62000L_VAL_S7                               7

CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_CURRENT_STRING

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_UserDefine                       1

CHR62000L_VAL_DEF                              2

CHR62000L_VAL_MIN                              3

CHR62000L_VAL_MAX                              4

CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_DWELL

Data Type:     ViInt32

Description:

CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_DWELL_STEP

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_S0                               0

CHR62000L_VAL_S1                               1

CHR62000L_VAL_S2                               2

CHR62000L_VAL_S3                               3

CHR62000L_VAL_S4                               4

CHR62000L_VAL_S5                               5

CHR62000L_VAL_S6                               6

CHR62000L_VAL_S7                               7

CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_DWELL_STRING

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_UserDefine                       1

CHR62000L_VAL_MAX                              4

CHR62000L_VAL_MIN                              3

CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_RISE_FALL

Data Type:     ViInt32

Description:

CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_RISE_FALL_STEP

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_S0                               0

CHR62000L_VAL_S1                               1

CHR62000L_VAL_S2                               2

CHR62000L_VAL_S3                               3

CHR62000L_VAL_S4                               4

CHR62000L_VAL_S5                               5

CHR62000L_VAL_S6                               6

CHR62000L_VAL_S7                               7

CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_RISE_FALL_STRING

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_UserDefine                       1

CHR62000L_VAL_MAX                              4

CHR62000L_VAL_MIN                              3

CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_SETTING

Data Type:     ViString

Description:

CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_SETTING_STEP

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_S0                               0

CHR62000L_VAL_S1                               1

CHR62000L_VAL_S2                               2

CHR62000L_VAL_S3                               3

CHR62000L_VAL_S4                               4

CHR62000L_VAL_S5                               5

CHR62000L_VAL_S6                               6

CHR62000L_VAL_S7                               7

CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_START

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_S0                               0

CHR62000L_VAL_S1                               1

CHR62000L_VAL_S2                               2

CHR62000L_VAL_S3                               3

CHR62000L_VAL_S4                               4

CHR62000L_VAL_S5                               5

CHR62000L_VAL_S6                               6

CHR62000L_VAL_S7                               7

CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_STOP

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_S0                               0

CHR62000L_VAL_S1                               1

CHR62000L_VAL_S2                               2

CHR62000L_VAL_S3                               3

CHR62000L_VAL_S4                               4

CHR62000L_VAL_S5                               5

CHR62000L_VAL_S6                               6

CHR62000L_VAL_S7                               7

CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_VOLTAGE

Data Type:     ViReal64

Description:

CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_VOLTAGE_STEP

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_S0                               0

CHR62000L_VAL_S1                               1

CHR62000L_VAL_S2                               2

CHR62000L_VAL_S3                               3

CHR62000L_VAL_S4                               4

CHR62000L_VAL_S5                               5

CHR62000L_VAL_S6                               6

CHR62000L_VAL_S7                               7

CHR62000L_ATTR_CONTINUOUS_OUTPUT_STEP_VOLTAGE_STRING

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_UserDefine                       1

CHR62000L_VAL_DEF                              2

CHR62000L_VAL_MIN                              3

CHR62000L_VAL_MAX                              4

CHR62000L_ATTR_CURRENT_LIMIT

Data Type:     ViReal64

Description:   This channel-based attribute specifies the output current limit.

The units are Amps.

The value of the CHR62000L_ATTR_CURRENT_LIMIT_BEHAVIOR attribute determines the

behavior of the power supply when the output current is equal to or greater

than the value of this attribute.

/*=CHANGE:==========*/

Modify the following to represent the valid range for your instrument.

Valid Range:

/*=END=CHANGE=======*/

CHR62000L_ATTR_CURRENT_LIMIT_BEHAVIOR

Data Type:     ViInt32

Description:   This channel-based attribute specifies the behavior of the power

supply when the output current is equal to or greater than the value of the

CHR62000L_ATTR_CURRENT_LIMIT attribute.

Values:

CHR62000L_VAL_CURRENT_REGULATE                 0

When the output current reaches the value of the

CHR62000L_ATTR_CURRENT_LIMIT attribute, the power supply restricts the

output voltage such that the output current is not greater than the

value of the CHR62000L_ATTR_CURRENT_LIMIT attribute.

CHR62000L_VAL_CURRENT_TRIP                     1

When the output current reaches or exceeds the value of the

CHR62000L_ATTR_CURRENT_LIMIT attribute, the power supply disables the

output.

CHR62000L_ATTR_CURRENT_STEP

Data Type:     ViReal64

Description:

CHR62000L_ATTR_CURRENT_STEP_STRING

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_UserDefine                       1

CHR62000L_VAL_DEF                              2

CHR62000L_VAL_MIN                              3

CHR62000L_VAL_MAX                              4

CHR62000L_VAL_UP                               5

CHR62000L_VAL_DOWN                             6

CHR62000L_ATTR_CURRENT_STRING

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_UserDefine                       1

CHR62000L_VAL_DEF                              2

CHR62000L_VAL_MIN                              3

CHR62000L_VAL_MAX                              4

CHR62000L_VAL_UP                               5

CHR62000L_VAL_DOWN                             6

CHR62000L_ATTR_CV_CC_LIMIT

Data Type:     ViString

Description:

CHR62000L_ATTR_DISPLAY_TEXT

Data Type:     ViString

Description:

CHR62000L_ATTR_DRIVER_SETUP

Data Type:     ViString

Restrictions:  Not settable.

Description:   Some cases exist where you must specify instrument driver

options at initialization time.  An example of this is specifying a particular

instrument model from among a family of instruments that the driver supports.

This is useful when using simulation.  You can specify driver-specific options

through the DriverSetup keyword in the optionsString parameter to the

Chr62000L_InitWithOptions function.  If you open an instrument using a logical

name, you can also specify the options through the IVI Configuration Utility.

The default value is an empty string.

CHR62000L_ATTR_FRONT_PANEL_DISPLAY

Data Type:     ViBoolean

Description:

CHR62000L_ATTR_GROUP_CAPABILITIES

Data Type:     ViString

Restrictions:  Not settable.

Description:   A string that contains a comma-separated list of class-extension

groups that this driver implements.

CHR62000L_ATTR_ID_QUERY_RESPONSE

Data Type:     ViString

Restrictions:  Not settable.

Description:   Returns the ID Query response string.  The instrument driver

gets the value of this attribute when you pass VI_TRUE for the ID Query

parameter to the Chr62000L_init or Chr62000L_InitWithOptions function.

CHR62000L_ATTR_INSTRUMENT_FIRMWARE_REVISION

Data Type:     ViString

Restrictions:  Not settable.

Description:   A string that contains the firmware revision information for the

instrument you are currently using.

CHR62000L_ATTR_INSTRUMENT_MANUFACTURER

Data Type:     ViString

Restrictions:  Not settable.

Description:   A string that contains the name of the instrument manufacturer

you are currently using.

CHR62000L_ATTR_INSTRUMENT_MODEL

Data Type:     ViString

Restrictions:  Not settable.

Description:   A string that contains the model number or name of the

instrument that you are currently using.

CHR62000L_ATTR_INTERCHANGE_CHECK

Data Type:     ViBoolean

Description:   Specifies whether to perform interchangeability checking and

retrieve interchangeability warnings.

The default value is VI_FALSE.

Interchangeability warnings indicate that using your application with a

different instrument might cause different behavior.  You call Chr62000L Get

Next Interchange Warning to extract interchange warnings.  Call the

Chr62000L_ClearInterchangeWarnings function to clear the list of

interchangeability warnings without reading them.

Interchangeability checking logs a warning for each attribute you have not set

that affects the behavior of the instrument.

CHR62000L_ATTR_IO_RESOURCE_DESCRIPTOR

Data Type:     ViString

Restrictions:  Not settable.

Description:   Indicates the resource descriptor the driver uses to identify

the physical device.

If you initialize the driver with a logical name, this attribute contains

the resource descriptor that corresponds to the entry in the IVI Configuration

utility.

If you initialize the instrument driver with the resource descriptor, this

attribute contains that value.

CHR62000L_ATTR_LOGICAL_NAME

Data Type:     ViString

Restrictions:  Not settable.

Description:   A string containing the logical name you specified when opening

the current IVI session.

You may pass a logical name to the Chr62000L_init or

Chr62000L_InitWithOptions functions.  The IVI Configuration utility must

contain an entry for the logical name.  The logical name entry refers to a

virtual instrument section in the IVI Configuration file.  The virtual

instrument section specifies a physical device and initial user options.

CHR62000L_ATTR_MASTER_SLAVE_ENABLED

Data Type:     ViBoolean

Description:

CHR62000L_ATTR_MODE

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_P_MASTER                         0

CHR62000L_VAL_P_SLAVE                          1

CHR62000L_VAL_S_MASTER                         2

CHR62000L_VAL_S_SLAVE                          3

CHR62000L_VAL_NORMAL                           4

CHR62000L_VAL_TRACK                            5

CHR62000L_ATTR_NORMAL_BEEPER

Data Type:     ViBoolean

Description:

CHR62000L_ATTR_OCP_BEEPER

Data Type:     ViBoolean

Description:

CHR62000L_ATTR_OCP_ENABLED

Data Type:     ViBoolean

Description:

CHR62000L_ATTR_OCP_LIMIT

Data Type:     ViReal64

Description:

CHR62000L_ATTR_OCP_LIMIT_STRING

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_UserDefine                       1

CHR62000L_VAL_DEF                              2

CHR62000L_VAL_MIN                              3

CHR62000L_VAL_MAX                              4

CHR62000L_VAL_UP                               5

CHR62000L_VAL_DOWN                             6

CHR62000L_ATTR_OCP_OPEN

Data Type:     ViBoolean

Description:

CHR62000L_ATTR_OCP_OPEN_CLEAR

Data Type:     ViInt32

Description:

CHR62000L_ATTR_OFF_DELAY

Data Type:     ViInt32

Description:

CHR62000L_ATTR_OFF_DELAY_STRING

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_UserDefine                       1

CHR62000L_VAL_MAX                              4

CHR62000L_VAL_MIN                              3

CHR62000L_ATTR_ON_DELAY

Data Type:     ViInt32

Description:

CHR62000L_ATTR_ON_DELAY_STRING

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_UserDefine                       1

CHR62000L_VAL_MAX                              4

CHR62000L_VAL_MIN                              3

CHR62000L_ATTR_OPERATION_MODE_STATUS

Data Type:     ViInt32

Description:

CHR62000L_ATTR_OUTPUT_ENABLED

Data Type:     ViBoolean

Description:   This channel-based attribute specifies whether the signal the

power supply produces appears at the output connector.

CHR62000L_ATTR_OVP_BEEPER

Data Type:     ViBoolean

Description:

CHR62000L_ATTR_OVP_ENABLED

Data Type:     ViBoolean

Description:   This channel-based attribute specifies whether the power supply

provides over-voltage protection.  If this attribute is set to VI_TRUE, the

power supply disables the output when the output voltage is greater than or

equal to the value of the CHR62000L_ATTR_OVP_LIMIT attribute.

CHR62000L_ATTR_OVP_LIMIT

Data Type:     ViReal64

Description:   This channel-based attribute specifies the voltage the power

supply allows. The units are Volts.

If the CHR62000L_ATTR_OVP_ENABLED attribute is set to VI_TRUE, the power supply

disables the output when the output voltage is greater than or equal to the

value of this attribute.

If the CHR62000L_ATTR_OVP_ENABLED attribute is set to VI_FALSE, this attribute

does not affect the behavior of the instrument.

/*=CHANGE:==========*/

Modify the following to represent the valid range for your instrument.

Valid Range:

/*=END=CHANGE=======*/

CHR62000L_ATTR_OVP_LIMIT_STRING

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_UserDefine                       1

CHR62000L_VAL_DEF                              2

CHR62000L_VAL_MIN                              3

CHR62000L_VAL_MAX                              4

CHR62000L_VAL_UP                               5

CHR62000L_VAL_DOWN                             6

CHR62000L_ATTR_OVP_OPEN

Data Type:     ViBoolean

Description:

CHR62000L_ATTR_OVP_OPEN_CLEAR

Data Type:     ViInt32

Description:

CHR62000L_ATTR_QUERY_INSTRUMENT_STATUS

Data Type:     ViBoolean

Description:   Specifies whether the instrument driver queries the instrument

status after each operation.  Querying the instrument status is very useful for

debugging.  After you validate your program, you can set this attribute to

VI_FALSE to disable status checking and maximize performance

The instrument driver can choose to ignore status checking for particular

attributes regardless of the setting of this attribute.

The default value is VI_FALSE.  Use the Chr62000L_InitWithOptions function

to override this value.

CHR62000L_ATTR_QUESTIONABLE_ENABLE_REGISTER

Data Type:     ViInt32

Description:

CHR62000L_ATTR_QUESTIONABLE_OPERATION_REGISTER

Data Type:     ViInt32

Description:

CHR62000L_ATTR_RANGE_CHECK

Data Type:     ViBoolean

Description:   Specifies whether to validate attribute values and function

parameters.  If enabled, the instrument driver validates the parameter values

that you pass to driver functions.  Range checking parameters is very useful

for debugging.  After you validate your program, you can set this attribute to

VI_FALSE to disable range checking and maximize performance.

The default value is VI_TRUE.  Use the Chr62000L_InitWithOptions function

to override this value.

CHR62000L_ATTR_RECORD_COERCIONS

Data Type:     ViBoolean

Description:   Specifies whether the IVI engine keeps a list of the value

coercions it makes for integer and real type attributes.  You call Chr62000L

Get Next Coercion Record to extract and delete the oldest coercion record from

the list.

The default value is VI_FALSE.  Use the Chr62000L_InitWithOptions function

to override this value.

CHR62000L_ATTR_SCPI_VERSION

Data Type:     ViString

Description:

CHR62000L_ATTR_SIMULATE

Data Type:     ViBoolean

Description:   Specifies whether or not to simulate instrument driver I/O

operations.  If simulation is enabled, instrument driver functions perform

range checking and call Ivi_GetAttribute and Ivi_SetAttribute functions, but

they do not perform instrument I/O.  For output parameters that represent

instrument data, the instrument driver functions return calculated values.

The default value is VI_FALSE.  Use the Chr62000L_InitWithOptions function

to override this value.

CHR62000L_ATTR_SPECIFIC_DRIVER_CLASS_SPEC_MAJOR_VERSION

Data Type:     ViInt32

Restrictions:  Not settable.

Description:   The major version number of the class specification with which

this driver is compliant.

CHR62000L_ATTR_SPECIFIC_DRIVER_CLASS_SPEC_MINOR_VERSION

Data Type:     ViInt32

Restrictions:  Not settable.

Description:   The minor version number of the class specification with which

this driver is compliant.

CHR62000L_ATTR_SPECIFIC_DRIVER_DESCRIPTION

Data Type:     ViString

Restrictions:  Not settable.

Description:   A string that contains a brief description of the specific

driver.

CHR62000L_ATTR_SPECIFIC_DRIVER_PREFIX

Data Type:     ViString

Restrictions:  Not settable.

Description:   A string that contains the prefix for the instrument driver.

The name of each user-callable function in this driver starts with this prefix.

CHR62000L_ATTR_SPECIFIC_DRIVER_REVISION

Data Type:     ViString

Restrictions:  Not settable.

Description:   A string that contains additional version information about this

instrument driver.

CHR62000L_ATTR_SPECIFIC_DRIVER_VENDOR

Data Type:     ViString

Restrictions:  Not settable.

Description:   A string that contains the name of the vendor that supplies this

driver.

CHR62000L_ATTR_SUPPORTED_INSTRUMENT_MODELS

Data Type:     ViString

Restrictions:  Not settable.

Description:   Contains a model code of the instrument. For drivers that

support more than one device, this attribute contains a comma-separated list of

supported instrument models.

CHR62000L_ATTR_TRIGGER_DELAY

Data Type:     ViInt32

Description:

CHR62000L_ATTR_TRIGGER_SOURCE

Data Type:     ViInt32

Description:   This channel-based attribute specifies the trigger source.

After you call Chr62000L_Initiate, the power supply waits for a trigger event

from the source you specify with this attribute.  After a trigger event occurs,

the power supply changes the voltage level to the value of the

CHR62000L_ATTR_TRIGGERED_VOLTAGE_LEVEL and the current limit to the value of

the CHR62000L_ATTR_TRIGGERED_CURRENT_LIMIT attributes.

Values:

CHR62000L_VAL_TRIG_IMMEDIATE                   0

The power supply does not wait for a trigger before changing the

output signa.

CHR62000L_VAL_TRIG_BUS                         21

The power supply waits for a trigger on bus.

CHR62000L_ATTR_TRIGGER_STRING

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_UserDefine                       1

CHR62000L_VAL_MAX                              4

CHR62000L_VAL_MIN                              3

CHR62000L_ATTR_TRIGGERED_CURRENT_LIMIT

Data Type:     ViReal64

Description:   This channel-based attribute specifies the value to  which the

power supply sets the current limit after a trigger event occurs. The units are

amps.

After you call Chr62000L_Initiate, the power supply waits for a trigger event

from the source you specify with the CHR62000L_ATTR_TRIGGER_SOURCE attribute.

After a trigger event occurs, the power supply sets the current limit to the

value of this attribute.  After a trigger occurs, the value of the

CHR62000L_ATTR_CURRENT_LIMIT attribute reflects the new value to which the

current limit has been set.

/*=CHANGE:==========*/

Modify the following to represent the valid range for your instrument.

Valid Range:

/*=END=CHANGE=======*/

CHR62000L_ATTR_TRIGGERED_CURRENT_STRING

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_UserDefine                       1

CHR62000L_VAL_DEF                              2

CHR62000L_VAL_MIN                              3

CHR62000L_VAL_MAX                              4

CHR62000L_VAL_UP                               5

CHR62000L_VAL_DOWN                             6

CHR62000L_ATTR_TRIGGERED_VOLTAGE_LEVEL

Data Type:     ViReal64

Description:   This channel-based attribute specifies the value to  which the

power supply sets the voltage level after a trigger event occurs. The units are

Volts.

After you call Chr62000L_Initiate, the power supply waits for a trigger event

from the source you specify with the CHR62000L_ATTR_TRIGGER_SOURCE attribute.

After a trigger event occurs, the power supply sets the voltage level to the

value of this attribute.  After a trigger occurs, the value of the

CHR62000L_ATTR_VOLTAGE_LEVEL attribute reflects the new value to which the

voltage level has been set.

/*=CHANGE:==========*/

Modify the following to represent the valid range for your instrument.

Valid Range:

/*=END=CHANGE=======*/

CHR62000L_ATTR_TRIGGERED_VOLTAGE_STRING

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_UserDefine                       1

CHR62000L_VAL_DEF                              2

CHR62000L_VAL_MIN                              3

CHR62000L_VAL_MAX                              4

CHR62000L_VAL_UP                               5

CHR62000L_VAL_DOWN                             6

CHR62000L_ATTR_VOLTAGE_LEVEL

Data Type:     ViReal64

Description:   This channel-based attribute specifies the voltage level the DC

power supply attempts to generate.  The units are Volts.

/*=CHANGE:==========*/

Modify the following to represent the valid range for your instrument.

Valid Range:

/*=END=CHANGE=======*/

CHR62000L_ATTR_VOLTAGE_STEP

Data Type:     ViReal64

Description:

CHR62000L_ATTR_VOLTAGE_STEP_STRING

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_UserDefine                       1

CHR62000L_VAL_DEF                              2

CHR62000L_VAL_MIN                              3

CHR62000L_VAL_MAX                              4

CHR62000L_VAL_UP                               5

CHR62000L_VAL_DOWN                             6

CHR62000L_ATTR_VOLTAGE_STRING

Data Type:     ViInt32

Description:

Values:

CHR62000L_VAL_UserDefine                       1

CHR62000L_VAL_DEF                              2

CHR62000L_VAL_MIN                              3

CHR62000L_VAL_MAX                              4

CHR62000L_VAL_UP                               5

CHR62000L_VAL_DOWN                             6