Chr63000

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Chr63000.

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instrument drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

For VXI instrument drivers:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

MXI

MXI2

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Each function in this instrument driver returns a status code that

either indicates success or describes an error or warning condition.

Your program should examine the status code from each call to an

instrument driver function to determine if an error occurred.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

The description of each instrument driver function lists possible error

codes and their meanings

________________________________________________________________________________

How To Use This Document:

Use this document as a programming reference manual.

It describes each function in the

Chr63000

instrument.  The functions appear in alphabetical order, with a description

of the function and its C syntax, a description of each parameter, and a list

of possible error codes.

________________________________________________________________________________

Function Tree Layout:

Class/Panel Name:                                 Function Name:

Initialize                                     Chr63000_init

Initialize With Options                        Chr63000_InitWithOptions

Action/Status Functions

Abort                                       Chr63000_Abort

Fetch                                       Chr63000_Fetch

Operating Mode                              Chr63000_OperatingMode

Electronic Load State                       Chr63000_ElectronicLoadState

Measure                                     Chr63000_Measure

Start Specification Test                    Chr63000_StartSpecificationTest

Specification Judgment                      Chr63000_SpecificationJudgment

Digitizing

Digitizing Initiate                      Chr63000_DigitizingInitiate

Digitizing Abort                         Chr63000_DigitizingAbort

Digitizing Sampling Time                 Chr63000_DigitizingSamplingTime

Digitizing Sampling Point                Chr63000_DigitizingSamplingPoint

Digitizing Trigger                       Chr63000_DigitizingTrigger

Digitizing Trigger Point                 Chr63000_DigitizingTriggerPoint

Digitizing Trigger Source                Chr63000_DigitizingTriggerSource

Digitizing Waveform                      Chr63000_DigitizingWaveform

Configuration Functions

Constant Current Mode

Configure CC Mode Measure Range          Chr63000_ConfigureCCModeMeasureRange

Configure CC Mode Load Value             Chr63000_ConfigureCCModeLoadValue

Configure CC Mode Slew Rate              Chr63000_ConfigureCCModeSlewRate

Constant Resistance Mode

Configure CR Mode Measure Range          Chr63000_ConfigureCRModeMeasureRange

Configure CR Mode Load Value             Chr63000_ConfigureCRModeLoadValue

Configure CR Mode Slew Rate              Chr63000_ConfigureCRModeSlewRate

Constant Power Mode

Configure CP Mode Measure Range          Chr63000_ConfigureCPModeMeasureRange

Configure CP Mode Load Value             Chr63000_ConfigureCPModeLoadValue

Configure CP Mode Slew Rate              Chr63000_ConfigureCPModeSlewRate

Constant Voltage Mode

Configure CV Mode Load Value             Chr63000_ConfigureCVModeLoadValue

Configure CV Mode Current Limit          Chr63000_ConfigureCVModeCurrentLimit

Configure CV Mode Response Speed         Chr63000_ConfigureCVModeResponseSpeed

Dynamic Current  Mode

Configure CCD Mode Measure Range         Chr63000_ConfigureCCDModeMeasureRange

Configure CCD Mode Repeat                Chr63000_ConfigureCCDModeRepeat

Configure CCD Mode Slew Rate             Chr63000_ConfigureCCDModeSlewRate

Configure CCD Mode Load Value            Chr63000_ConfigureCCDModeLoadValue

Configure CCD Mode Time                  Chr63000_ConfigureCCDModeTime

Advance Mode

Program Mode

Configure Program Type                Chr63000_ConfigureProgramType

Configure Program Data

Select Program                     Chr63000_SelectProgram

Configure Program Mode             Chr63000_ConfigureProgramMode

Load Value

Configure Program List Value    Chr63000_ConfigureProgramListValue

Configure Program Step Value    Chr63000_ConfigureProgramStepValue

Configure Program Time             Chr63000_ConfigureProgramTime

Configure Program Specification    Chr63000_ConfigureProgramSpecification

Save Program                          Chr63000_SaveProgram

Get Program States                    Chr63000_GetProgramStates

Sequence

Clear Sequence                     Chr63000_ClearSequence

Fail Of Sequence                   Chr63000_FailOfSequence

Remain Sequence                    Chr63000_RemainSequence

Specification

Configure Voltage Specification          Chr63000_ConfigureVoltageSpecification

Configure Current Specification          Chr63000_ConfigureCurrentSpecification

Configure Power Specification            Chr63000_ConfigurePowerSpecification

Set/Get/Check Attribute

Set Attribute

Set Attribute ViInt32                 Chr63000_SetAttributeViInt32

Set Attribute ViReal64                Chr63000_SetAttributeViReal64

Set Attribute ViString                Chr63000_SetAttributeViString

Set Attribute ViBoolean               Chr63000_SetAttributeViBoolean

Set Attribute ViSession               Chr63000_SetAttributeViSession

Get Attribute

Get Attribute ViInt32                 Chr63000_GetAttributeViInt32

Get Attribute ViReal64                Chr63000_GetAttributeViReal64

Get Attribute ViString                Chr63000_GetAttributeViString

Get Attribute ViBoolean               Chr63000_GetAttributeViBoolean

Get Attribute ViSession               Chr63000_GetAttributeViSession

Check Attribute

Check Attribute ViInt32               Chr63000_CheckAttributeViInt32

Check Attribute ViReal64              Chr63000_CheckAttributeViReal64

Check Attribute ViString              Chr63000_CheckAttributeViString

Check Attribute ViBoolean             Chr63000_CheckAttributeViBoolean

Check Attribute ViSession             Chr63000_CheckAttributeViSession

Utility Functions

Miscellaneous

System Setup                             Chr63000_SystemSetup

Load Setup                               Chr63000_LoadSetup

Measurement Setting                      Chr63000_MeasurementSetting

DIO                                      Chr63000_DIO

OCP and OPP                              Chr63000_OCPandOPP

Timing                                   Chr63000_Timing

Reset                                       Chr63000_reset

Reset With Defaults                         Chr63000_ResetWithDefaults

Revision Query                              Chr63000_revision_query

Error-Query                                 Chr63000_error_query

Error Message                               Chr63000_error_message

Invalidate All Attributes                   Chr63000_InvalidateAllAttributes

Error Info

Get Error                                Chr63000_GetError

Clear Error                              Chr63000_ClearError

Coercion Info

Get Next Coercion Record                 Chr63000_GetNextCoercionRecord

Interchangeability Info

Get Next Interchange Warning             Chr63000_GetNextInterchangeWarning

Clear Interchange Warnings               Chr63000_ClearInterchangeWarnings

Reset Interchange Check                  Chr63000_ResetInterchangeCheck

Locking

Lock Session                             Chr63000_LockSession

Unlock Session                           Chr63000_UnlockSession

Instrument I/O

Write Instrument Data                    Chr63000_WriteInstrData

Read Instrument Data                     Chr63000_ReadInstrData

Close                                          Chr63000_close

________________________________________________________________________________

Chr63000

This instrument driver contains programming support for the Chroma 63200E

series.  This driver has all the functions that IVI and VXIplug&play

require.

Note:  This driver requires the VISA and IVI libraries.

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

Chr63000_Abort

ViStatus Chr63000_Abort (ViSession instrumentHandle);

Purpose

Set all electronic loads as "OFF".

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_CheckAttributeViBoolean

ViStatus Chr63000_CheckAttributeViBoolean (ViSession instrumentHandle,

ViChar _VI_FAR channelName[],

ViAttr attributeID,

ViBoolean attributeValue);

Purpose

This function checks the validity of a value you specify for a ViBoolean

attribute.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViChar[]

If the attribute is channel-based, this parameter specifies the name

of the channel on which to check the attribute value. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViBoolean

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViBoolean are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViBoolean

Pass the value which you want to verify as a valid value for the

attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_CheckAttributeViInt32

ViStatus Chr63000_CheckAttributeViInt32 (ViSession instrumentHandle,

ViChar _VI_FAR channelName[],

ViAttr attributeID,

ViInt32 attributeValue);

Purpose

This function checks the validity of a value you specify for a ViInt32

attribute.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViChar[]

If the attribute is channel-based, this parameter specifies the name

of the channel on which to check the attribute value. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViInt32 type.

If you choose to see all IVI attributes, the data types appear

to the right of the attribute names in the list box.

Attributes with data types other than ViInt32 are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViInt32

Pass the value which you want to verify as a valid value for the

attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_CheckAttributeViReal64

ViStatus Chr63000_CheckAttributeViReal64 (ViSession instrumentHandle,

ViChar _VI_FAR channelName[],

ViAttr attributeID,

ViReal64 attributeValue);

Purpose

This function checks the validity of a value you specify for a ViReal64

attribute.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViChar[]

If the attribute is channel-based, this parameter specifies the name

of the channel on which to check the attribute value. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViReal64

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViReal64 are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViReal64

Pass the value which you want to verify as a valid value for the

attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_CheckAttributeViSession

ViStatus Chr63000_CheckAttributeViSession (ViSession instrumentHandle,

ViChar _VI_FAR channelName[],

ViAttr attributeID,

ViSession attributeValue);

Purpose

This function checks the validity of a value you specify for a ViSession

attribute.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViChar[]

If the attribute is channel-based, this parameter specifies the name

of the channel on which to check the attribute value. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViSession

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViSession are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViSession

Pass the value which you want to verify as a valid value for the

attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_CheckAttributeViString

ViStatus Chr63000_CheckAttributeViString (ViSession instrumentHandle,

ViChar _VI_FAR channelName[],

ViAttr attributeID,

ViChar _VI_FAR attributeValue[]);

Purpose

This function checks the validity of a value you specify for a ViString

attribute.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViChar[]

If the attribute is channel-based, this parameter specifies the name

of the channel on which to check the attribute value. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViString

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViString are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViChar[]

Pass the value which you want to verify as a valid value for the

attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ClearError

ViStatus Chr63000_ClearError (ViSession instrumentHandle);

Purpose

This function clears the error code and error description for the IVI

session. If the user specifies a valid IVI session for the

instrument_handle parameter, this function clears the error information

for the session. If the user passes VI_NULL for the Vi parameter, this

function clears the error information for the current execution thread.

If the Vi parameter is an invalid session, the function does nothing and

returns an error.

The function clears the error code by setting it to VI_SUCCESS.  If the

error description string is non-NULL, the function de-allocates the error

description string and sets the address to VI_NULL.

Maintaining the error information separately for each thread is useful if

the user does not have a session handle to pass to the Chr63000_GetError

function, which occurs when a call to Chr63000_init or

Chr63000_InitWithOptions fails.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ClearInterchangeWarnings

ViStatus Chr63000_ClearInterchangeWarnings (ViSession instrumentHandle);

Purpose

This function clears the list of current interchange warnings.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ClearSequence

ViStatus Chr63000_ClearSequence (ViSession instrumentHandle,

ViInt32 programNumber);

Purpose

Clear all sequence in program file what specified.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

programNumber

Variable Type       ViInt32

Clear all sequence in program file what specified.

Valid Values: 1~10

Default Value: 1

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_close

ViStatus Chr63000_close (ViSession instrumentHandle);

Purpose

This function performs the following operations:

- Closes the instrument I/O session.

- Destroys the instrument driver session and all of its attributes.

- Deallocates any memory resources the driver uses.

Notes:

(1) You must unlock the session before calling Chr63000_close.

(2) After calling Chr63000_close, you cannot use the instrument driver

again until you call Chr63000_init or Chr63000_InitWithOptions.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureCCDModeLoadValue

ViStatus Chr63000_ConfigureCCDModeLoadValue (ViSession instrumentHandle,

ViReal64 l1, ViReal64 l2);

Purpose

Configure CCD Mode Load Current.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

l1

Variable Type       ViReal64

Value of Load

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          Ampere

Default Value: 0.5

Notice : When device run parallel mode,the range of command will be

changed  and this description will be invalid.

l2

Variable Type       ViReal64

Value of Load

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          Ampere

Default Value: 0.5

Notice : When device run parallel mode,the range of command will be

changed  and this description will be invalid.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureCCDModeMeasureRange

ViStatus Chr63000_ConfigureCCDModeMeasureRange

(ViSession instrumentHandle, ViInt32 measureRange);

Purpose

Configure CCD Mode Voltage Measure Range

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

measureRange

Variable Type       ViInt32

Set Measure Range

Valid Values:

0 Low (Default Value)

1 Middle

2 High

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureCCDModeRepeat

ViStatus Chr63000_ConfigureCCDModeRepeat (ViSession instrumentHandle,

ViInt32 repeat);

Purpose

Configure CCD Mode Repeat.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

repeat

Variable Type       ViInt32

Set the repeat count for constant current dynamic mode.

Valid Range:

Range         0-65535

Resolution    1

Unit          None

Default Value:0

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureCCDModeSlewRate

ViStatus Chr63000_ConfigureCCDModeSlewRate (ViSession instrumentHandle,

ViReal64 fallingSlewRate,

ViReal64 risingSlewRate);

Purpose

Configure CCD Mode Current Slew Rate

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

fallingSlewRate

Variable Type       ViReal64

Set the falling slew rate of current for constant current dynamic

mode.

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          A/gs

Default Value:0.1

risingSlewRate

Variable Type       ViReal64

Set the rising slew rate of current for constant current dynamic

mode.

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          A/gs

Default Value:0.1

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureCCDModeTime

ViStatus Chr63000_ConfigureCCDModeTime (ViSession instrumentHandle,

ViReal64 t1, ViReal64 t2);

Purpose

Configure CCD Mode Time

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

t1

Variable Type       ViReal64

Set duration parameter T1 for dynamic mode.

Valid Range:

Range         0.00005 ~ 99.999

Resolution    0.000001

Unit          Second

Default Value:1

t2

Variable Type       ViReal64

Set duration parameter T2 for dynamic mode.

Valid Range:

Range         0.00005 ~ 99.999

Resolution    0.000001

Unit          Second

Default Value:1

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureCCModeLoadValue

ViStatus Chr63000_ConfigureCCModeLoadValue (ViSession instrumentHandle,

ViReal64 loadValue);

Purpose

Set the static load current for constant current static mode.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

loadValue

Variable Type       ViReal64

Set the static load current for constant current static mode.

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          Ampere

Default Value:0

Notice : When device run parallel mode,the range of command will be

changed  and this description will be invalid.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureCCModeMeasureRange

ViStatus Chr63000_ConfigureCCModeMeasureRange

(ViSession instrumentHandle, ViInt32 measureRange);

Purpose

Set the voltage measurement range in CC mode.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

measureRange

Variable Type       ViInt32

Set Measure Range

Valid Values:

0 Low (Default Value)

1 Middle

2 High

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureCCModeSlewRate

ViStatus Chr63000_ConfigureCCModeSlewRate (ViSession instrumentHandle,

ViReal64 fallingSlewRate,

ViReal64 risingSlewRate);

Purpose

Configure CC static Mode Slew Rate of current.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

fallingSlewRate

Variable Type       ViReal64

Set the falling slew rate of current for constant current static

mode.

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          A/gs

Default Value: 0.01

risingSlewRate

Variable Type       ViReal64

Set the rising slew rate of current for constant current static mode.

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          A/gs

Default Value: 0.01

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureCPModeLoadValue

ViStatus Chr63000_ConfigureCPModeLoadValue (ViSession instrumentHandle,

ViReal64 loadValue);

Purpose

Configure CP Mode Static Load Power

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

loadValue

Variable Type       ViReal64

Set the static load power for constant power mode.

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          Watt

Default Value:0

Notice : When device run parallel mode,the range of command will be

changed  and this description will be invalid.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureCPModeMeasureRange

ViStatus Chr63000_ConfigureCPModeMeasureRange

(ViSession instrumentHandle, ViInt32 measureRange);

Purpose

Configure CP Mode Voltage Measure Range

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

measureRange

Variable Type       ViInt32

Set Measure Range

Valid Values:

0 Low (Default Value)

1 Middle

2 High

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureCPModeSlewRate

ViStatus Chr63000_ConfigureCPModeSlewRate (ViSession instrumentHandle,

ViReal64 fallingSlewRate,

ViReal64 risingSlewRate);

Purpose

Configure CP Mode Current Slew Rate

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

fallingSlewRate

Variable Type       ViReal64

Set the falling slew rate of current for constant power mode.

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          A/gs

Default Value:0.1

risingSlewRate

Variable Type       ViReal64

Set the rising slew rate of current for constant power mode.

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          A/gs

Default Value:0.1

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureCRModeLoadValue

ViStatus Chr63000_ConfigureCRModeLoadValue (ViSession instrumentHandle,

ViReal64 loadValue);

Purpose

Configure CR Mode Static Resistance Level

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

loadValue

Variable Type       ViReal64

Set static resistance level for constant resistance mode.

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          Ohm

Default Value: 0.5

Notice : When device run parallel mode,the range of command will be

changed  and this description will be invalid.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureCRModeMeasureRange

ViStatus Chr63000_ConfigureCRModeMeasureRange

(ViSession instrumentHandle, ViInt32 measureRange);

Purpose

Configure CR Mode Current Measure Range

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

measureRange

Variable Type       ViInt32

Set Measure Range

Valid Values:

0 Low (Default Value)

1 Middle

2 High

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureCRModeSlewRate

ViStatus Chr63000_ConfigureCRModeSlewRate (ViSession instrumentHandle,

ViReal64 fallingSlewRate,

ViReal64 risingSlewRate);

Purpose

Configure CR Mode Slew Rate of current

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

fallingSlewRate

Variable Type       ViReal64

Set the falling slew rate of current for constant resistance mode.

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          A/gs

Default Value: 0.1

risingSlewRate

Variable Type       ViReal64

Set the rising slew rate of current for constant resistance mode.

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          A/gs

Default Value: 0.1

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureCurrentSpecification

ViStatus Chr63000_ConfigureCurrentSpecification

(ViSession instrumentHandle, ViReal64 lowLevelCurrnet,

ViReal64 centerLevelCurrnet, ViReal64 highLevelCurrnet);

Purpose

Configure Current Specification

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

lowLevelCurrnet

Variable Type       ViReal64

Set the low-level current specification.

Valid Range:

Refer to the user manual for the valid range of the model.

Default Value: 0

centerLevelCurrnet

Variable Type       ViReal64

Set the center-level current specification.

Valid Range:

Refer to the user manual for the valid range of the model.

Default Value: 0

highLevelCurrnet

Variable Type       ViReal64

Set the high-level current specification.

Valid Range:

Refer to the user manual for the valid range of the model.

Default Value: 0

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureCVModeCurrentLimit

ViStatus Chr63000_ConfigureCVModeCurrentLimit

(ViSession instrumentHandle, ViReal64 currentLimit);

Purpose

Configure CV Mode Current Limit

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

currentLimit

Variable Type       ViReal64

Set the current limit for constant voltage mode.

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          Ampere

Default Value:0

Notice : When device run parallel mode,the range of command will be

changed  and this description will be invalid.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureCVModeLoadValue

ViStatus Chr63000_ConfigureCVModeLoadValue (ViSession instrumentHandle,

ViReal64 loadValue);

Purpose

Configure CV Mode Static Load Voltage

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

loadValue

Variable Type       ViReal64

Set the static load voltage in constant voltage mode.

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          Volt

Default Value:0

Notice : When device run parallel mode,the range of command will be

changed  and this description will be invalid.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureCVModeResponseSpeed

ViStatus Chr63000_ConfigureCVModeResponseSpeed

(ViSession instrumentHandle, ViInt32 responseSpeed);

Purpose

Configure CV Mode Response Speed

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

responseSpeed

Variable Type       ViInt32

Set the response speed in constant voltage mode.

Valid Values:

0 - Slow (Default Value)

1 - Normal

2 - Fast

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigurePowerSpecification

ViStatus Chr63000_ConfigurePowerSpecification

(ViSession instrumentHandle, ViReal64 lowLevelPower,

ViReal64 centerLevelPower, ViReal64 highLevelPower);

Purpose

Configure Power Specification

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

lowLevelPower

Variable Type       ViReal64

Set the low-level power specification.

Valid Range:

Refer to the user manual for the valid range of the model.

Default Value: 0

centerLevelPower

Variable Type       ViReal64

Set the center-level power specification.

Valid Range:

Refer to the user manual for the valid range of the model.

Default Value: 0

highLevelPower

Variable Type       ViReal64

Set the high-level power specification.

Valid Range:

Refer to the user manual for the valid range of the model.

Default Value: 0

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureProgramListValue

ViStatus Chr63000_ConfigureProgramListValue (ViSession instrumentHandle,

ViReal64 loadValue,

ViInt32 sequenceIndex);

Purpose

Set the program parameters.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

loadValue

Variable Type       ViReal64

Set the load value according to run mode in sequence:

Valid Range:

Refer to the user manual for the valid range of the model.

Default Value: 0

sequenceIndex

Variable Type       ViInt32

Selects a sequence to be set:

Valid Range :

1 to X

X = Total sequences, Use ConfigureProgramType function to setting.

Default value: 1

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureProgramMode

ViStatus Chr63000_ConfigureProgramMode (ViSession instrumentHandle,

ViInt32 runMode,

ViInt32 modeRange,

ViInt32 triggerMode);

Purpose

Set the program parameters.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

runMode

Variable Type       ViInt32

Set the run mode of sequence

Valid Values: CC(0), CR(1), CV(2), CP(3)

Default Value: CC(0)

modeRange

Variable Type       ViInt32

Set the modes range of sequence

Valid Values: Low(0), Middle(1), High(2)

Default Value: Low(0)

triggerMode

Variable Type       ViInt32

Set the trigger mode of sequence

Valid Values: SKIP(0), AUTO(1), MANUAL(2), EXTERNAL(3)

Default Value: SKIP(0)

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureProgramSpecification

ViStatus Chr63000_ConfigureProgramSpecification

(ViSession instrumentHandle, ViReal64 highLevelVoltage,

ViReal64 lowLevelVoltage, ViReal64 highLevelCurrent,

ViReal64 lowLevelCurrent, ViReal64 highLevelPower,

ViReal64 lowLevelPower);

Purpose

Set the program parameters.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

highLevelVoltage

Variable Type       ViReal64

Set the high-level of voltage specific in sequence:

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          Volt

# -1 :Don't care

Default Value: -1

lowLevelVoltage

Variable Type       ViReal64

Set the low-level of voltage specific in sequence:

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          Volt

# -1 :Don't care

Default Value: -1

highLevelCurrent

Variable Type       ViReal64

Set the high-level of current specific in sequence:

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          Ampere

# -1 :Don't care

Default Value: -1

lowLevelCurrent

Variable Type       ViReal64

Set the low-level of current specific in sequence:

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          Ampere

# -1 :Don't care

Default Value: -1

highLevelPower

Variable Type       ViReal64

Set the high-level of power specific in sequence:

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          Watt

# -1 :Don't care

Default Value: -1

lowLevelPower

Variable Type       ViReal64

Set the low-level of power specific in sequence:

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          Watt

# -1 :Don't care

Default Value: -1

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureProgramStepValue

ViStatus Chr63000_ConfigureProgramStepValue (ViSession instrumentHandle,

ViReal64 startValue,

ViReal64 endValue);

Purpose

Set the program parameters.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

startValue

Variable Type       ViReal64

Set the start value according to run mode in sequence:

Valid Range:

Refer to the user manual for the valid range of the model.

Default Value: 0

endValue

Variable Type       ViReal64

Set the end value according to run mode in sequence:

Valid Range:

Refer to the user manual for the valid range of the model.

Default Value: 0

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureProgramTime

ViStatus Chr63000_ConfigureProgramTime (ViSession instrumentHandle,

ViReal64 risingSlewRate,

ViReal64 fallingSlewRate,

ViReal64 dwellTime,

ViReal64 delayTime);

Purpose

Set the program parameters.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

risingSlewRate

Variable Type       ViReal64

Set the rising of slew rate in sequence

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          A/gs

Default Value: 0.5

fallingSlewRate

Variable Type       ViReal64

Set the falling of slew rate in sequence

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          A/gs

Default Value: 0.5

dwellTime

Variable Type       ViReal64

Set the dwell time of sequence:

Valid Range:

Range         0.5m ~ 30

Resolution    0.1m

Unit          Second

Default Value: 1

delayTime

Variable Type       ViReal64

Set the delay time of Pass/Fail in sequence:

Valid Range:

Range         0s ~ 30s

Resolution    0.0001

Unit          Second

Default Value: 0

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureProgramType

ViStatus Chr63000_ConfigureProgramType (ViSession instrumentHandle,

ViInt32 type, ViInt32 chain,

ViInt32 repeat,

ViInt32 sequence,

ViInt32 programNumber);

Purpose

Set the program parameters.

If you want to use program mode, you need use this function at the

beginning.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

type

Variable Type       ViInt32

Set the type of program

Valid Values:

List(0) - Default Value

Step(1)

chain

Variable Type       ViInt32

Set the chain parameter in program

Valid Range:

Range         0-10

Resolution    1

Unit          None

Default Value: 0

repeat

Variable Type       ViInt32

Set the repeat count of program

0 999

Valid Range:

Range         0-100

Resolution    1

Unit          None

Default Value: 0

sequence

Variable Type       ViInt32

Set Total number of sequence in program

Valid Range:

Range         1-120

Resolution    1

Unit          None

Default Value: 1

programNumber

Variable Type       ViInt32

Selects a program to be set

Valid Values: 1~10

Default Value: 1

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ConfigureVoltageSpecification

ViStatus Chr63000_ConfigureVoltageSpecification

(ViSession instrumentHandle, ViReal64 lowLevelVoltage,

ViReal64 centerLevelVoltage, ViReal64 highLevelVoltage);

Purpose

Configure Voltage Specification

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

lowLevelVoltage

Variable Type       ViReal64

Set the low-level voltage specification.

Valid Range:

Refer to the user manual for the valid range of the model.

Default Value: 0

centerLevelVoltage

Variable Type       ViReal64

Set the center-level voltage specification.

Valid Range:

Refer to the user manual for the valid range of the model.

Default Value: 0

highLevelVoltage

Variable Type       ViReal64

Set the high-level voltage specification.

Valid Range:

Refer to the user manual for the valid range of the model.

Default Value:0

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_DigitizingAbort

ViStatus Chr63000_DigitizingAbort (ViSession instrumentHandle);

Purpose

Abort digitizing function.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_DigitizingInitiate

ViStatus Chr63000_DigitizingInitiate (ViSession instrumentHandle);

Purpose

Init digitizing function.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_DigitizingSamplingPoint

ViStatus Chr63000_DigitizingSamplingPoint (ViSession instrumentHandle,

ViInt32 point);

Purpose

Set the sampling Point

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the chr63200A_init or

chr63200A_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

point

Variable Type       ViInt32

Set the sampling points for digitizing function.

Valid Range:

1024 - 15000

Unit        None

Default Value: 1024

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

chr63200A_error_message function.  To obtain additional information

about the error condition, call the chr63200A_GetError function.  To

clear the error information from the driver, call the

chr63200A_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_DigitizingSamplingTime

ViStatus Chr63000_DigitizingSamplingTime (ViSession instrumentHandle,

ViReal64 time);

Purpose

Set the sampling time

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the chr63200A_init or

chr63200A_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

time

Variable Type       ViReal64

Set the sampling time for digitizing function.

Valid Range:

0.002 to 100

Unit: Millisecond

Default Value: 0.1

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

chr63200A_error_message function.  To obtain additional information

about the error condition, call the chr63200A_GetError function.  To

clear the error information from the driver, call the

chr63200A_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_DigitizingTrigger

ViStatus Chr63000_DigitizingTrigger (ViSession instrumentHandle,

ViInt32 state);

Purpose

Set Trigger State of Digitizing Function.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

state

Variable Type       ViInt32

Set the software trigger for digitizing function.

Valid Values:

Off(0) - Default Value

On(1)

Trigger Source in Bus can use.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_DigitizingTriggerPoint

ViStatus Chr63000_DigitizingTriggerPoint (ViSession instrumentHandle,

ViInt32 point);

Purpose

Set Trigger Point of Digitizing Function.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the chr63200A_init or

chr63200A_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

point

Variable Type       ViInt32

Set the trigger points for digitizing function.

Valid Range:

1 - 15000

Unit        None

Default Value: 1

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

chr63200A_error_message function.  To obtain additional information

about the error condition, call the chr63200A_GetError function.  To

clear the error information from the driver, call the

chr63200A_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_DigitizingTriggerSource

ViStatus Chr63000_DigitizingTriggerSource (ViSession instrumentHandle,

ViInt32 source);

Purpose

Set Trigger Source of Digitizing Function.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the chr63200A_init or

chr63200A_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

source

Variable Type       ViInt32

Set the trigger source for digitizing function.

Valid Values:

0 - load on (Default Value)

1 - load off

2 - TTL

3 - BUS

4 - MANUAL

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

chr63200A_error_message function.  To obtain additional information

about the error condition, call the chr63200A_GetError function.  To

clear the error information from the driver, call the

chr63200A_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_DigitizingWaveform

ViStatus Chr63000_DigitizingWaveform (ViSession instrumentHandle,

ViReal64 _VI_FAR dataResponse[],

ViInt32 returnItem);

Purpose

This query returns voltage or current waveform data from the DC

Electronic Load in binary format. The waveform either voltage or current

are consist of number points correspond to sampling points that user

specified in format of 32bits float point.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

dataResponse

Variable Type       ViReal64[]

This query returns voltage or current waveform data from the DC

Electronic Load in binary format. The waveform either voltage or

current are consist of number points correspond to sampling points

that user specified in format of 32bits float point.

returnItem

Variable Type       ViInt32

Select Return item

Valid Values:

Voltage(0) - Default Value

Current(1)

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_DIO

ViStatus Chr63000_DIO (ViSession instrumentHandle, ViInt32 DI1Type,

ViInt32 DO1Type, ViBoolean DO1, ViInt32 DI2Type,

ViInt32 DO2Type, ViBoolean DO2);

Purpose

Configure Digital I/O

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

DI1Type

Variable Type       ViInt32

Set the DI1 type the pin No.11 in System I/O Port.

Valid Values:

0 - NONE

1 - EXTERNAL LOAD ON/OFF

2 - REMOTE INHIBIT

Default Value : 0

DO1Type

Variable Type       ViInt32

Set the DO1 type the pin No.8 in System I/O Port.

Valid Values:

0GNONE

1GGONG TOTAL PASS-H

2GGONG TOTAL FAIL-L

3GOTP OVP OCP OPP REV-H

4GBUS CTRL. ACT-H

5GBUS CTRL. ACT-L

Default Value : 0

DO1

Variable Type       ViBoolean

It sets the system I/O port pin 8 DO1 status when the BUS CTRL. mode

is selected for DO1.

Valid Values:

Off (0) - Off (Default Value)

On  (1) - On

DI2Type

Variable Type       ViInt32

Set the DI2 type the pin No.12 in System I/O Port.

Valid Values:

0 - NONE

1 - EXTERNAL LOAD ON/OFF

2 - REMOTE INHIBIT

Default Value : 0

DO2Type

Variable Type       ViInt32

Set the DO2 type the pin No.9 in System I/O Port.

Valid Values:

0GNONE

1GGONG TOTAL PASS-H

2GGONG TOTAL FAIL-L

3GOTP OVP OCP OPP REV-H

4GBUS CTRL. ACT-H

5GBUS CTRL. ACT-L

Default Value : 0

DO2

Variable Type       ViBoolean

It sets the system I/O port pin 9 DO2 status when the BUS CTRL. mode

is selected for DO2.

Valid Values:

Off (0) - Off (Default Value)

On  (1) - On

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ElectronicLoadState

ViStatus Chr63000_ElectronicLoadState (ViSession instrumentHandle,

ViBoolean trigger,

ViBoolean shortState,

ViBoolean _VI_FAR loadStatus[],

ViBoolean clear);

Purpose

Set the load and short states.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

trigger

Variable Type       ViBoolean

Electronic load active/on or inactive/off.

Valid Values:

0 - Off(Default value)

1 - On

shortState

Variable Type       ViBoolean

Activate or inactivate short-circuited simulation.

Valid Values:

0 - Off(Default value)

1 - On

loadStatus

Variable Type       ViBoolean[]

Return the load status in Boolean Array with 15 elements.

Status[i]

0 : OV1

1 : OV2

2 : REV

3 : OCP1

4 : OCP2

5 : OCP3

6 : OPP1

7 : OPP2

8 : OPP3

9 : OTP

10:

11: FAN

12: VCC

13: RMT_INH

14:

15:

clear

Variable Type       ViBoolean

This command resets the status of electronic load.

Valid Values:

0 - Off(Default value)

1 - On

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_error_message

ViStatus Chr63000_error_message (ViSession instrumentHandle,

ViStatus errorCode,

ViChar _VI_FAR errorMessage[]);

Purpose

This function converts a status code returned by an instrument driver

function into a user-readable string.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

You can pass VI_NULL for this parameter.  This is useful when one of

the initialize functions fail.

Default Value:  VI_NULL

errorCode

Variable Type       ViStatus

Pass the Status parameter that is returned from any of the instrument

driver functions.

Default Value:  0  (VI_SUCCESS)

errorMessage

Variable Type       ViChar[]

Returns the user-readable message string that corresponds to the

status code you specify.

You must pass a ViChar array with at least 256 bytes.

Return Value

Reports the status of this operation.

This function can return only three possible status codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0085  Unknown status code (warning).

BFFF000A  Invalid parameter (Error Message buffer is VI_NULL).

________________________________________________________________________________

Chr63000_error_query

ViStatus Chr63000_error_query (ViSession instrumentHandle,

ViPInt32 errorCode,

ViChar _VI_FAR errorMessage[]);

Purpose

This function reads an error code and a message from the instrument's

error queue.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

errorCode

Variable Type       ViInt32 (passed by reference)

Returns the error code read from the instrument's error queue.

0,No Error,

1,Data Format Error,

2,Data Range Error,

3,Command Error,

4,Execution Error,

5,Too Many Errors

errorMessage

Variable Type       ViChar[]

Returns the error message string read from the instrument's error

message queue.

You must pass a ViChar array with at least 256 bytes.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_FailOfSequence

ViStatus Chr63000_FailOfSequence (ViSession instrumentHandle,

ViChar _VI_FAR result[]);

Purpose

This command returns the fail of sequence in specification.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

result

Variable Type       ViChar[]

xx-xxx,xx-xxx,xx-xxxKetc, which front of - is the program number

and rear of - is the sequence number.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_Fetch

ViStatus Chr63000_Fetch (ViSession instrumentHandle, ViInt32 item,

ViPReal64 result);

Purpose

Returns the value measured.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

item

Variable Type       ViInt32

Select Item

Valid Values:

0:CURRent

1:POWer

2:STATus

3:TIME

4:VOLTage

Default Value: 0

result

Variable Type       ViReal64 (passed by reference)

FETCh:CURRent?

Returns the current measured at electronic load input.

FETCh:POWer?

Returns the power measured at electronic load input.

FETCh:STATus?

Returns real time status of the load module.

-----------------------

Weight

32768

16384     MAX_LIM

8192      RMT_INH

4096      VCC

2048      FAN

1024      SYNC

512       OTP

256       OPP3

128       OPP2

64        OPP1

32        OCP3

16        OCP2

8         OCP1

4         REV

2         OV2

1         OV1

------------------------

FETCh:TIME?

Returns the time measured in timing mode.

FETCh:VOLTage?

Returns the voltage measured at electronic load input.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_GetAttributeViBoolean

ViStatus Chr63000_GetAttributeViBoolean (ViSession instrumentHandle,

ViChar _VI_FAR channelName[],

ViAttr attributeID,

ViPBoolean attributeValue);

Purpose

This function queries the value of a ViBoolean attribute.

You can use this function to get the values of instrument- specific

attributes and inherent IVI attributes.  If the attribute represents an

instrument state, this function performs instrument I/O in the following

cases:

- State caching is disabled for the entire session or for the particular

attribute.

- State caching is enabled and the currently cached value is invalid.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViChar[]

If the attribute is channel-based, this parameter specifies the name

of the channel on which to obtain the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Help text is

shown for each attribute.  Select an attribute by

double-clicking on it or by selecting it and then pressing

<ENTER>.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViBoolean

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViBoolean are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViBoolean (passed by reference)

Returns the current value of the attribute.  Pass the address of a

ViBoolean variable.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has named constants as valid values, you can view a

list of the constants by pressing <ENTER> on this control.

Select a value by double-clicking on it or by selecting it and

then pressing <ENTER>.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_GetAttributeViInt32

ViStatus Chr63000_GetAttributeViInt32 (ViSession instrumentHandle,

ViChar _VI_FAR channelName[],

ViAttr attributeID,

ViPInt32 attributeValue);

Purpose

This function queries the value of a ViInt32 attribute.

You can use this function to get the values of instrument- specific

attributes and inherent IVI attributes.  If the attribute represents an

instrument state, this function performs instrument I/O in the following

cases:

- State caching is disabled for the entire session or for the particular

attribute.

- State caching is enabled and the currently cached value is invalid.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViChar[]

If the attribute is channel-based, this parameter specifies the name

of the channel on which to obtain the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Help text is

shown for each attribute.  Select an attribute by

double-clicking on it or by selecting it and then pressing

<ENTER>.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViInt32 type.

If you choose to see all IVI attributes, the data types appear

to the right of the attribute names in the list box.

Attributes with data types other than ViInt32 are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViInt32 (passed by reference)

Returns the current value of the attribute.  Pass the address of a

ViInt32 variable.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has named constants as valid values, you can view a

list of the constants by pressing <ENTER> on this control.

Select a value by double-clicking on it or by selecting it and

then pressing <ENTER>.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_GetAttributeViReal64

ViStatus Chr63000_GetAttributeViReal64 (ViSession instrumentHandle,

ViChar _VI_FAR channelName[],

ViAttr attributeID,

ViPReal64 attributeValue);

Purpose

This function queries the value of a ViReal64 attribute.

You can use this function to get the values of instrument- specific

attributes and inherent IVI attributes.  If the attribute represents an

instrument state, this function performs instrument I/O in the following

cases:

- State caching is disabled for the entire session or for the particular

attribute.

- State caching is enabled and the currently cached value is invalid.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViChar[]

If the attribute is channel-based, this parameter specifies the name

of the channel on which to obtain the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Help text is

shown for each attribute.  Select an attribute by

double-clicking on it or by selecting it and then pressing

<ENTER>.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViReal64

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViReal64 are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViReal64 (passed by reference)

Returns the current value of the attribute.  Pass the address of a

ViReal64 variable.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has named constants as valid values, you can view a

list of the constants by pressing <ENTER> on this control.

Select a value by double-clicking on it or by selecting it and

then pressing <ENTER>.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_GetAttributeViSession

ViStatus Chr63000_GetAttributeViSession (ViSession instrumentHandle,

ViChar _VI_FAR channelName[],

ViAttr attributeID,

ViPSession attributeValue);

Purpose

This function queries the value of a ViSession attribute.

You can use this function to get the values of instrument- specific

attributes and inherent IVI attributes.  If the attribute represents an

instrument state, this function performs instrument I/O in the following

cases:

- State caching is disabled for the entire session or for the particular

attribute.

- State caching is enabled and the currently cached value is invalid.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViChar[]

If the attribute is channel-based, this parameter specifies the name

of the channel on which to obtain the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Help text is

shown for each attribute.  Select an attribute by

double-clicking on it or by selecting it and then pressing

<ENTER>.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViSession

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViSession are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViSession (passed by reference)

Returns the current value of the attribute.  Pass the address of a

ViSession variable.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has named constants as valid values, you can view a

list of the constants by pressing <ENTER> on this control.

Select a value by double-clicking on it or by selecting it and

then pressing <ENTER>.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_GetAttributeViString

ViStatus Chr63000_GetAttributeViString (ViSession instrumentHandle,

ViChar _VI_FAR channelName[],

ViAttr attributeID,

ViInt32 bufferSize,

ViChar _VI_FAR attributeValue[]);

Purpose

This function queries the value of a ViString attribute.

You can use this function to get the values of instrument- specific

attributes and inherent IVI attributes.  If the attribute represents an

instrument state, this function performs instrument I/O in the following

cases:

- State caching is disabled for the entire session or for the particular

attribute.

- State caching is enabled and the currently cached value is invalid.

You must provide a ViChar array to serve as a buffer for the value.  You

pass the number of bytes in the buffer as the Buffer Size parameter.  If

the current value of the attribute, including the terminating NUL byte,

is larger than the size you indicate in the Buffer Size parameter, the

function copies Buffer Size - 1 bytes into the buffer, places an ASCII

NUL byte at the end of the buffer, and returns the buffer size you must

pass to get the entire value.  For example, if the value is "123456" and

the Buffer Size is 4, the function places "123" into the buffer and

returns 7.

If you want to call this function just to get the required buffer size,

you can pass 0 for the Buffer Size and VI_NULL for the Attribute Value

buffer.

If you want the function to fill in the buffer regardless of the   number

of bytes in the value, pass a negative number for the Buffer Size

parameter.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViChar[]

If the attribute is channel-based, this parameter specifies the name

of the channel on which to obtain the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Help text is

shown for each attribute.  Select an attribute by

double-clicking on it or by selecting it and then pressing

<ENTER>.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViString

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViString are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

bufferSize

Variable Type       ViInt32

Pass the number of bytes in the ViChar array you specify for the

Attribute Value parameter.

If the current value of the attribute, including the terminating NUL

byte, contains more bytes that you indicate in this parameter, the

function copies Buffer Size - 1 bytes into the buffer, places an

ASCII NUL byte at the end of the buffer, and returns the buffer size

you must pass to get the entire value.  For example, if the value is

"123456" and the Buffer Size is 4, the function places "123" into the

buffer and returns 7.

If you pass a negative number, the function copies the value to the

buffer regardless of the number of bytes in the value.

If you pass 0, you can pass VI_NULL for the Attribute Value buffer

parameter.

Default Value:512

attributeValue

Variable Type       ViChar[]

The buffer in which the function returns the current value of the

attribute.  The buffer must be of type ViChar and have at least as

many bytes as indicated in the Buffer Size parameter.

If the current value of the attribute, including the terminating NUL

byte, contains more bytes that you indicate in this parameter, the

function copies Buffer Size - 1 bytes into the buffer, places an

ASCII NUL byte at the end of the buffer, and returns the buffer size

you must pass to get the entire value.  For example, if the value is

"123456" and the Buffer Size is 4, the function places "123" into the

buffer and returns 7.

If you specify 0 for the Buffer Size parameter, you can pass VI_NULL

for this parameter.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has named constants as valid values, you can view a

list of the constants by pressing <ENTER> on this control.

Select a value by double-clicking on it or by selecting it and

then pressing <ENTER>.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

If the current value of the return buffer, including the terminating

NUL byte, is larger than the size you indicate in the Buffer Size

parameter, the function copies Buffer Size - 1 bytes into the buffer,

places an ASCII NUL byte at the end of the buffer, and returns the

buffer size you must pass to get the entire value.  For example, if

the value is "123456" and the Buffer Size is 4, the function places

"123" into the buffer and returns 7.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_GetError

ViStatus Chr63000_GetError (ViSession instrumentHandle, ViPStatus code,

ViInt32 bufferSize,

ViChar _VI_FAR description[]);

Purpose

This function retrieves and then clears the IVI error information for the

session or the current execution thread. One exception exists: If the

BufferSize parameter is 0, the function does not clear the error

information. By passing 0 for the buffer size, the caller can ascertain

the buffer size required to get the entire error description string and

then call the function again with a sufficiently large buffer.

If the user specifies a valid IVI session for the InstrumentHandle

parameter, Get Error retrieves and then clears the error information for

the session.  If the user passes VI_NULL for the InstrumentHandle

parameter, this function retrieves and then clears the error information

for the current execution thread.  If the InstrumentHandle parameter is

an invalid session, the function does nothing and returns an error.

Normally, the error information describes the first error that occurred

since the user last called Chr63000_GetError or Chr63000_ClearError.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

code

Variable Type       ViStatus (passed by reference)

Returns the error code for the session or execution thread.

If you pass 0 for the Buffer Size, you can pass VI_NULL for this

parameter.

bufferSize

Variable Type       ViInt32

Pass the number of bytes in the ViChar array you specify for the

Description parameter.

If the error description, including the terminating NUL byte,

contains more bytes than you indicate in this parameter, the function

copies BufferSize - 1 bytes into the buffer, places an ASCII NUL byte

at the end of the buffer, and returns the buffer size you must pass

to get the entire value.  For example, if the value is "123456" and

the Buffer Size is 4, the function places "123" into the buffer and

returns 7.

If you pass a negative number, the function copies the value to the

buffer regardless of the number of bytes in the value.

If you pass 0, you can pass VI_NULL for the Description buffer

parameter.

Default Value:  None

description

Variable Type       ViChar[]

Returns the error description for the IVI session or execution

thread.  If there is no description, the function returns an empty

string.

The buffer must contain at least as many elements as the value you

specify with the Buffer Size parameter.  If the error description,

including the terminating NUL byte, contains more bytes than you

indicate with the Buffer Size parameter, the function copies Buffer

Size - 1 bytes into the buffer, places an ASCII NUL byte at the end

of the buffer, and returns the buffer size you must pass to get the

entire value.  For example, if the value is "123456" and the Buffer

Size is 4, the function places "123" into the buffer and returns 7.

If you pass 0 for the Buffer Size, you can pass VI_NULL for this

parameter.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

If the current value of the return buffer, including the terminating

NUL byte, is larger than the size you indicate in the Buffer Size

parameter, the function copies Buffer Size - 1 bytes into the buffer,

places an ASCII NUL byte at the end of the buffer, and returns the

buffer size you must pass to get the entire value.  For example, if

the value is "123456" and the Buffer Size is 4, the function places

"123" into the buffer and returns 7.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This driver defines the following status codes:

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_GetNextCoercionRecord

ViStatus Chr63000_GetNextCoercionRecord (ViSession instrumentHandle,

ViInt32 bufferSize,

ViChar _VI_FAR coercionRecord[]);

Purpose

This function returns the coercion information associated with the IVI

session.  This function retrieves and clears the oldest instance in which

the instrument driver coerced a value you specified to another value.

If you set the CHR63000_ATTR_RECORD_COERCIONS attribute to VI_TRUE, the

instrument driver keeps a list of all coercions it makes on ViInt32 or

ViReal64 values you pass to instrument driver functions.  You use this

function to retrieve information from that list.

If the next coercion record string, including the terminating NUL byte,

contains more bytes than you indicate in this parameter, the function

copies Buffer Size - 1 bytes into the buffer, places an ASCII NUL byte at

the end of the buffer, and returns the buffer size you must pass to get

the entire value.  For example, if the value is "123456" and the Buffer

Size is 4, the function places "123" into the buffer and returns 7.

If you pass a negative number, the function copies the value to the

buffer regardless of the number of bytes in the value.

If you pass 0, you can pass VI_NULL for the Coercion Record buffer

parameter.

The function returns an empty string in the Coercion Record parameter if

no coercion records remain for the session.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init function.

The handle identifies a particular instrument session.

Default Value:  None

bufferSize

Variable Type       ViInt32

Pass the number of bytes in the ViChar array you specify for the

Coercion Record parameter.

If the next coercion record string, including the terminating NUL

byte, contains more bytes than you indicate in this parameter, the

function copies Buffer Size - 1 bytes into the buffer, places an

ASCII NUL byte at the end of the buffer, and returns the buffer size

you must pass to get the entire value.  For example, if the value is

"123456" and the Buffer Size is 4, the function places "123" into the

buffer and returns 7.

If you pass a negative number, the function copies the value to the

buffer regardless of the number of bytes in the value.

If you pass 0, you can pass VI_NULL for the Coercion Record buffer

parameter.

Default Value:  None

coercionRecord

Variable Type       ViChar[]

Returns the next coercion record for the IVI session.  If there are

no coercion records, the function returns an empty string.

The buffer must contain at least as many elements as the value you

specify with the Buffer Size parameter.  If the next coercion record

string, including the terminating NUL byte, contains more bytes than

you indicate with the Buffer Size parameter, the function copies

Buffer Size - 1 bytes into the buffer, places an ASCII NUL byte at

the end of the buffer, and returns the buffer size you must pass to

get the entire value.  For example, if the value is "123456" and the

Buffer Size is 4, the function places "123" into the buffer and

returns 7.

This parameter returns an empty string if no coercion records remain

for the session.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

If the current value of the return buffer, including the terminating

NUL byte, is larger than the size you indicate in the Buffer Size

parameter, the function copies Buffer Size - 1 bytes into the buffer,

places an ASCII NUL byte at the end of the buffer, and returns the

buffer size you must pass to get the entire value.  For example, if

the value is "123456" and the Buffer Size is 4, the function places

"123" into the buffer and returns 7.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_GetNextInterchangeWarning

ViStatus Chr63000_GetNextInterchangeWarning (ViSession instrumentHandle,

ViInt32 bufferSize,

ViChar _VI_FAR interchangeWarning[]);

Purpose

You use this function to retrieve interchangeability warnings

This function returns the interchangeability warnings associated with the

IVI session.

It retrieves and clears the oldest instance in which the class driver

recorded an interchangeability warning.

Interchangeability warnings indicate that using your application with a

different instrument might cause different behavior.

The function returns an empty string in the Interchange Warning parameter

if no interchangeability warnings remain for the session.

In general, the instrument driver generates interchangeability warnings

when an attribute that affects the behavior of the instrument is in a

state that you did not specify.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

bufferSize

Variable Type       ViInt32

Pass the number of bytes in the ViChar array you specify for the

Interchange Warning parameter.

If the next interchangeability warning string, including the

terminating NUL byte, contains more bytes than you indicate in this

parameter, the function copies Buffer Size - 1 bytes into the buffer,

places an ASCII NUL byte at the end of the buffer, and returns the

buffer size you must pass to get the entire value. For example, if

the value is "123456" and the Buffer Size is 4, the function places

"123" into the buffer and returns 7.

If you pass a negative number, the function copies the value to the

buffer regardless of the number of bytes in the value.

If you pass 0, you can pass VI_NULL for the Interchange Warning

buffer parameter.

Default Value:  None

interchangeWarning

Variable Type       ViChar[]

Returns the next interchange warning for the IVI session. If there

are no interchange warnings, the function returns an empty  string.

The buffer must contain at least as many elements as the value you

specify with the Buffer Size parameter. If the next

interchangeability warning string, including the terminating NUL

byte, contains more bytes than you indicate with the Buffer Size

parameter, the function copies Buffer Size - 1 bytes into the buffer,

places an ASCII NUL byte at the end of the buffer, and returns the

buffer size you must pass to get the entire value.  For example, if

the value is "123456" and the Buffer Size is 4, the function places

"123" into the buffer and returns 7.

This parameter returns an empty string if no interchangeability

warnings remain for the session.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

If the current value of the return buffer, including the terminating

NUL byte, is larger than the size you indicate in the Buffer Size

parameter, the function copies Buffer Size - 1 bytes into the buffer,

places an ASCII NUL byte at the end of the buffer, and returns the

buffer size you must pass to get the entire value.  For example, if

the value is "123456" and the Buffer Size is 4, the function places

"123" into the buffer and returns 7.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_GetProgramStates

ViStatus Chr63000_GetProgramStates (ViSession instrumentHandle,

ViPInt32 programNumber,

ViPInt32 sequnceNumber,

ViPInt32 loadMode,

ViPInt32 executionState);

Purpose

This command returns the information of program running.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

programNumber

Variable Type       ViInt32 (passed by reference)

program number.

sequnceNumber

Variable Type       ViInt32 (passed by reference)

sequence number.

loadMode

Variable Type       ViInt32 (passed by reference)

Load mode

0:CCL,

1:CCM,

2:CCH,

3:CRL,

4:CRM,

5:CRH,

6:CVL,

7:CVM,

8:CVH,

9:CPL,

10:CPM,

11:CPH

executionState

Variable Type       ViInt32 (passed by reference)

execution state, 0:Idle, 1:running, 2:Wait manual trigger, 3:Wait

external trigger

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_init

ViStatus Chr63000_init (ViRsrc resourceName, ViBoolean IDQuery,

ViBoolean resetDevice,

ViPSession instrumentHandle);

Purpose

This function performs the following initialization actions:

- Creates a new IVI instrument driver session.

- Opens a session to the specified device using the interface and address

you specify for the Resource Name parameter.

- If the ID Query parameter is set to VI_TRUE, this function queries the

instrument ID and checks that it is valid for this instrument driver.

- If the Reset parameter is set to VI_TRUE, this function resets the

instrument to a known state.

- Sends initialization commands to set the instrument to the state

necessary for the operation of the instrument driver.

- Returns a ViSession handle that you use to identify the instrument in

all subsequent instrument driver function calls.

Note:  This function creates a new session each time you invoke it.

Although you can open more than one IVI session for the same resource, it

is best not to do so.  You can use the same session in multiple program

threads.  You can use the Chr63000_LockSession and Chr63000_UnlockSession

functions to protect sections of code that require exclusive access to

the resource.

Content from the IVI specifications reproduced with permission from the

IVI Foundation.

The IVI Foundation and its member companies make no warranty of any kind

with regard to this material, including, but not limited to, the implied

warranties of merchantability and fitness for a particular purpose. The

IVI Foundation and its member companies shall not be liable for errors

contained herein or for incidental or consequential damages in connection

with the furnishing, performance, or use of this material.

Parameter List

resourceName

Variable Type       ViRsrc

Pass the resource name of the device to initialize.

You can also pass the name of a driver session or logical name that

you configure with the IVI Configuration utility.  The driver session

identifies a specific device and specifies the initial settings for

the session.  A logical Name identifies a particular driver session.

Refer to the following table below for the exact grammar to use for

this parameter.  Optional fields are shown in square brackets ([]).

Syntax

------------------------------------------------------

GPIB[board]::<primary address>[::secondary address]::INSTR

VXI[board]::<logical address>::INSTR

GPIB-VXI[board]::<logical address>::INSTR

ASRL<port>::INSTR

<LogicalName>

<Driver Session>

If you do not specify a value for an optional field, the following

values are used:

Optional Field - Value

------------------------------------------------------

board - 0

secondary address - none (31)

The following table contains example valid values for this parameter.

"Valid Value" - Description

------------------------------------------------------

"GPIB::22::INSTR" - GPIB board 0, primary address 22 no

secondary address

"GPIB::22::5::INSTR" - GPIB board 0, primary address 22

secondary address 5

"GPIB1::22::5::INSTR" - GPIB board 1, primary address 22

secondary address 5

"VXI::64::INSTR" - VXI board 0, logical address 64

"VXI1::64::INSTR" - VXI board 1, logical address 64

"GPIB-VXI::64::INSTR" - GPIB-VXI board 0, logical address 64

"GPIB-VXI1::64::INSTR" - GPIB-VXI board 1, logical address 64

"ASRL2::INSTR" - COM port 2

"SampleInstr" - Logical name "SampleInstr"

"xyz432" - Logical Name or Driver Session "xyz432"

Default Value:  "USB0::0x0A69::0x0879::63200E000001::INSTR"

IDQuery

Variable Type       ViBoolean

Specify whether you want the instrument driver to perform an ID

Query.

Valid Range:

VI_TRUE  (1) - Perform ID Query (Default Value)

VI_FALSE (0) - Skip ID Query

When you set this parameter to VI_TRUE, the driver verifies that the

instrument you initialize is a type that this driver supports.

Circumstances can arise where it is undesirable to send an ID Query

command string to the instrument.  When you set this parameter to

VI_FALSE, the function initializes the instrument without performing

an ID Query.

resetDevice

Variable Type       ViBoolean

Specify whether you want the to reset the instrument during the

initialization procedure.

Valid Range:

VI_TRUE  (1) - Reset Device (Default Value)

VI_FALSE (0) - Don't Reset

instrumentHandle

Variable Type       ViSession (passed by reference)

Returns a ViSession handle that you use to identify the instrument in

all subsequent instrument driver function calls.

Notes:

(1) This function creates a new session each time you invoke it.

This is useful if you have multiple physical instances of the same

type of instrument.

(2) Avoid creating multiple concurrent sessions to the same physical

instrument.  Although you can create more than one IVI session for

the same resource, it is best not to do so.  A better approach is to

use the same IVI session in multiple execution threads.  You can use

functions Chr63000_LockSession and Chr63000_UnlockSession to protect

sections of code that require exclusive access to the resource.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_InitWithOptions

ViStatus Chr63000_InitWithOptions (ViRsrc resourceName,

ViBoolean IDQuery,

ViBoolean resetDevice,

ViString optionString,

ViPSession instrumentHandle);

Purpose

This function performs the following initialization actions:

- Creates a new IVI instrument driver and optionally sets the initial

state of the following session attributes:

CHR63000_ATTR_RANGE_CHECK

CHR63000_ATTR_QUERY_INSTRUMENT_STATUS

CHR63000_ATTR_CACHE

CHR63000_ATTR_SIMULATE

CHR63000_ATTR_RECORD_COERCIONS

- Opens a session to the specified device using the interface and address

you specify for the Resource Name parameter.

- If the ID Query parameter is set to VI_TRUE, this function queries the

instrument ID and checks that it is valid for this instrument driver.

- If the Reset parameter is set to VI_TRUE, this function resets the

instrument to a known state.

- Sends initialization commands to set the instrument to the state

necessary for the operation of the instrument driver.

- Returns a ViSession handle that you use to identify the instrument in

all subsequent instrument driver function calls.

Note:  This function creates a new session each time you invoke it.

Although you can open more than one IVI session for the same resource, it

is best not to do so.  You can use the same session in multiple program

threads.  You can use the Chr63000_LockSession and Chr63000_UnlockSession

functions to protect sections of code that require exclusive access to

the resource.

Content from the IVI specifications reproduced with permission from the

IVI Foundation.

The IVI Foundation and its member companies make no warranty of any kind

with regard to this material, including, but not limited to, the implied

warranties of merchantability and fitness for a particular purpose. The

IVI Foundation and its member companies shall not be liable for errors

contained herein or for incidental or consequential damages in connection

with the furnishing, performance, or use of this material.

Parameter List

resourceName

Variable Type       ViRsrc

Pass the resource name of the device to initialize.

You can also pass the name of a driver session or logical name that

you configure with the IVI Configuration utility.  The driver session

identifies a specific device and specifies the initial settings for

the session.  A logical Name identifies a particular driver session.

Refer to the following table below for the exact grammar to use for

this parameter.  Optional fields are shown in square brackets ([]).

Syntax

------------------------------------------------------

GPIB[board]::<primary address>[::secondary address]::INSTR

VXI[board]::<logical address>::INSTR

GPIB-VXI[board]::<logical address>::INSTR

ASRL<port>::INSTR

<LogicalName>

<Driver Session>

If you do not specify a value for an optional field, the following

values are used:

Optional Field - Value

------------------------------------------------------

board - 0

secondary address - none (31)

The following table contains example valid values for this parameter.

"Valid Value" - Description

------------------------------------------------------

"GPIB::22::INSTR" - GPIB board 0, primary address 22 no

secondary address

"GPIB::22::5::INSTR" - GPIB board 0, primary address 22

secondary address 5

"GPIB1::22::5::INSTR" - GPIB board 1, primary address 22

secondary address 5

"VXI::64::INSTR" - VXI board 0, logical address 64

"VXI1::64::INSTR" - VXI board 1, logical address 64

"GPIB-VXI::64::INSTR" - GPIB-VXI board 0, logical address 64

"GPIB-VXI1::64::INSTR" - GPIB-VXI board 1, logical address 64

"ASRL2::INSTR" - COM port 2

"SampleInstr" - Logical name "SampleInstr"

"xyz432" - Logical Name or Driver Session "xyz432"

Default Value:  "USB0::0x0A69::0x0879::63205E000001::INSTR"

IDQuery

Variable Type       ViBoolean

Specify whether you want the instrument driver to perform an ID

Query.

Valid Range:

VI_TRUE  (1) - Perform ID Query (Default Value)

VI_FALSE (0) - Skip ID Query

When you set this parameter to VI_TRUE, the driver verifies that the

instrument you initialize is a type that this driver supports.

Circumstances can arise where it is undesirable to send an ID Query

command string to the instrument.  When you set this parameter to

VI_FALSE, the function initializes the instrument without performing

an ID Query.

resetDevice

Variable Type       ViBoolean

Specify whether you want the to reset the instrument during the

initialization procedure.

Valid Range:

VI_TRUE  (1) - Reset Device (Default Value)

VI_FALSE (0) - Don't Reset

optionString

Variable Type       ViString

You can use this control to set the initial value of certain

attributes for the session.  The following table lists the attributes

and the name you use in this parameter to identify the attribute.

Name              Attribute Defined Constant

--------------------------------------------

RangeCheck        CHR63000_ATTR_RANGE_CHECK

QueryInstrStatus  CHR63000_ATTR_QUERY_INSTRUMENT_STATUS

Cache             CHR63000_ATTR_CACHE

Simulate          CHR63000_ATTR_SIMULATE

RecordCoercions   CHR63000_ATTR_RECORD_COERCIONS

The format of this string is, "AttributeName=Value" where

AttributeName is the name of the attribute and Value is the value to

which the attribute will be set.  To set multiple attributes,

separate their assignments with a comma.

If you pass NULL or an empty string for this parameter, the session

uses the default values for the attributes.   You can override the

default values by assigning a value explicitly in a string you pass

for this parameter.  You do not have to specify all of the attributes

and may leave any of them out.  If you do not specify one of the

attributes, its default value will be used.

The default values for the attributes are shown below:

Attribute Name     Default Value

----------------   -------------

RangeCheck         VI_TRUE

QueryInstrStatus   VI_FALSE

Cache              VI_TRUE

Simulate           VI_FALSE

RecordCoercions    VI_FALSE

The following are the valid values for ViBoolean attributes:

True:     1, True, or VI_TRUE

False:    0, False, or VI_FALSE

Default Value:

"Simulate=0,RangeCheck=1,QueryInstrStatus=1,Cache=1"

instrumentHandle

Variable Type       ViSession (passed by reference)

Returns a ViSession handle that you use to identify the instrument in

all subsequent instrument driver function calls.

Notes:

(1) This function creates a new session each time you invoke it.

This is useful if you have multiple physical instances of the same

type of instrument.

(2) Avoid creating multiple concurrent sessions to the same physical

instrument.  Although you can create more than one IVI session for

the same resource, it is best not to do so.  A better approach is to

use the same IVI session in multiple execution threads.  You can use

functions Chr63000_LockSession and Chr63000_UnlockSession to protect

sections of code that require exclusive access to the resource.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_InvalidateAllAttributes

ViStatus Chr63000_InvalidateAllAttributes (ViSession instrumentHandle);

Purpose

This function invalidates the cached values of all attributes for the

session.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_LoadSetup

ViStatus Chr63000_LoadSetup (ViSession instrumentHandle,

ViInt32 latchReset, ViInt32 latch,

ViReal64 von, ViReal64 voff,

ViInt32 shortKey, ViInt32 autoOn);

Purpose

Configure load setting

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

latchReset

Variable Type       ViInt32

Resets the Von signal.

Valid Values:

Off (0) - Off (Default Value)

On  (1) - On

latch

Variable Type       ViInt32

Set the action type of Von.

Valid Values:

Off (0) - Off (Default Value)

On  (1) - On

von

Variable Type       ViReal64

Set the voltage of sink current on.

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          Volt

Default Value:0

voff

Variable Type       ViReal64

Set the voltage of sink current off.

Valid Range:

Refer to the user manual for the valid range of the model.

Unit          Volt

Default Value:0

shortKey

Variable Type       ViInt32

Set the action enable or disable of SHORT key.

Valid Values:  Disable(0), Toggle(1)

Default Value: Disable(0)

autoOn

Variable Type       ViInt32

Set the load module to perform auto load on during power-on.

Valid Values:

Off(0) (Default Value)

On(1)

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_LockSession

ViStatus Chr63000_LockSession (ViSession instrumentHandle,

ViPBoolean callerHasLock);

Purpose

This function obtains a multithread lock on the instrument session.

Before it does so, it waits until all other execution threads have

released their locks on the instrument session.

Other threads might have obtained a lock on this session in the following

ways:

- The user's application called Chr63000_LockSession.

- A call to the instrument driver locked the session.

- A call to the IVI engine locked the session.

After your call to Chr63000_LockSession returns successfully, no other

threads can access the instrument session until you call

Chr63000_UnlockSession.

Use Chr63000_LockSession and Chr63000_UnlockSession around a sequence of

calls to instrument driver functions if you require that the instrument

retain its settings through the end of the sequence.

You can safely make nested calls to Chr63000_LockSession within the same

thread.  To completely unlock the session, you must balance each call to

Chr63000_LockSession with a call to Chr63000_UnlockSession.  If, however,

you use the Caller Has Lock parameter in all calls to

Chr63000_LockSession and Chr63000_UnlockSession within a function, the

IVI Library locks the session only once within the function regardless of

the number of calls you make to Chr63000_LockSession.  This allows you to

call Chr63000_UnlockSession just once at the end of the function.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

callerHasLock

Variable Type       ViBoolean (passed by reference)

This parameter serves as a convenience.  If you do not want to use

this parameter, pass VI_NULL.

Use this parameter in complex functions to keep track of whether you

obtain a lock and therefore need to unlock the session.  Pass the

address of a local ViBoolean variable.  In the declaration of the

local variable, initialize it to VI_FALSE.  Pass the address of the

same local variable to any other calls you make to

Chr63000_LockSession or Chr63000_UnlockSession in the same function.

The parameter is an input/output parameter.  Chr63000_LockSession and

Chr63000_UnlockSession each inspect the current value and take the

following actions:

- If the value is VI_TRUE, Chr63000_LockSession does not lock the

session again.  If the value is VI_FALSE, Chr63000_LockSession

obtains the lock and sets the value of the parameter to VI_TRUE.

- If the value is VI_FALSE, Chr63000_UnlockSession does not attempt

to unlock the session.  If the value is VI_TRUE,

Chr63000_UnlockSession releases the lock and sets the value of the

parameter to VI_FALSE.

Thus, you can, call Chr63000_UnlockSession at the end of your

function without worrying about whether you actually have the lock.

Example:

ViStatus TestFunc (ViSession vi, ViInt32 flags)

{

ViStatus error = VI_SUCCESS;

ViBoolean haveLock = VI_FALSE;

if (flags & BIT_1)

{

viCheckErr( Chr63000_LockSession(vi, &haveLock));

viCheckErr( TakeAction1(vi));

if (flags & BIT_2)

{

viCheckErr( Chr63000_UnlockSession(vi, &haveLock));

viCheckErr( TakeAction2(vi));

viCheckErr( Chr63000_LockSession(vi, &haveLock);

}

if (flags & BIT_3)

viCheckErr( TakeAction3(vi));

}

Error:

/*

At this point, you cannot really be sure that

you have the lock.  Fortunately, the haveLock

variable takes care of that for you.

*/

Chr63000_UnlockSession(vi, &haveLock);

return error;

}

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_Measure

ViStatus Chr63000_Measure (ViSession instrumentHandle,

ViInt32 inputPort, ViInt32 item,

ViPReal64 response);

Purpose

Returns the real time value measured at the load module input.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

inputPort

Variable Type       ViInt32

Selects the input port of the electronic load to measure voltage.

Valid Values:

0 - Load (Default Value)

1 - UUT

item

Variable Type       ViInt32

Select Item to query.

Valid Values:

0 - Current (Default Value)

1 - Power

2 - Voltage

response

Variable Type       ViReal64 (passed by reference)

Response of instrument.

Ampere/Watt/Volt

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_MeasurementSetting

ViStatus Chr63000_MeasurementSetting (ViSession instrumentHandle,

ViInt32 sign,

ViReal64 windowTime);

Purpose

Configure sign and window time

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

sign

Variable Type       ViInt32

Set the sign of voltage measurement to Plus/Minus.

Valid Values:

Off (0) - Plus (Default Value)

On  (1) - Minus

windowTime

Variable Type       ViReal64

Set the time of measure over which the window calculation is to be

performed

Valid Range:

Range         0.02 ~ 2.00

Resolution    20m

Unit          Second

Default Value: 1.00

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_OCPandOPP

ViStatus Chr63000_OCPandOPP (ViSession instrumentHandle,

ViInt32 ringStateOCP, ViInt32 ringStateOPP,

ViReal64 inputPointOCP,

ViReal64 inputPointOPP,

ViReal64 inputDelayOCP,

ViReal64 inputDelayOPP);

Purpose

Configure OCP and OPP.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

ringStateOCP

Variable Type       ViInt32

Set the action to enable or disable the user-defined OCP function.

Valid Values:

0 - Disable

1 - Enable

Default Value : 0

ringStateOPP

Variable Type       ViInt32

Set the action to enable or disable the user-defined OPP function.

Valid Values:

0 - Disable

1 - Enable

Default Value : 0

inputPointOCP

Variable Type       ViReal64

Set the current limit for the user-defined OCP function.

Valid Range:

Refer to respective specification for valid value range.

Unit          Ampere

inputPointOPP

Variable Type       ViReal64

Set the power limit for the user-defined OPP function.

Valid Range:

Refer to respective specification for valid value range.

Unit          Watt

inputDelayOCP

Variable Type       ViReal64

Set the delay time for the user-defined OCP function.

Valid Range:

<NRf+>, 1ms ~ 20s, Resolution = 1ms

Unit          Second

inputDelayOPP

Variable Type       ViReal64

Set the delay time for the user-defined OPP function.

Valid Range:

<NRf+>, 1ms ~ 20s, Resolution = 1ms

Unit          Second

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_OperatingMode

ViStatus Chr63000_OperatingMode (ViSession instrumentHandle,

ViInt32 mode, ViInt32 level);

Purpose

This command sets the operational mode for the electronic load.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

mode

Variable Type       ViInt32

Select the active mode

Valid Values:

CHR63000_VAL_MOD_CCL :CC

CHR63000_VAL_MOD_CRL :CR

CHR63000_VAL_MOD_CVL :CV

CHR63000_VAL_MOD_CPL :CP

CHR63000_VAL_MOD_CCDL:CCD

CHR63000_VAL_MOD_PROG:PROG

Default Value : CC(CHR63000_VAL_MOD_CCL)

level

Variable Type       ViInt32

Set the mode level.

CVCC,CRCC,CVCR,Auto and Program mode don't have to set this

parameter.

Valid values: 0 to 2

0 - Low

1 - Middle

2 - High

Default value : 0

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ReadInstrData

ViStatus Chr63000_ReadInstrData (ViSession instrumentHandle,

ViInt32 numberOfBytesToRead,

ViChar _VI_FAR readBuffer[],

ViPInt32 numBytesRead);

Purpose

This function reads data from the instrument.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

numberOfBytesToRead

Variable Type       ViInt32

Pass the maximum number of bytes to read from the instruments.

Valid Range:  0 to the number of elements in the Read Buffer.

Default Value:  512

readBuffer

Variable Type       ViChar[]

After this function executes, this parameter contains the data that

was read from the instrument.

numBytesRead

Variable Type       ViInt32 (passed by reference)

Returns the number of bytes actually read from the instrument and

stored in the Read Buffer.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_RemainSequence

ViStatus Chr63000_RemainSequence (ViSession instrumentHandle,

ViPInt32 result);

Purpose

This command returns the remains sequence of unused.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

result

Variable Type       ViInt32 (passed by reference)

This command returns the remains sequence of unused

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_reset

ViStatus Chr63000_reset (ViSession instrumentHandle);

Purpose

This function resets the instrument to a known state and sends

initialization commands to the instrument.  The initialization commands

set instrument settings such as Headers Off, Short Command form, and Data

Transfer Binary to the state necessary for the operation of the

instrument driver.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ResetInterchangeCheck

ViStatus Chr63000_ResetInterchangeCheck (ViSession instrumentHandle);

Purpose

When developing a complex test system that consists of multiple test

modules, it is generally a good idea to design the test modules so that

they can run in any order.  To do so requires ensuring that each test

module completely configures the state of each instrument it uses.  If a

particular test module does not completely configure the state of an

instrument, the state of the instrument depends on the configuration from

a previously executed test module.  If you execute the test modules in a

different order, the behavior of the instrument and therefore the entire

test module is likely to change.  This change in behavior is generally

instrument specific and represents an interchangeability problem.

You can use this function to test for such cases.  After you call this

function, the interchangeability checking algorithms in the specific

driver ignore all previous configuration operations.  By calling this

function at the beginning of a test module, you can determine whether the

test module has dependencies on the operation of previously executed test

modules.

This function does not clear the interchangeability warnings from the

list of previously recorded interchangeability warnings.  If you want to

guarantee that the Chr63000_GetNextInterchangeWarning function only

returns those interchangeability warnings that are generated after

calling this function, you must clear the list of interchangeability

warnings.  You can clear the interchangeability warnings list by

repeatedly calling the Chr63000_GetNextInterchangeWarning function until

no more interchangeability warnings are returned.  If you are not

interested in the content of those warnings, you can call the

Chr63000_ClearInterchangeWarnings function.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_ResetWithDefaults

ViStatus Chr63000_ResetWithDefaults (ViSession instrumentHandle);

Purpose

This function resets the instrument and applies initial user specified

settings from the Logical Name which was used to initialize the session.

If the session was created without a Logical Name, this function is

equivalent to the Chr63000_reset function.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_revision_query

ViStatus Chr63000_revision_query (ViSession instrumentHandle,

ViChar _VI_FAR instrumentDriverRevision[],

ViChar _VI_FAR firmwareRevision[]);

Purpose

This function returns the revision numbers of the instrument driver and

instrument firmware.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

instrumentDriverRevision

Variable Type       ViChar[]

Returns the instrument driver software revision numbers in the form

of a string.

You must pass a ViChar array with at least 256 bytes.

firmwareRevision

Variable Type       ViChar[]

Returns the instrument firmware revision numbers in the form of a

string.

You must pass a ViChar array with at least 256 bytes.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_SaveProgram

ViStatus Chr63000_SaveProgram (ViSession instrumentHandle);

Purpose

Send Program setting value to device, and save program.

If you want to use PROGRAM Mode, you need Configure Program Type and

Program Data Class.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_SelectProgram

ViStatus Chr63000_SelectProgram (ViSession instrumentHandle,

ViInt32 programNumber);

Purpose

The program number which to be executed.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

programNumber

Variable Type       ViInt32

Selects a program to be set

Valid Range:

Range         1-10

Resolution    1

Unit          None

Default Value: 1

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_SetAttributeViBoolean

ViStatus Chr63000_SetAttributeViBoolean (ViSession instrumentHandle,

ViChar _VI_FAR channelName[],

ViAttr attributeID,

ViBoolean attributeValue);

Purpose

This function sets the value of a ViBoolean attribute.

This is a low-level function that you can use to set the values of

instrument-specific attributes and inherent IVI attributes.  If the

attribute represents an instrument state, this function performs

instrument I/O in the following cases:

- State caching is disabled for the entire session or for the particular

attribute.

- State caching is enabled and the currently cached value is invalid or

is different than the value you specify.

This instrument driver contains high-level functions that set most of the

instrument attributes.  It is best to use the high-level driver functions

as much as possible.  They handle order dependencies and multithread

locking for you.  In addition, they perform status checking only after

setting all of the attributes.  In contrast, when you set multiple

attributes using the SetAttribute functions, the functions check the

instrument status after each call.

Also, when state caching is enabled, the high-level functions that

configure multiple attributes perform instrument I/O only for the

attributes whose value you change.  Thus, you can safely call the

high-level functions without the penalty of redundant instrument I/O.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViChar[]

If the attribute is channel-based, this parameter specifies the name

of the channel on which to set the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViBoolean

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViBoolean are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViBoolean

Pass the value to which you want to set the attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_SetAttributeViInt32

ViStatus Chr63000_SetAttributeViInt32 (ViSession instrumentHandle,

ViChar _VI_FAR channelName[],

ViAttr attributeID,

ViInt32 attributeValue);

Purpose

This function sets the value of a ViInt32 attribute.

This is a low-level function that you can use to set the values of

instrument-specific attributes and inherent IVI attributes.  If the

attribute represents an instrument state, this function performs

instrument I/O in the following cases:

State caching is disabled for the entire session or for the particular

attribute.

State caching is enabled and the currently cached value is invalid or is

different than the value you specify.

This instrument driver contains high-level functions that set most of the

instrument attributes.  It is best to use the high-level driver functions

as much as possible.  They handle order dependencies and multithread

locking for you.  In addition, they perform status checking only after

setting all of the attributes.  In contrast, when you set multiple

attributes using the SetAttribute functions, the functions check the

instrument status after each call.

Also, when state caching is enabled, the high-level functions that

configure multiple attributes perform instrument I/O only for the

attributes whose value you change.  Thus, you can safely call the

high-level functions without the penalty of redundant instrument I/O.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViChar[]

If the attribute is channel-based, this parameter specifies the name

of the channel on which to set the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViInt32 type.

If you choose to see all IVI attributes, the data types appear

to the right of the attribute names in the list box.

Attributes with data types other than ViInt32 are dim.  If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViInt32

Pass the value to which you want to set the attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_SetAttributeViReal64

ViStatus Chr63000_SetAttributeViReal64 (ViSession instrumentHandle,

ViChar _VI_FAR channelName[],

ViAttr attributeID,

ViReal64 attributeValue);

Purpose

This function sets the value of a ViReal64 attribute.

This is a low-level function that you can use to set the values of

instrument-specific attributes and inherent IVI attributes.  If the

attribute represents an instrument state, this function performs

instrument I/O in the following cases:

- State caching is disabled for the entire session or for the particular

attribute.

- State caching is enabled and the currently cached value is invalid or

is different than the value you specify.

This instrument driver contains high-level functions that set most of the

instrument attributes.  It is best to use the high-level driver functions

as much as possible.  They handle order dependencies and multithread

locking for you.  In addition, they perform status checking only after

setting all of the attributes.  In contrast, when you set multiple

attributes using the SetAttribute functions, the functions check the

instrument status after each call.

Also, when state caching is enabled, the high-level functions that

configure multiple attributes perform instrument I/O only for the

attributes whose value you change.  Thus, you can safely call the

high-level functions without the penalty of redundant instrument I/O.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViChar[]

If the attribute is channel-based, this parameter specifies the name

of the channel on which to set the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViReal64

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViReal64 are dim.  If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViReal64

Pass the value to which you want to set the attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_SetAttributeViSession

ViStatus Chr63000_SetAttributeViSession (ViSession instrumentHandle,

ViChar _VI_FAR channelName[],

ViAttr attributeID,

ViSession attributeValue);

Purpose

This function sets the value of a ViSession attribute.

This is a low-level function that you can use to set the values of

instrument-specific attributes and inherent IVI attributes.  If the

attribute represents an instrument state, this function performs

instrument I/O in the following cases:

- State caching is disabled for the entire session or for the particular

attribute.

- State caching is enabled and the currently cached value is invalid or

is different than the value you specify.

This instrument driver contains high-level functions that set most of the

instrument attributes.  It is best to use the high-level driver functions

as much as possible.  They handle order dependencies and multithread

locking for you.  In addition, they perform status checking only after

setting all of the attributes.  In contrast, when you set multiple

attributes using the SetAttribute functions, the functions check the

instrument status after each call.

Also, when state caching is enabled, the high-level functions that

configure multiple attributes perform instrument I/O only for the

attributes whose value you change.  Thus, you can safely call the

high-level functions without the penalty of redundant instrument I/O.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViChar[]

If the attribute is channel-based, this parameter specifies the name

of the channel on which to set the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViSession

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViSession are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViSession

Pass the value to which you want to set the attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_SetAttributeViString

ViStatus Chr63000_SetAttributeViString (ViSession instrumentHandle,

ViChar _VI_FAR channelName[],

ViAttr attributeID,

ViChar _VI_FAR attributeValue[]);

Purpose

This function sets the value of a ViString attribute.

This is a low-level function that you can use to set the values of

instrument-specific attributes and inherent IVI attributes.  If the

attribute represents an instrument state, this function performs

instrument I/O in the following cases:

- State caching is disabled for the entire session or for the particular

attribute.

- State caching is enabled and the currently cached value is invalid or

is different than the value you specify.

This instrument driver contains high-level functions that set most of the

instrument attributes.  It is best to use the high-level driver functions

as much as possible.  They handle order dependencies and multithread

locking for you.  In addition, they perform status checking only after

setting all of the attributes.  In contrast, when you set multiple

attributes using the SetAttribute functions, the functions check the

instrument status after each call.

Also, when state caching is enabled, the high-level functions that

configure multiple attributes perform instrument I/O only for the

attributes whose value you change.  Thus, you can safely call the

high-level functions without the penalty of redundant instrument I/O.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

channelName

Variable Type       ViChar[]

If the attribute is channel-based, this parameter specifies the name

of the channel on which to set the value of the attribute. If the

attribute is not channel-based, then pass VI_NULL or an empty string.

Valid Channel Names:  1

Default Value:  ""

attributeID

Variable Type       ViAttr

Pass the ID of an attribute.

From the function panel window, you can use this control as follows.

- Click on the control or press <ENTER>, <spacebar>, or

<ctrl-down arrow>, to display a dialog box containing a

hierarchical list of the available attributes.  Attributes

whose value cannot be set are dim.  Help text is shown for

each attribute.  Select an attribute by double-clicking on it

or by selecting it and then pressing <ENTER>.

Read-only attributes appear dim in the list box.  If you

select a read-only attribute, an error message appears.

A ring control at the top of the dialog box allows you to see

all IVI attributes or only the attributes of the ViString

type.  If you choose to see all IVI attributes, the data types

appear to the right of the attribute names in the list box.

Attributes with data types other than ViString are dim. If

you select an attribute data type that is dim, LabWindows/CVI

transfers you to the function panel for the corresponding

function that is consistent with the data type.

- If you want to enter a variable name, press <CTRL-T> to change

this ring control to a manual input box.

- If the attribute in this ring control has named constants as

valid values, you can view the constants by moving to the

Attribute Value control and pressing <ENTER>.

attributeValue

Variable Type       ViChar[]

Pass the value to which you want to set the attribute.

From the function panel window, you can use this control as follows.

- If the attribute currently showing in the Attribute ID ring

control has constants as valid values, you can view a list of

the constants by pressing <ENTER> on this control.  Select a

value by double-clicking on it or by selecting it and then

pressing <ENTER>.

Note:  Some of the values might not be valid depending on the

current settings of the instrument session.

Default Value: none

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_SpecificationJudgment

ViStatus Chr63000_SpecificationJudgment (ViSession instrumentHandle,

ViInt32 item,

ViChar _VI_FAR testResult[]);

Purpose

Request GO-NG result reference to specifications.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

item

Variable Type       ViInt32

Select Item of test.

Valid Values:

0 - All (Default Value)

1 - Current

2 - Power

3 - Voltage

testResult

Variable Type       ViChar[]

Result of specification test.

IDLE, GO, NG

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_StartSpecificationTest

ViStatus Chr63000_StartSpecificationTest (ViSession instrumentHandle,

ViBoolean enable,

ViInt32 unit);

Purpose

Start or close the specification test.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

enable

Variable Type       ViBoolean

Enable the specification test.

Valid Values:

Off(0) - Default Value

On(1)

unit

Variable Type       ViInt32

Select the specific entry mode.

Valid Values:

Percent(0) - Default Value

Value(1)

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_SystemSetup

ViStatus Chr63000_SystemSetup (ViSession instrumentHandle,

ViInt32 sound, ViInt32 enterKey);

Purpose

Setting Misc of hardware.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

sound

Variable Type       ViInt32

Set the buzzer

Valid Values:

On  (1) - On (Default Value)

Off (0) - Off

enterKey

Variable Type       ViInt32

Set the action type of ENTER key.

Valid Values:

Next(0) - Default Value

Fixed(1)

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_Timing

ViStatus Chr63000_Timing (ViSession instrumentHandle, ViInt32 ringState,

ViInt32 ringMode, ViReal64 inputVstart,

ViReal64 inputVend, ViReal64 inputTout);

Purpose

Configure Timing

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

ringState

Variable Type       ViInt32

Set the action enable or disable of TIMING function.

Valid Values:

0 - Disable

1 - Enable

Default Value : 0

ringMode

Variable Type       ViInt32

Set the action rise or fall of TIMING function.

Valid Values:

0 - Rise

1 - Fall

Default Value : 0

inputVstart

Variable Type       ViReal64

Set the trigger start voltage in TIMING function.

Valid Range:

Refer to respective specification for valid value range.

Unit          Volt

inputVend

Variable Type       ViReal64

Set the trigger end voltage in TIMING function.

Valid Range:

Refer to respective specification for valid value range.

Unit          Volt

inputTout

Variable Type       ViReal64

Set the timeout for TINING function.

Valid Range:

<NRf+>, 0s ~ 100000s, Resolution = 1S

Unit          Second

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_UnlockSession

ViStatus Chr63000_UnlockSession (ViSession instrumentHandle,

ViPBoolean callerHasLock);

Purpose

This function releases a lock that you acquired on an instrument session

using Chr63000_LockSession.  Refer to Chr63000_LockSession for additional

information on session locks.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

callerHasLock

Variable Type       ViBoolean (passed by reference)

This parameter serves as a convenience.  If you do not want to use

this parameter, pass VI_NULL.

Use this parameter in complex functions to keep track of whether you

obtain a lock and therefore need to unlock the session.

Pass the address of a local ViBoolean variable.  In the declaration

of the local variable, initialize it to VI_FALSE.  Pass the address

of the same local variable to any other calls you make to

Chr63000_LockSession or Chr63000_UnlockSession in the same function.

The parameter is an input/output parameter.  Chr63000_LockSession and

Chr63000_UnlockSession each inspect the current value and take the

following actions:

- If the value is VI_TRUE, Chr63000_LockSession does not lock the

session again.  If the value is VI_FALSE, Chr63000_LockSession

obtains the lock and sets the value of the parameter to VI_TRUE.

- If the value is VI_FALSE, Chr63000_UnlockSession does not attempt

to unlock the session.  If the value is VI_TRUE,

Chr63000_UnlockSession releases the lock and sets the value of the

parameter to VI_FALSE.

Thus, you can, call Chr63000_UnlockSession at the end of your

function without worrying about whether you actually have the lock.

Example:

ViStatus TestFunc (ViSession vi, ViInt32 flags)

{

ViStatus error = VI_SUCCESS;

ViBoolean haveLock = VI_FALSE;

if (flags & BIT_1)

{

viCheckErr( Chr63000_LockSession(vi, &haveLock));

viCheckErr( TakeAction1(vi));

if (flags & BIT_2)

{

viCheckErr( Chr63000_UnlockSession(vi, &haveLock));

viCheckErr( TakeAction2(vi));

viCheckErr( Chr63000_LockSession(vi, &haveLock);

}

if (flags & BIT_3)

viCheckErr( TakeAction3(vi));

}

Error:

/*

At this point, you cannot really be sure that

you have the lock.  Fortunately, the haveLock

variable takes care of that for you.

*/

Chr63000_UnlockSession(vi, &haveLock);

return error;

}

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Chr63000_WriteInstrData

ViStatus Chr63000_WriteInstrData (ViSession instrumentHandle,

ViChar _VI_FAR writeBuffer[]);

Purpose

This function writes a user-specified string to the instrument.

Note:  This function bypasses IVI attribute state caching.  Therefore,

when you call this function, the cached values for all attributes will be

invalidated.

Parameter List

instrumentHandle

Variable Type       ViSession

The ViSession handle that you obtain from the Chr63000_init or

Chr63000_InitWithOptions function.  The handle identifies a

particular instrument session.

Default Value:  None

writeBuffer

Variable Type       ViChar[]

Pass the string to be written to the instrument.

Return Value

Returns the status code of this operation.  The status code  either

indicates success or describes an error or warning condition.  You

examine the status code from each call to an instrument driver

function to determine if an error occurred.

To obtain a text description of the status code, call the

Chr63000_error_message function.  To obtain additional information

about the error condition, call the Chr63000_GetError function.  To

clear the error information from the driver, call the

Chr63000_ClearError function.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

This instrument driver also returns errors and warnings defined by

other sources.  The following table defines the ranges of additional

status codes that this driver can return.  The table lists the

different include files that contain the defined constants for the

particular status codes:

Numeric Range (in Hex)   Status Code Types

-------------------------------------------------

3FFA0000 to 3FFA1FFF     IVI      Warnings

3FFF0000 to 3FFFFFFF     VISA     Warnings

3FFC0000 to 3FFCFFFF     VXIPnP   Driver Warnings

BFFA0000 to BFFA1FFF     IVI      Errors

BFFF0000 to BFFFFFFF     VISA     Errors

BFFC0000 to BFFCFFFF     VXIPnP   Driver Errors

________________________________________________________________________________

Attribute Information for the Following Functions:

Chr63000_SetAttributeViInt32

Chr63000_GetAttributeViInt32

Chr63000_CheckAttributeViInt32

Chr63000_SetAttributeViInt64

Chr63000_GetAttributeViInt64

Chr63000_CheckAttributeViInt64

Chr63000_SetAttributeViReal64

Chr63000_GetAttributeViReal64

Chr63000_CheckAttributeViReal64

Chr63000_SetAttributeViSession

Chr63000_GetAttributeViSession

Chr63000_CheckAttributeViSession

Chr63000_SetAttributeViBoolean

Chr63000_GetAttributeViBoolean

Chr63000_CheckAttributeViBoolean

Chr63000_SetAttributeViString

Chr63000_GetAttributeViString

Chr63000_CheckAttributeViString

Inherent IVI Attributes

User Options

Range Check                              CHR63000_ATTR_RANGE_CHECK

Query Instrument Status                  CHR63000_ATTR_QUERY_INSTRUMENT_STATUS

Cache                                    CHR63000_ATTR_CACHE

Simulate                                 CHR63000_ATTR_SIMULATE

Record Value Coercions                   CHR63000_ATTR_RECORD_COERCIONS

Interchange Check                        CHR63000_ATTR_INTERCHANGE_CHECK

Driver Identification

Description                              CHR63000_ATTR_SPECIFIC_DRIVER_DESCRIPTION

Driver Prefix                            CHR63000_ATTR_SPECIFIC_DRIVER_PREFIX

Driver Vendor                            CHR63000_ATTR_SPECIFIC_DRIVER_VENDOR

Revision                                 CHR63000_ATTR_SPECIFIC_DRIVER_REVISION

Class Specification Major Version        CHR63000_ATTR_SPECIFIC_DRIVER_CLASS_SPEC_MAJOR_VERSION

Class Specification Minor Version        CHR63000_ATTR_SPECIFIC_DRIVER_CLASS_SPEC_MINOR_VERSION

Driver Capabilities

Supported Instrument Models              CHR63000_ATTR_SUPPORTED_INSTRUMENT_MODELS

Class Group Capabilities                 CHR63000_ATTR_GROUP_CAPABILITIES

Instrument Identification

Manufacturer                             CHR63000_ATTR_INSTRUMENT_MANUFACTURER

Model                                    CHR63000_ATTR_INSTRUMENT_MODEL

Firmware Revision                        CHR63000_ATTR_INSTRUMENT_FIRMWARE_REVISION

Advanced Session Information

Logical Name                             CHR63000_ATTR_LOGICAL_NAME

I/O Resource Descriptor                  CHR63000_ATTR_IO_RESOURCE_DESCRIPTOR

Driver Setup                             CHR63000_ATTR_DRIVER_SETUP

Miscellaneous Attributes

ID Query Response                           CHR63000_ATTR_ID_QUERY_RESPONSE

Advance

Program

CHR63000_ATTR_PROG_NUM                   CHR63000_ATTR_PROG_NUM

CHR63000_ATTR_PROG_MODE                  CHR63000_ATTR_PROG_MODE

CHR63000_ATTR_PROG_STEP                  CHR63000_ATTR_PROG_STEP

CHR63000_ATTR_PROG_TIME                  CHR63000_ATTR_PROG_TIME

CHR63000_ATTR_PROG_SPEC                  CHR63000_ATTR_PROG_SPEC

Configure

CHR63000_ATTR_CONF_AUTO_ON                  CHR63000_ATTR_CONF_AUTO_ON

CHR63000_ATTR_CONF_DIO_IN1                  CHR63000_ATTR_CONF_DIO_IN1

CHR63000_ATTR_CONF_DIO_IN2                  CHR63000_ATTR_CONF_DIO_IN2

CHR63000_ATTR_CONF_DIO_OUT1                 CHR63000_ATTR_CONF_DIO_OUT1

CHR63000_ATTR_CONF_DIO_OUT2                 CHR63000_ATTR_CONF_DIO_OUT2

CHR63000_ATTR_CONF_ENT_KEY                  CHR63000_ATTR_CONF_ENT_KEY

CHR63000_ATTR_CONF_SHOR_KEY                 CHR63000_ATTR_CONF_SHOR_KEY

CHR63000_ATTR_CONF_SOUN                     CHR63000_ATTR_CONF_SOUN

CHR63000_ATTR_CONF_VOLT_LATC                CHR63000_ATTR_CONF_VOLT_LATC

CHR63000_ATTR_CONF_VOLT_OFF                 CHR63000_ATTR_CONF_VOLT_OFF

CHR63000_ATTR_CONF_VOLT_ON                  CHR63000_ATTR_CONF_VOLT_ON

CHR63000_ATTR_CONF_VOLT_SIGN                CHR63000_ATTR_CONF_VOLT_SIGN

CHR63000_ATTR_CONF_WIND                     CHR63000_ATTR_CONF_WIND

CHR63000_ATTR_DIO_OUT1                      CHR63000_ATTR_DIO_OUT1

CHR63000_ATTR_DIO_OUT2                      CHR63000_ATTR_DIO_OUT2

CHR63000_ATTR_CONF_OCP                      CHR63000_ATTR_CONF_OCP

CHR63000_ATTR_CONF_OCP_POINT                CHR63000_ATTR_CONF_OCP_POINT

CHR63000_ATTR_CONF_OCP_DELAY                CHR63000_ATTR_CONF_OCP_DELAY

CHR63000_ATTR_CONF_OPP                      CHR63000_ATTR_CONF_OPP

CHR63000_ATTR_CONF_OPP_POINT                CHR63000_ATTR_CONF_OPP_POINT

CHR63000_ATTR_CONF_OPP_DELAY                CHR63000_ATTR_CONF_OPP_DELAY

Current

CHR63000_ATTR_CURR_DYN_FALL                 CHR63000_ATTR_CURR_DYN_FALL

CHR63000_ATTR_CURR_DYN_L1                   CHR63000_ATTR_CURR_DYN_L1

CHR63000_ATTR_CURR_DYN_L2                   CHR63000_ATTR_CURR_DYN_L2

CHR63000_ATTR_CURR_DYN_REP                  CHR63000_ATTR_CURR_DYN_REP

CHR63000_ATTR_CURR_DYN_RISE                 CHR63000_ATTR_CURR_DYN_RISE

CHR63000_ATTR_CURR_DYN_T1                   CHR63000_ATTR_CURR_DYN_T1

CHR63000_ATTR_CURR_DYN_T2                   CHR63000_ATTR_CURR_DYN_T2

CHR63000_ATTR_CURR_DYN_VRNG                 CHR63000_ATTR_CURR_DYN_VRNG

CHR63000_ATTR_CURR_STAT_FALL                CHR63000_ATTR_CURR_STAT_FALL

CHR63000_ATTR_CURR_STAT_L1                  CHR63000_ATTR_CURR_STAT_L1

CHR63000_ATTR_CURR_STAT_RISE                CHR63000_ATTR_CURR_STAT_RISE

CHR63000_ATTR_CURR_STAT_VRNG                CHR63000_ATTR_CURR_STAT_VRNG

Timing

CHR63000_ATTR_TIMING                        CHR63000_ATTR_TIMING

CHR63000_ATTR_TIMING_TRIGGER_MODE           CHR63000_ATTR_TIMING_TRIGGER_MODE

CHR63000_ATTR_TIMING_TRIGGER_VSTART         CHR63000_ATTR_TIMING_TRIGGER_VSTART

CHR63000_ATTR_TIMING_TRIGGER_VEND           CHR63000_ATTR_TIMING_TRIGGER_VEND

CHR63000_ATTR_TIMING_TOUT                   CHR63000_ATTR_TIMING_TOUT

Digitizing

CHR63000_ATTR_DIG_SAMP_POIN                 CHR63000_ATTR_DIG_SAMP_POIN

CHR63000_ATTR_DIG_SAMP_TIME                 CHR63000_ATTR_DIG_SAMP_TIME

CHR63000_ATTR_DIG_TRIG                      CHR63000_ATTR_DIG_TRIG

CHR63000_ATTR_DIG_TRIG_POIN                 CHR63000_ATTR_DIG_TRIG_POIN

CHR63000_ATTR_DIG_TRIG_SOUR                 CHR63000_ATTR_DIG_TRIG_SOUR

CHR63000_ATTR_DIG_WAV_CAP                   CHR63000_ATTR_DIG_WAV_CAP

Load

CHR63000_ATTR_LOAD                          CHR63000_ATTR_LOAD

CHR63000_ATTR_LOAD_PROT                     CHR63000_ATTR_LOAD_PROT

CHR63000_ATTR_LOAD_PROT_CLE                 CHR63000_ATTR_LOAD_PROT_CLE

CHR63000_ATTR_LOAD_SHOR                     CHR63000_ATTR_LOAD_SHOR

Mode

CHR63000_ATTR_MODE                          CHR63000_ATTR_MODE

Power

CHR63000_ATTR_POW_STAT_FALL                 CHR63000_ATTR_POW_STAT_FALL

CHR63000_ATTR_POW_STAT_L1                   CHR63000_ATTR_POW_STAT_L1

CHR63000_ATTR_POW_STAT_RISE                 CHR63000_ATTR_POW_STAT_RISE

CHR63000_ATTR_POW_STAT_VRNG                 CHR63000_ATTR_POW_STAT_VRNG

Resistance

CHR63000_ATTR_RES_STAT_FALL                 CHR63000_ATTR_RES_STAT_FALL

CHR63000_ATTR_RES_STAT_IRNG                 CHR63000_ATTR_RES_STAT_IRNG

CHR63000_ATTR_RES_STAT_L1                   CHR63000_ATTR_RES_STAT_L1

CHR63000_ATTR_RES_STAT_RISE                 CHR63000_ATTR_RES_STAT_RISE

Specification

CHR63000_ATTR_SPEC_CURR_C                   CHR63000_ATTR_SPEC_CURR_C

CHR63000_ATTR_SPEC_CURR_H                   CHR63000_ATTR_SPEC_CURR_H

CHR63000_ATTR_SPEC_CURR_L                   CHR63000_ATTR_SPEC_CURR_L

CHR63000_ATTR_SPEC_POW_C                    CHR63000_ATTR_SPEC_POW_C

CHR63000_ATTR_SPEC_POW_H                    CHR63000_ATTR_SPEC_POW_H

CHR63000_ATTR_SPEC_POW_L                    CHR63000_ATTR_SPEC_POW_L

CHR63000_ATTR_SPEC_TEST                     CHR63000_ATTR_SPEC_TEST

CHR63000_ATTR_SPEC_UNIT                     CHR63000_ATTR_SPEC_UNIT

CHR63000_ATTR_SPEC_VOLT_C                   CHR63000_ATTR_SPEC_VOLT_C

CHR63000_ATTR_SPEC_VOLT_H                   CHR63000_ATTR_SPEC_VOLT_H

CHR63000_ATTR_SPEC_VOLT_L                   CHR63000_ATTR_SPEC_VOLT_L

Voltage

CHR63000_ATTR_VOLT_STAT_ILIM                CHR63000_ATTR_VOLT_STAT_ILIM

CHR63000_ATTR_VOLT_STAT_L1                  CHR63000_ATTR_VOLT_STAT_L1

CHR63000_ATTR_VOLT_STAT_RES                 CHR63000_ATTR_VOLT_STAT_RES

CHR63000_ATTR_CACHE

Data Type:     ViBoolean

Description:   Specifies whether to cache the value of attributes.  When

caching is enabled, the instrument driver keeps track of the current instrument

settings and avoids sending redundant commands to the instrument.  Thus, you

can significantly increase execution speed.

The instrument driver can choose always to cache or never to cache

particular attributes regardless of the setting of this attribute.

The default value is VI_TRUE.  Use the Chr63000_InitWithOptions function to

override this value.

CHR63000_ATTR_CONF_AUTO_ON

Data Type:     ViInt32

Description:   Set the load module to perform auto load on during power-on.

Values:

CHR63000_VAL_OFF                               0

OFF

CHR63000_VAL_ON                                1

ON

CHR63000_ATTR_CONF_DIO_IN1

Data Type:     ViInt32

Description:   Set the DI1 type the pin No.11 in System I/O Port.

Values:

CHR63000_VAL_NONE                              0

NONE

CHR63000_VAL_EXT                               1

EXTERNAL LOAD ON/OFF

CHR63000_VAL_REM                               2

REMOTE INHIBIT

CHR63000_ATTR_CONF_DIO_IN2

Data Type:     ViInt32

Description:   Set the DI2 type the pin No.12 in System I/O Port.

Values:

CHR63000_VAL_NONE                              0

NONE

CHR63000_VAL_EXT                               1

EXTERNAL LOAD ON/OFF

CHR63000_VAL_REM                               2

REMOTE INHIBIT

CHR63000_ATTR_CONF_DIO_OUT1

Data Type:     ViInt32

Description:   Set the DO1 type the pin No.8 in System I/O Port.

0GNONE

1GGONG TOTAL PASS-H

2GGONG TOTAL FAIL-L

3GOTP OVP OCP OPP REV-H

4GBUS CTRL. ACT-H

5GBUS CTRL. ACT-L

CHR63000_ATTR_CONF_DIO_OUT2

Data Type:     ViInt32

Description:   Set the DO2 type the pin No.9 in System I/O Port.

0GNONE

1GGONG TOTAL PASS-H

2GGONG TOTAL FAIL-L

3GOTP OVP OCP OPP REV-H

4GBUS CTRL. ACT-H

5GBUS CTRL. ACT-L

CHR63000_ATTR_CONF_ENT_KEY

Data Type:     ViInt32

Description:   Set the action type of ENTER key

Values:

CHR63000_VAL_NEXT                              0

NEXT

CHR63000_VAL_FIXED                             1

FIXED

CHR63000_ATTR_CONF_OCP

Data Type:     ViInt32

Description:   Set the action to enable or disable the user-defined OCP

function.

Values:

CHR63000_VAL_Disable                           0

Disable

CHR63000_VAL_Enable                            1

Enable

CHR63000_ATTR_CONF_OCP_DELAY

Data Type:     ViReal64

Description:   Set the delay time for the user-defined OCP function.

CHR63000_ATTR_CONF_OCP_POINT

Data Type:     ViReal64

Description:   Set the current limit for the user-defined OCP function.

CHR63000_ATTR_CONF_OPP

Data Type:     ViInt32

Description:   Set the action to enable or disable the user-defined OPP

function.

Values:

CHR63000_VAL_Disable                           0

Disable

CHR63000_VAL_Enable                            1

Enable

CHR63000_ATTR_CONF_OPP_DELAY

Data Type:     ViReal64

Description:   Set the delay time for the user-defined OPP function.

CHR63000_ATTR_CONF_OPP_POINT

Data Type:     ViReal64

Description:   Set the power limit for the user-defined OPP function.

CHR63000_ATTR_CONF_SHOR_KEY

Data Type:     ViInt32

Description:   Set the action enable or disable of SHORT key.

Values:

CHR63000_VAL_TOGGLE                            1

TOGGLE

CHR63000_VAL_DISABLE                           0

DISABLE

CHR63000_ATTR_CONF_SOUN

Data Type:     ViInt32

Description:   Set the buzzer on/off in Load.

Values:

CHR63000_VAL_OFF                               0

OFF

CHR63000_VAL_ON                                1

ON

CHR63000_ATTR_CONF_VOLT_LATC

Data Type:     ViInt32

Description:   the action type of Von.

Values:

CHR63000_VAL_OFF                               0

OFF

CHR63000_VAL_ON                                1

ON

CHR63000_ATTR_CONF_VOLT_OFF

Data Type:     ViReal64

Description:   voltage of sink current off.

CHR63000_ATTR_CONF_VOLT_ON

Data Type:     ViReal64

Description:   voltage of sink current on.

CHR63000_ATTR_CONF_VOLT_SIGN

Data Type:     ViInt32

Description:   Set the sign of voltage measurement to Plus/Minus.

Values:

CHR63000_VAL_PLUS                              0

PLUS

CHR63000_VAL_MINUS                             1

MINUS

CHR63000_ATTR_CONF_WIND

Data Type:     ViReal64

Description:   Set the time of measure over which the window calculation is to

be performed.

CHR63000_ATTR_CURR_DYN_FALL

Data Type:     ViReal64

Description:   Set the falling slew rate of current for constant current

dynamic mode.

CHR63000_ATTR_CURR_DYN_L1

Data Type:     ViReal64

Description:   Set the load current during T1 period for constant current

dynamic mode.

CHR63000_ATTR_CURR_DYN_L2

Data Type:     ViReal64

Description:   Set the load current during T2 period for constant current

dynamic mode.

CHR63000_ATTR_CURR_DYN_REP

Data Type:     ViInt32

Description:   Set the repeat count for constant current dynamic mode.

CHR63000_ATTR_CURR_DYN_RISE

Data Type:     ViReal64

Description:   Set the rising slew rate of current for constant current dynamic

mode.

CHR63000_ATTR_CURR_DYN_T1

Data Type:     ViReal64

Description:   Set duration parameter T1 for constant current dynamic mode.

CHR63000_ATTR_CURR_DYN_T2

Data Type:     ViReal64

Description:   Set duration parameter T2 for constant current dynamic mode.

CHR63000_ATTR_CURR_DYN_VRNG

Data Type:     ViInt32

Description:   Set the voltage measurement range in CCD mode.

Values:

CHR63000_VAL_LOW                               0

LOW

CHR63000_VAL_MIDDLE                            1

MIDDLE

CHR63000_VAL_HIGH                              2

HIGH

CHR63000_ATTR_CURR_STAT_FALL

Data Type:     ViReal64

Description:   Set the falling slew rate of current for constant current static

mode

CHR63000_ATTR_CURR_STAT_L1

Data Type:     ViReal64

Description:   Set the static load current for constant current static mode.

CHR63000_ATTR_CURR_STAT_RISE

Data Type:     ViReal64

Description:   Set the rising slew rate of current for constant current static

mode

CHR63000_ATTR_CURR_STAT_VRNG

Data Type:     ViInt32

Description:   Set the voltage measurement range in CC mode.

Values:

CHR63000_VAL_LOW                               0

LOW

CHR63000_VAL_MIDDLE                            1

MIDDLE

CHR63000_VAL_HIGH                              2

HIGH

CHR63000_ATTR_DIG_SAMP_POIN

Data Type:     ViInt32

Description:   Set the sampling points for digitizing function.

CHR63000_ATTR_DIG_SAMP_TIME

Data Type:     ViReal64

Description:   Set the sampling time for digitizing function.

CHR63000_ATTR_DIG_TRIG

Data Type:     ViInt32

Description:   Set the software trigger for digitizing function.

Values:

CHR63000_VAL_DIGI_TRIG_OFF                     0

OFF(0), ON(1) ,IDLE(101), PRE_TRIG(102), WAIT_TRIG(103),

POST_TRIG(104)

CHR63000_VAL_DIGI_TRIG_ON                      1

OFF(0), ON(1) ,IDLE(101), PRE_TRIG(102), WAIT_TRIG(103),

POST_TRIG(104)

CHR63000_VAL_DIGI_TRIG_IDLE                    101

OFF(0), ON(1) ,IDLE(101), PRE_TRIG(102), WAIT_TRIG(103),

POST_TRIG(104)

CHR63000_VAL_DIGI_TRIG_PRE_TRIG                102

OFF(0), ON(1) ,IDLE(101), PRE_TRIG(102), WAIT_TRIG(103),

POST_TRIG(104)

CHR63000_VAL_DIGI_TRIG_WAIT_TRIG               103

OFF(0), ON(1) ,IDLE(101), PRE_TRIG(102), WAIT_TRIG(103),

POST_TRIG(104)

CHR63000_VAL_DIGI_TRIG_POST_TRIG               104

OFF(0), ON(1) ,IDLE(101), PRE_TRIG(102), WAIT_TRIG(103),

POST_TRIG(104)

CHR63000_ATTR_DIG_TRIG_POIN

Data Type:     ViInt32

Description:   Set the trigger points for digitizing function.

CHR63000_ATTR_DIG_TRIG_SOUR

Data Type:     ViInt32

Description:   Set the trigger source for digitizing function.

Values:

CHR63000_VAL_DIGI_TRIG_SOUR_LOADON             0

LOADON(0), LOADOFF(1), TTL(2), BUS(3) | MANUAL(4)

CHR63000_VAL_DIGI_TRIG_SOUR_LOADOFF            1

LOADON(0), LOADOFF(1), TTL(2), BUS(3) | MANUAL(4)

CHR63000_VAL_DIGI_TRIG_SOUR_TTL                2

LOADON(0), LOADOFF(1), TTL(2), BUS(3) | MANUAL(4)

CHR63000_VAL_DIGI_TRIG_SOUR_BUS                3

LOADON(0), LOADOFF(1), TTL(2), BUS(3) | MANUAL(4)

CHR63000_VAL_DIGI_TRIG_SOUR_MANUAL             4

LOADON(0), LOADOFF(1), TTL(2), BUS(3) | MANUAL(4)

CHR63000_ATTR_DIG_WAV_CAP

Data Type:     ViString

Restrictions:  Not settable.

Description:   Start waveform data transmit from Module to Frame.

WAIT, OK, ERROR

CHR63000_ATTR_DIO_OUT1

Data Type:     ViInt32

Description:   It sets the system I/O port pin 8 DO1 status when the BUS CTRL.

mode is selected for DO1.

Values:

CHR63000_VAL_OFF                               0

OFF

CHR63000_VAL_ON                                1

ON

CHR63000_ATTR_DIO_OUT2

Data Type:     ViInt32

Description:   It sets the system I/O port pin 9 DO2 status when the BUS CTRL.

mode is selected for DO2.

Values:

CHR63000_VAL_OFF                               0

OFF

CHR63000_VAL_ON                                1

ON

CHR63000_ATTR_DRIVER_SETUP

Data Type:     ViString

Restrictions:  Not settable.

Description:   Some cases exist where you must specify instrument driver

options at initialization time.  An example of this is specifying a particular

instrument model from among a family of instruments that the driver supports.

This is useful when using simulation.  You can specify driver-specific options

through the DriverSetup keyword in the optionsString parameter to the

Chr63000_InitWithOptions function.  If you open an instrument using a logical

name, you can also specify the options through the IVI Configuration Utility.

The default value is an empty string.

CHR63000_ATTR_GROUP_CAPABILITIES

Data Type:     ViString

Restrictions:  Not settable.

Description:   A string that contains a comma-separated list of class-extension

groups that this driver implements.

CHR63000_ATTR_ID_QUERY_RESPONSE

Data Type:     ViString

Restrictions:  Not settable.

Description:   Returns the ID Query response string.  The instrument driver

gets the value of this attribute when you pass VI_TRUE for the ID Query

parameter to the Chr63000_init or Chr63000_InitWithOptions function.

CHR63000_ATTR_INSTRUMENT_FIRMWARE_REVISION

Data Type:     ViString

Restrictions:  Not settable.

Description:   A string that contains the firmware revision information for the

instrument you are currently using.

CHR63000_ATTR_INSTRUMENT_MANUFACTURER

Data Type:     ViString

Restrictions:  Not settable.

Description:   A string that contains the name of the instrument manufacturer

you are currently using.

CHR63000_ATTR_INSTRUMENT_MODEL

Data Type:     ViString

Restrictions:  Not settable.

Description:   A string that contains the model number or name of the

instrument that you are currently using.

CHR63000_ATTR_INTERCHANGE_CHECK

Data Type:     ViBoolean

Description:   Specifies whether to perform interchangeability checking and

retrieve interchangeability warnings.

The default value is VI_FALSE.

Interchangeability warnings indicate that using your application with a

different instrument might cause different behavior.  You call Chr63000 Get

Next Interchange Warning to extract interchange warnings.  Call the

Chr63000_ClearInterchangeWarnings function to clear the list of

interchangeability warnings without reading them.

Interchangeability checking logs a warning for each attribute you have not set

that affects the behavior of the instrument.

CHR63000_ATTR_IO_RESOURCE_DESCRIPTOR

Data Type:     ViString

Restrictions:  Not settable.

Description:   Indicates the resource descriptor the driver uses to identify

the physical device.

If you initialize the driver with a logical name, this attribute contains

the resource descriptor that corresponds to the entry in the IVI Configuration

utility.

If you initialize the instrument driver with the resource descriptor, this

attribute contains that value.

CHR63000_ATTR_LOAD

Data Type:     ViInt32

Description:   The LOAD command makes the electronic load active/on or

inactive/off.

Values:

CHR63000_VAL_OFF                               0

OFF

CHR63000_VAL_ON                                1

ON

CHR63000_ATTR_LOAD_PROT

Data Type:     ViInt32

Description:   This command returns the status of electronic load.

CHR63000_ATTR_LOAD_PROT_CLE

Data Type:     ViBoolean

Description:   resets the status of electronic load.

CHR63000_ATTR_LOAD_SHOR

Data Type:     ViInt32

Description:   Activate or inactivate short-circuited simulation.

Values:

CHR63000_VAL_OFF                               0

OFF

CHR63000_VAL_ON                                1

ON

CHR63000_ATTR_LOGICAL_NAME

Data Type:     ViString

Restrictions:  Not settable.

Description:   A string containing the logical name you specified when opening

the current IVI session.

You may pass a logical name to the Chr63000_init or

Chr63000_InitWithOptions functions.  The IVI Configuration utility must contain

an entry for the logical name.  The logical name entry refers to a virtual

instrument section in the IVI Configuration file.  The virtual instrument

section specifies a physical device and initial user options.

CHR63000_ATTR_MODE

Data Type:     ViInt32

Description:   operational mode for the electronic load.

Values:

CHR63000_VAL_MOD_CCL                           1

constant current

CHR63000_VAL_MOD_CCM                           2

constant current

CHR63000_VAL_MOD_CCH                           3

constant current

CHR63000_VAL_MOD_CRL                           4

constant resistance

CHR63000_VAL_MOD_CRM                           5

constant resistance

CHR63000_VAL_MOD_CRH                           6

constant resistance

CHR63000_VAL_MOD_CVL                           7

constant voltage

CHR63000_VAL_MOD_CVM                           8

constant voltage

CHR63000_VAL_MOD_CVH                           9

constant voltage

CHR63000_VAL_MOD_CPL                           10

constant power

CHR63000_VAL_MOD_CPM                           11

constant power

CHR63000_VAL_MOD_CPH                           12

constant power

CHR63000_VAL_MOD_CCDL                          13

constant current dynamic

CHR63000_VAL_MOD_CCDM                          14

constant current dynamic

CHR63000_VAL_MOD_CCDH                          15

constant current dynamic

CHR63000_VAL_MOD_PROG                          16

program

CHR63000_ATTR_POW_STAT_FALL

Data Type:     ViReal64

Description:   Set the falling slew rate of current for constant power mode.

CHR63000_ATTR_POW_STAT_L1

Data Type:     ViReal64

Description:   Set the static load power for constant power mode.

CHR63000_ATTR_POW_STAT_RISE

Data Type:     ViReal64

Description:   Set the rising slew rate of current for constant power mode.

CHR63000_ATTR_POW_STAT_VRNG

Data Type:     ViInt32

Description:   Set the voltage measurement range in constant power mode.

Values:

CHR63000_VAL_LOW                               0

LOW

CHR63000_VAL_MIDDLE                            1

MIDDLE

CHR63000_VAL_HIGH                              2

HIGH

CHR63000_ATTR_PROG_MODE

Data Type:     ViString

Description:   Set Program Value

CHR63000_ATTR_PROG_NUM

Data Type:     ViInt32

Description:   Active program index.

CHR63000_ATTR_PROG_SPEC

Data Type:     ViString

Description:   Set Program Value

CHR63000_ATTR_PROG_STEP

Data Type:     ViString

Description:   Set Program Value

CHR63000_ATTR_PROG_TIME

Data Type:     ViString

Description:   Set Program Value

CHR63000_ATTR_QUERY_INSTRUMENT_STATUS

Data Type:     ViBoolean

Description:   Specifies whether the instrument driver queries the instrument

status after each operation.  Querying the instrument status is very useful for

debugging.  After you validate your program, you can set this attribute to

VI_FALSE to disable status checking and maximize performance

The instrument driver can choose to ignore status checking for particular

attributes regardless of the setting of this attribute.

The default value is VI_FALSE.  Use the Chr63000_InitWithOptions function

to override this value.

CHR63000_ATTR_RANGE_CHECK

Data Type:     ViBoolean

Description:   Specifies whether to validate attribute values and function

parameters.  If enabled, the instrument driver validates the parameter values

that you pass to driver functions.  Range checking parameters is very useful

for debugging.  After you validate your program, you can set this attribute to

VI_FALSE to disable range checking and maximize performance.

The default value is VI_TRUE.  Use the Chr63000_InitWithOptions function to

override this value.

CHR63000_ATTR_RECORD_COERCIONS

Data Type:     ViBoolean

Description:   Specifies whether the IVI engine keeps a list of the value

coercions it makes for integer and real type attributes.  You call Chr63000 Get

Next Coercion Record to extract and delete the oldest coercion record from the

list.

The default value is VI_FALSE.  Use the Chr63000_InitWithOptions function

to override this value.

CHR63000_ATTR_RES_STAT_FALL

Data Type:     ViReal64

Description:   Set the falling slew rate of current for constant resistance

mode.

CHR63000_ATTR_RES_STAT_IRNG

Data Type:     ViInt32

Description:   Set the current measurement range in CR mode.

Values:

CHR63000_VAL_LOW                               0

LOW

CHR63000_VAL_MIDDLE                            1

MIDDLE

CHR63000_VAL_HIGH                              2

HIGH

CHR63000_ATTR_RES_STAT_L1

Data Type:     ViReal64

Description:   Set static resistance level for constant resistance mode.

CHR63000_ATTR_RES_STAT_RISE

Data Type:     ViReal64

Description:   Set the rising slew rate of current for constant resistance

mode.

CHR63000_ATTR_SIMULATE

Data Type:     ViBoolean

Description:   Specifies whether or not to simulate instrument driver I/O

operations.  If simulation is enabled, instrument driver functions perform

range checking and call Ivi_GetAttribute and Ivi_SetAttribute functions, but

they do not perform instrument I/O.  For output parameters that represent

instrument data, the instrument driver functions return calculated values.

The default value is VI_FALSE.  Use the Chr63000_InitWithOptions function

to override this value.

CHR63000_ATTR_SPEC_CURR_C

Data Type:     ViReal64

Description:   Set the center-level current specification. The -1 mean dont

care.

CHR63000_ATTR_SPEC_CURR_H

Data Type:     ViReal64

Description:   Set the high-level current specification. The -1 mean dont

care.

CHR63000_ATTR_SPEC_CURR_L

Data Type:     ViReal64

Description:   Set the low-level current specification. The -1 mean dont

care.

CHR63000_ATTR_SPEC_POW_C

Data Type:     ViReal64

Description:   Set the center-level power specification. The -1 mean dont

care.

CHR63000_ATTR_SPEC_POW_H

Data Type:     ViReal64

Description:   Set the high-level power specification. The -1 mean dont care.

CHR63000_ATTR_SPEC_POW_L

Data Type:     ViReal64

Description:   Set the low-level power specification. The -1 mean dont care

CHR63000_ATTR_SPEC_TEST

Data Type:     ViInt32

Description:   Start or close the all channel specification test.

Values:

CHR63000_VAL_OFF                               0

OFF

CHR63000_VAL_ON                                1

ON

CHR63000_ATTR_SPEC_UNIT

Data Type:     ViInt32

Description:   Set the specific entry mode.

Values:

CHR63000_VAL_SPEC_PERCENT                      0

VALUE(1), PERCENT(0)

CHR63000_VAL_SPEC_VAL                          1

VALUE(1), PERCENT(0)

CHR63000_ATTR_SPEC_VOLT_C

Data Type:     ViReal64

Description:   Set the center-level voltage specification. The -1 mean dont

care.

CHR63000_ATTR_SPEC_VOLT_H

Data Type:     ViReal64

Description:   Set the high-level voltage specification. The -1 mean dont

care.

CHR63000_ATTR_SPEC_VOLT_L

Data Type:     ViReal64

Description:   Set the low-level voltage specification. The -1 mean dont

care.

CHR63000_ATTR_SPECIFIC_DRIVER_CLASS_SPEC_MAJOR_VERSION

Data Type:     ViInt32

Restrictions:  Not settable.

Description:   The major version number of the class specification with which

this driver is compliant.

CHR63000_ATTR_SPECIFIC_DRIVER_CLASS_SPEC_MINOR_VERSION

Data Type:     ViInt32

Restrictions:  Not settable.

Description:   The minor version number of the class specification with which

this driver is compliant.

CHR63000_ATTR_SPECIFIC_DRIVER_DESCRIPTION

Data Type:     ViString

Restrictions:  Not settable.

Description:   A string that contains a brief description of the specific

driver.

CHR63000_ATTR_SPECIFIC_DRIVER_PREFIX

Data Type:     ViString

Restrictions:  Not settable.

Description:   A string that contains the prefix for the instrument driver.

The name of each user-callable function in this driver starts with this prefix.

CHR63000_ATTR_SPECIFIC_DRIVER_REVISION

Data Type:     ViString

Restrictions:  Not settable.

Description:   A string that contains additional version information about this

instrument driver.

CHR63000_ATTR_SPECIFIC_DRIVER_VENDOR

Data Type:     ViString

Restrictions:  Not settable.

Description:   A string that contains the name of the vendor that supplies this

driver.

CHR63000_ATTR_SUPPORTED_INSTRUMENT_MODELS

Data Type:     ViString

Restrictions:  Not settable.

Description:   Contains a model code of the instrument. For drivers that

support more than one device, this attribute contains a comma-separated list of

supported instrument models.

CHR63000_ATTR_TIMING

Data Type:     ViInt32

Description:   Set the action enable or disable of TIMING function.

Values:

CHR63000_VAL_Disable                           0

Disable

CHR63000_VAL_Enable                            1

Enable

CHR63000_ATTR_TIMING_TOUT

Data Type:     ViReal64

Description:   Set the timeout for TINING function.

CHR63000_ATTR_TIMING_TRIGGER_MODE

Data Type:     ViInt32

Description:   Set the action rise or fall of TIMING function.

Values:

CHR63000_VAL_RISE                              0

RISE

CHR63000_VAL_FALL                              1

FALL

CHR63000_ATTR_TIMING_TRIGGER_VEND

Data Type:     ViReal64

Description:   Set the trigger end voltage in TIMING function.

CHR63000_ATTR_TIMING_TRIGGER_VSTART

Data Type:     ViReal64

Description:   Set the trigger start voltage in TIMING function.

CHR63000_ATTR_VOLT_STAT_ILIM

Data Type:     ViReal64

Description:   Set the current limit for constant voltage mode.

CHR63000_ATTR_VOLT_STAT_L1

Data Type:     ViReal64

Description:   Set the static load voltage in constant voltage mode.

CHR63000_ATTR_VOLT_STAT_RES

Data Type:     ViInt32

Description:   Set the response speed in constant voltage mode.

Values:

CHR63000_VAL_SLOW                              0

SLOW

CHR63000_VAL_NORMAL                            1

NORMAL

CHR63000_VAL_FAST                              2

FAST